3GPP RAN WG4 Meeting #118		R4-2600108

Gothenburg, Sweden, 9th – 13th February 2026

Agenda item:	10.6.1

Source:	Apple

Title:	Further considerations on enabling ETSI requirements for NTN NR/IOT L-band and S-band

WI/SI:	NR_NTN_solutions-Core

Release:	Rel-17

Document for:	Discussion

# 1	Introduction

RAN#86 meeting approved the Rel-17 WI that aimed to enable the 5G/NR radio access technology for non-terrestrial satellite deployments [1], whereupon one of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of the Rel-17 discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already at that time it was noted that there exist other satellite deployments. For instance, after the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band (band n254) where the UL is on the L-band and the DL is on the S-band [2]. After the RAN#104 meeting another spectrum WI was agreed to introduce new bands operating in the L-band frequency range [3].

While most of the satellite MSS bands have global ITU allocations and can be enabled in different regions, there exist regional and country specific requirements. As an example, US FCC defines additional out-of-band emission masks for all MSS bands that can be operated in US, and so ETSI does for the satellite MSS bands that can be activated in the ETSI regulatory domain. As identified and discussed in previous contributions from different proponents [4][5], 3GPP did not capture all ETSI regulations for all applicable MSS bands, which makes it impossible to ensure compliant UE behaviour in countries following the ETSI standards.

So, as discussed and concluded during the RAN4#117 meeting [6], missing ETSI requirements should be added to the 3GPP specifications. Thus, in this discussion paper we present our further considerations the ETSI requirements how they can be enabled to the 3GPP specifications.

# 2	3GPP and ETSI standards

One of the key aspects for a correct introduction of a band to the 3GPP core specifications is a complete set of regulatory requirements from regions, where the considered band is going to be used. And since satellite bands are intended to be deployed on a large geographical scale, it becomes especially crucial and important to reflect the corresponding requirements so that a band can be activated in desired geographical areas. As quickly mentioned in the Introduction part, MSS bands are regulated by international and region-specific regulations, whereupon ETSI has its own sets of standards that govern usage of all MSS bands that can be activated in the ETSI regulatory domain. Tabe 2-1 below makes a summary of NTN bands applicable to the ETSI region with the corresponding mapping to the 3GPP bands and ETSI specification(s) that provides additional band specific requirements.

As can be seen from the table, combined L-/S-band is fully covered by the 3GPP specifications, for which the corresponding NS flags were already defined. The L-band situation is more tangled: while 3GPP defined the corresponding NS flags in Rel-19, they are not available now for the Rel-17 bands 255/n255. And speaking of the S-band, no ETSI specific NS flags exist at all.

Observation 1a:	3GPP specifications fully support ETSI standards for the Rel-18 band 254/n254 and the Rel-19 IOT band 249.

Observation 1b:	3GPP specifications do not support ETSI standards for the Rel-17 band 255/n255 and Rel-18 IOT band 253; but the corresponding NS flags exist for the Rel-19 NR bands n250 and n251.

Observation 1c:	3GPP specifications do not support ETSI standards for the Rel-17 band 256/n256.

Table 2-1:	Summary of satellite bands and applicable ETSI requirements

| Band | 3GPP band | 3GPP release | ETSI standards | Applicable NS flags for ETSI |
| --- | --- | --- | --- | --- |
| L-band | 255/n255 | Rel-17 | EN 301 681 | no NS defined at the moment |
|  | 253/n253 | Rel-18/Rel-19 |  | NS_09N, NS_13N |
|  | n250 | Rel-19 |  | NS_09N, NS_13N |
|  | n251 | Rel-19 |  | NS_10N, NS_14N |
|  | 249 | Rel-19 | EN 301 441 | NS_04N, NS_05N, NS_11N, NS_12N |
| L-/S-band | 254/n254 | Rel-18 |  |  |
| S-band | 256/n256 | Rel-17 | EN 301 442 EN 302 574-2 EN 302 574-3 | no NS defined at the moment |

## 2.1	ETSI requirements for L-band

Referring back to Table 2-1 and our previous considerations, 3GPP Rel-19 introduced two new NR NTN bands n250 and n251. 3GPP already analysed applicable ETSI requirements from EN 301681, which were captured in the 3GPP specifications by new NS flags NS_09N, NS_10N, NS_13N and NS_14N (note that NS_13N and NS_14N are reserved for future new ETSI harmonised standards). And since band n255 UL shares the same frequency range with band n251, same ETSI requirements would apply to both bands meaning that same NS flag(s) can be re-used to enable ETSI requirements for the Rel-17 band n255.

Observation 2a:	Since 3GPP enabled support for the ETSI standards for the Rel-19 extended L-bands, same NS flags and A-MPR values can be enabled for the NR Rel-17 n255 and Rel-18 n253 bands, i.e. no extra analysis is needed at least for PC3.

Observation 2b:	3GPP did not conduct power back-off analysis for the ETSI standards with IOT Rel-17 255 and Rel-18 253 bands.

Observation 2c:	Previous studies indicated that no A-MPR is anticipated for the IOT bands 255/253, but additional check should be done to ensure that no A-MPR is needed for the aforementioned IOT bands.

Our contribution in [7] presents additional technical analysis of ETSI requirements from EN 301 681 with power back-off simulations for the IOT bands 253 and 255. And at least according to our simulation we do not need any power back-off for the IOT bands at least for the 15kHz SCS, i.e. there should be a straightforward path to enable existing L-band NS flags to the IOT bands 255 and 253 defined in earlier releases. At the same time, it bears mentioning that 3.5kHz SCS require extra A-MPR.

## 2.2	ETSI requirements for S-band

The situation for the ETSI requirements and the S-band is a bit different because no previous analysis was done for that band. There are several NS flags defined for the S-band, but they are not for the ETSI regulatory domain. In addition to that, it is worth noting that the S-band is regulated by three different ETSI standards depending on the type of the system and its parameters, such as wide- narrow-band channel. So 3GPP should also check which standards, or all of them, should be further considered.  At the same time, it is worth noting that at least EN 301442 relies upon the same emission limits outside the MSS band as the L-band ETSI EN, which means that we can leverage already agreed A-MPR values.

Observation 3a:	Since the MSS S-band is regulated by several ETSI standards, it should be checked further whether we can account only for EN 301442 or also 302574-2/3 should be considered.

Observation 3b:	3GPP did not conduct power back-off analysis for the ETSI standards with the Rel-17 256/n256.

Observation 3c:	At least ETSI EN 301442 relies upon the same emission limits as the L-band ETSI EN standard, i.e. we can leverage existing A-MPR studies.

Table 2.2-1: Summary of ETSI S-band standards.

| System bandwidth | non-GEO system | GEO system |
| --- | --- | --- |
| wide-band (>1MHz) | EN 301 442 | EN 302 574-2 |
| narrow-band (<1MHz) |  | EN 302 574-3 |

Our contribution in [8] presents further technical analysis of the ETSI requirements for the S-band covering both NB-IOT and NR NTN technologies.

## 2.3	Way forward

Given all considerations presented in previous sub-sections, our view is that 3GPP needs to add support for ETSI standards, which were not captured for the Rel-17 NTN bands 255/n255 and 256/n256. The corresponding work does not require development of new features but will just involve checking existing ETSI standards and whether requirements from ETSI standards require any A-MPR.

Proposal 1:	We ask RAN WG4 to make Rel-17 maintenance corrections to enable proper support of ETSI standards in the MSS L- and S-bands (considering two independent technical tracks).

Proposal 1a:	L-band track: enable support of ETSI standards for the Rel-17 NR band n255 and the Rel-18 IOT band 253/255 by porting NS flags into earlier releases.

Proposal 1b:	S-band track: enable support of ETSI standards for the Rel-17 NR band n256 and Rel-18 IOT band 256 by adding corresponding NS flag.

It bears mentioning that the Rel-19 NTN enhancements enabled higher power classes for bands 255/n255 and 256/n256. So, if 3GPP introduces support for the ETSI requirements for PC3 in Rel-17 and Rel-18, it will remain unclear how to address the ETSI requirements for Rel-19 power classes PC2 and PC1. That should be discussed further in 3GPP, for which we envision two major potential outcomes: a) conclude that L- and S-band ETSI requirements will be also supported with higher power classes through further Rel-19 maintenance corrections; or b) conclude that L- and S-band ETSI requirements with higher power classes will be introduced only in Rel-20 through the corresponding ongoing NTN Rel-20 basket WID [9].

Proposal 2a:	Higher power classes support for the ETSI requirements, i.e. PC2 and PC1, can be introduced through the Rel-19 maintenance correction (based on contributions from interested companies).

Proposal 2b:	Any leftovers can be further completed in the ongoing Rel-20 NTN basket WID.

# 3	Conclusions

In this discussion paper we presented our general considerations on enabling missing ETSI requirements for the L- and S-bands. As explained in the paper, 3GPP has already conducted analysis for the ETSI L-band requirements in Rel-19, which means that the existing NS flags and requirements can be enabled also for earlier releases.

Proposal 1:	We ask RAN WG4 to make Rel-17 maintenance corrections to enable proper support of ETSI standards in the MSS L- and S-bands (considering two independent technical tracks).

Proposal 1a:	L-band track: enable support of ETSI standards for the Rel-17 NR band n255 and the Rel-18 IOT band 253/255 by porting NS flags into earlier releases.

Proposal 1b:	S-band track: enable support of ETSI standards for the Rel-17 NR band n256 and Rel-18 IOT band 256 by adding corresponding NS flag.

Proposal 2a:	Higher power classes support for the ETSI requirements, i.e. PC2 and PC1, can be introduced through the Rel-19 maintenance correction (based on contributions from interested companies).

Proposal 2b:	Any leftovers can be further completed in the ongoing Rel-20 NTN basket WID.

# 4	References

RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

RP-241689, "Revised WID on New NR NTN bands to support Extended L-band and combined MSS L-band and Extended L-band ranges ", Inmarsat, Viasat

R4-2500292, "Emission requirements for the UE operating in 1626.5-1660.5MHz and 1668-1675MHz UL frequency ranges", Apple

R4-2513841, "Missing ETSI emission requirements for IoT NTN bands 253 and 255 and NR NTN band n255", ViaSat Satellite Holdings Ltd

R4-2522419, "WF for enabling ETSI requirements for NTN NR/IOT bands", Apple, Viasat, Thuraya, Vodafone, AST Space Mobile, Sony

R4-2600109, "Power back-off analysis for the ETSI requirements in the MSS L-band", Apple

R4-2600110, "Power back-off analysis for the ETSI requirements in the MSS S-band", Apple

RP-253841, "WID for the Introduction of the higher power classes, 3MHz channel bandwidth and (e)RedCap for the existing NR/IoT NTN bands", Apple, Viasat, Terrestar, Globalstar, Skyworks Solutions, Iridium, Echostar, MediaTek