3GPP RAN WG4 Meeting #118		R4-2600109

Gothenburg, Sweden, 9th – 13th February 2026

Agenda item:	10.6.1

Source:	Apple

Title:	Power back-off analysis for the ETSI requirements in the MSS L-band

WI/SI:	NR_NTN_solutions-Core

Release:	Rel-17

Document for:	Discussion

# 1	Introduction

RAN#86 meeting approved the Rel-17 WI that aimed to enable the 5G/NR radio access technology for non-terrestrial satellite deployments [1], whereupon one of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of the Rel-17 discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. While most of the satellite MSS bands have global ITU allocations and can be enabled in different regions, there exist regional and country specific requirements. As an example, US FCC defines additional out-of-band emission masks for all MSS bands that can be operated in US, and so ETSI does for the satellite MSS bands that can be activated in the ETSI regulatory domain.

3GPP did not capture existing ETSI requirements for all MSS L-bands. And as concluded during the RAN4#117 meeting, the corresponding requirements should be added to the 3GPP specifications.  Thus, in this discussion paper we present further overview of the existing ETSI requirements for the MSS L-bands with associated power back-off analysis.

# 2	ETSI standards for the MSS L-band

During the Rel-19 specification work for the so-called combined L-bands, 3GPP already analysed applicable ETSI standards and performed the power back-off analysis for the 5G/NR based NTN technologies. The outcome of that work was new NS flags that were defined for the NTN NR bands. However, there was no analysis performed for the IOT bands 255 (ETSI L-band sub-rage 1) and 253 (ETSI L-band sub-range 2).

## 2.1	ETSI emissions within the band

The L-band in-band and out-of-band emission requirements are defined in ETSI EN 301681. The emission limits within the band can be found in Annex A, whereupon Figure A-1 shows the super-position of the ETSI mask (pink) and the 3GPP NB-IOT mask (blue). In Figure A-1 all emission levels are scaled to the same measurement unit expressed in dBm/MHz, whereupon we indicate both original and scaled values.

As can be seen from the figure, the 3GPP NB-IOT mask is more stringent except a small frequency range at offset of 100-150kHz, where the ETSI mask has more stringent emission levels. Nevertheless, we conducted a set of additional power back-off simulations for the 15kHz SCS, presented in Figure 2.1-1, from which we can see that irrespective of how many tones are allocated – 1 tone or 12 tones – all emissions stay within the ETSI limits.

Observation 1a:	While the ETSI emission limits within the band are more stringent for the offset at 100-150kHz, actual power back-off simulations indicate that all emissions stay within the ETSI limits for the 15kHz SCS case.

Figure 2.1-1: Power back-off simulations for PC3 and the ETSI in-band mask (1, 3, 6 and 12 tones).

In addition to the 15kHz SCS case, we also simulated the 3.75kHz SCS, for which we observed a need to define some A-MPR. So, allocations at the edge of the channel will require up to 1.5dB A-MPR in this case.

Observation 1b:	Unlike 15kHz SCS, 3.75kHz SCS will require up to 1.5dB A-MPR for allocations close to the edge of the channel.

It is worth noting that according to ETSI EN 301681 same in-band mask is assumed for ETSI sub-range 1 and sub-range 2, i.e. IOT bands 255 and 253, respectively. Thus, same conclusions can be applied for the IOT bands 255 and 253.

Observation 1c:	Same ETSI mask from EN 301681 is assumed for emission limits within sub-range 1 and sub-range 2, i.e. IOT bands 255 and 253, meaning that the same A-MPR values can be adopted for both bands.

## 2.2	ETSI emissions outside band boundaries

ETSI EN 301681 follows different approaches while defining emission limits for frequencies outside the boundaries of the sub-range 1 and 2:

-	Sub-range 1 (band 255): The lower edge range emission limits start with a stringent limit of -10dBm/30kHz, i.e. 5dBm/MHz, which will cause power back-off for the NB-IOT channel if placed at the lower edge of the band (refer to Figure B-1 in Annex).

-	Sub-range 2 (band 253): Both lower and the upper edge out-of-band emission limits are based on the ETSI in-band emission mask and thus would not trigger any additional constraints.

Observation 2a:	Sub-range 1 (band 255) lower edge strict emission requirements will require power back-off.

Observation 2b:	Sub-range 1 (band 255) upper edge emission limits are based on the in-band mask and thus will not require any power back-off.

Observation 2c:	Sub-range 2 (band 253) lower and upper edge emission requirements are based on the in-band mask and thus will not require any power back-off.

Referring to the previous considerations on the band 255 lower edge emission limits (Observation 2a), it is worth noting that 3GPP has solved this problem by "blanking" the corresponding NB-IOT channel residing at the band lower edge. However, the corresponding restriction is captured as a generic statement for band 255 not accounting for the fact that it is not needed for band 255 operated under the FCC rules. Furthermore, as presented in Figure 2.2-1 below, no A-MPR is needed when the NB-IOT channel resides at the lower edge of the band, but the network schedules a rightmost subset of sub-carriers within the NB-IOT channel. For instance, the network can allocate 6 tones at position 6, i.e., the rightmost allocation, whereupon no A-MPR is needed. Similarly, the network can schedule 3 tones at position 3 or 1 tone at position 2. Thus, this NB-IOT channel still can be used under the ETSI rules.

Observation 2d:	The IOT band 255 lower-most NB-IOT channel does not need to be blanked and still can be used with the FCC rules.

Observation 2e:	The IOT band 255 lower-most NB-IOT channel can be used with no A-MPR under the ETSI rules if the network schedules a rightmost subset of sub-carriers within the NB-IOT channel.

Figure 2.1-2: Power back-off simulations for PC3 and the ETSI mask for the lower edge NB-IOT channel (1-tone@pos2, 3-tone@pos3, and 6-tone@pos6).

# 3	Conclusions

In this discussion paper we presented our considerations on the ETSI requirements for the L-band(s) and whether they can be met by the NB-IOT. Based on the presented analysis and additional power back-off simulations conducted for the ETSI in-band mask, our conclusion is that no additional power back-off is needed for the IOT bands 255 and 253 in case of 15kHz SCS, but 3.75kHz SCS may require additional A-MPR.

Observation 1a:	While the ETSI emission limits within the band are more stringent for the offset at 100-150kHz, actual power back-off simulations indicate that all emissions stay within the ETSI limits for the 15kHz SCS case.

Observation 1b:	Unlike 15kHz SCS, 3.75kHz SCS will require up to 1.5dB A-MPR for allocations close to the edge of the channel.

Observation 1c:	Same ETSI mask from EN 301681 is assumed for emission limits within sub-range 1 and sub-range 2, i.e. IOT bands 255 and 253, meaning that the same A-MPR values can be adopted for both bands.

Observation 2a:	Sub-range 1 (band 255) lower edge strict emission requirements will require power back-off.

Observation 2b:	Sub-range 1 (band 255) upper edge emission limits are based on the in-band mask and thus will not require any power back-off.

Observation 2c:	Sub-range 2 (band 253) lower and upper edge emission requirements are based on the in-band mask and thus will not require any power back-off.

Observation 2d:	The IOT band 255 lower-most NB-IOT channel does not need to be blanked and still can be used with the FCC rules.

Observation 2e:	The IOT band 255 lower-most NB-IOT channel can be used with no A-MPR under the ETSI rules if the network schedules a rightmost subset of sub-carriers within the NB-IOT channel.

# 4	References

RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

Annex A:	ETSI EN 301681 emission limits within the band

Table A-1: ETSI emission limits within the band (for sub-range 1 and sub-range 2).

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth | Note (measurement method) |
| --- | --- | --- | --- |
| ± 0-25 | 30 to 15 | 3 kHz | Average |
| ± 25-125 | 15 to -20 |  |  |
| ± 125-425 | -20 |  |  |
| ± 425-1500 | -20 to -35 |  |  |
| ± 1500-36000 | -25 | 30 kHz |  |

Figure A-1: Superposition of the ETSI in-band mask (pink) and 3GPP NB-IOT mask (blue).

Annex B:	ETSI EN 301681 emission limits outside the band

Figure B-1: Superposition of the ETSI mask for the sub-range 1 (pink) and 3GPP NB-IOT mask (blue).