3GPP TSG-RAN WG4 Meeting #118	R4-2600116

# Gothenburg, Sweden,  9th – 13st February 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.764 | CR | 0010 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of the NTN IOT band 256 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIoT_eMTC_NTN_req-Core |  |  |  |  |  | Date: |  |  | 2026-01-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When TR 36.764 was instantiated its main purpose was to collect informaion on NTN IOT bands with related information on reglations and 3GPP requirements. However, since it was created at the end of the corresponding Rel-18 NTN IOT WI, the TR does not capture some IOT bands, such as 256. Thus, there is no section in TR where we could capture band specific aspects, such as regulations and associated requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new section 8 is added to capture the NTN IOT band 256 related requirements, as well as regulatory related aspects with associated evaluations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no section for the NTN IOT band 256, which will prevent 3GPP from capturing band specific aspects, such as regulations and associated regional specific requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 8	FDD band B256 (S-band)

## 8.1	Regulation

### 8.1.1	ETSI requirements

ETSI requirements for the MSS S-band are captured in the following documents:

-	ETSI EN 301 442: non-GEO narrow-band (<1MHz) and wide-band (>1MHz) systems

-	ETSI EN 302 574-2: GEO wide-band (>1MHz) systems

-	ETSI EN 302 574-3: GEO narrow-band (<1MHz) systems

Based on the scope of these ETSI EN standards, NB1/NB2 devices are regulated by EN 301442 and EN 302574-3 standards; and M1 devices are regulated by EN 301442 and 302574-2 standards.

### 8.1.2	FCC requirements

This frequency range is not defined as an MSS band in US or Canada, and thus existing FCC requirements are not applicable to this band.

## 8.2	UE requirements

### 8.2.1	UE transmitter characteristics

#### 8.2.1.1	Maximum output power for category M1 and NB1/NB2

Table 8.2.1.1-1: UE Power Class for category M1

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |

Table 8.2.1.1-2: UE Power Class for category NB1 and NB2

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 23 | +/-2 | 20 | +/-2 |

#### 8.2.1.2	Emission requirements and NS values

##### 8.2.1.2.1	Spurious emission for category M1

Table 8.2.1.2.1-1: Requirements for spurious emissions for UE co-existence

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 256 | E-UTRA Band 1, 3, 5,7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 33, 32, 35, 38, 40, 41, 42, 43, 50, 51, 54, 65, 68, 69, 72, 74, 75, 76, 87, 88 NR Band n12, n13, n14, n24, n29, n30, n39, n48, n53, n66, n67, n71, n78, n79, n85, n90, n91, n92, n93, n94, n101, n105, n106, n109 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n2, n25, n70 | FDL_low | - | FDL_high | NA | NA | 3 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in TS 36.102 [4] Table 5.4A.2-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in TS 36.102 [4] Table 6.5A.4.2-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: The co-existence between 256 and band 2, 25 and 70 is subject to regional/national regulation. |  |  |  |  |  |  |  |

##### 8.2.1.3	Configured TX power

It is agreed the current TS 36.102 [4] requirements can be re-used for band B256.

##### 8.2.1.4	Power control

It is agreed the current TS 36.102 [4] requirements can be re-used for band B256.

##### 8.2.1.5	Frequency error

It is agreed the current TS 36.102 [4] requirements can be re-used for band B256.

##### 8.2.1.6	Transmit modulation quality

It is agreed the current TS 36.102 [4] requirements can be re-used for band B256.

### 8.2.2	UE receiver characteristics

#### 8.2.2.1	Reference sensitivity for category M1 and NB1/NB2

Table 8.2.2.1-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 256 | -102.2 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101 [3]. |  |  |

Table 8.2.2.1-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 256 | -103 | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 in TS 36.101 [3] |  |  |

Table 8.2.2.1-3: FDD UE category M1 Uplink configuration for reference sensitivity

| E-UTRA Band | NRB | Duplex Mode |
| --- | --- | --- |
| 256 | 61 | FDD and HD-FDD |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3A-1 in TS 36.102 [4]). |  |  |

Table 8.2.2.1-4: Reference sensitivity for UE category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| 256 | - 108.2 |

#### 8.2.2.2	Blocking requirements

##### 8.2.2.2.1	In Band blocking requirements for category M1

Table 8.2.2.2.1-1: In band blocking parameters

| Rx parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | 6 |
| BWInterferer | MHz | 1.4 |
| FIoffset, case 1 | MHz | 2.1+0.0125 |
| FIoffset, case 2 | MHz | 3.5+0.0075 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in TS 36.102 [4] Table 7.3A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101 [3]. NOTE 2: The interferer consists of the Reference measurement channel specified in TS 36.101 [3] Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 and set-up according to Annex C.3.1. NOTE 3: For DL category M1 UE, the reference sensitivity for category M1 in TS 36.102 [4] table 7.3A-1 should be used as REFSENS for the power in Transmission Bandwidth Configuration. NOTE 4: For DL category M1 UE, the parameters for the applicable channel bandwidth apply. |  |  |

Table 8.2.2.2.1-2: In-band blocking

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 256 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

##### 8.2.2.2.2	Out-of-band blocking requirements for category M1

Table 8.2.2.2.2-1: Out-of-band blocking parameters for category M1 UE

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 1.4 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 6 dB |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 in TS 36.101 [3] with PCMAX_L as defined in TS 36.101 [3] subclause 6.2.5. |  |  |

Table 8.2.2.2.2-2: Out of-band blocking for category M1 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 256 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |

##### 8.2.2.2.3	Narrow band blocking for category M1

Table 8.2.2.2.3-1: Narrow-band blocking

| Parameter | Unit | Channel Bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Pw | dBm | PREFSENS + channel-bandwidth specific value below |
|  |  | 22 |
| Puw (CW) | dBm | -55 |
| Fuw (offset for f = 15 kHz) | MHz | 0.9075 |
| Fuw (offset for f = 7.5 kHz) | MHz |  |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L at the minimum uplink configuration specified in TS 36.102 [4] Table 7.3A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101 [3]. NOTE 2: Reference measurement channel is specified in TS 36.101 [3] Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 of TS 36.101 [3]. NOTE 3: For DL category M1 UE, the reference sensitivity for category M1 in TS 36.102 [4] table 7.3A-1 should be used as PREFSENS for Pw. NOTE 4: For DL category M1 UE, the parameters for the applicable channel bandwidth apply. NOTE 5: For DL category M1 UE, the parameter, Pw, for all the channel bandwidth will be PREFSENS +22 dBm. |  |  |

##### 8.2.2.2.4	Out-of-band blocking requirements for category NB1 and NB2

Table 8.2.2.2.4-1: Out-of-band blocking parameters for category NB1 and NB2 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -153 |
| 256 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: For operating bands which downlink band frequency range is between 1475.9 MHz < f < 2690 MHz the power level of the interferer (PInterferer) for Range 3 shall be modified to: -20 dBm for the frequency range which is bounded by FDL_low- 200 MHz of the lowest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz and FDL_high + 200 MHz of the highest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz.” NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. |  |  |  |  |  |

## 8.3	Evaluation and simulations

### 8.3.1	NB1/NB2 evaluation

There are two ETSI standards – EN 301442 (non-GEO systems with any channel bandwidth) and 302574-3 (GEO systems with the channel bandwidth <1MHz) – that are applicable to the NB1/NB2 devices due to the 200kHz channel bandwidth. Figure 8.3.1-1 below presents a superposition of the ETSI EN 301442 mask and the NB-IOT 200kHz mask, from which one can see that the NB-IOT mask is more stringent. Furthermore, the ETSI mask in 302574-3 has even more relaxed emission limits, and thus should not cause any extra power back-offs.

Figure 8.3.1-1: Superposition of the ETSI mask (pink) with 3GPP NB-IOT 200kHz mask (blue).

Figure 8.3.1-2: Superposition of the ETSI mask (pink) with 3GPP NB-IOT 200kHz mask (blue).

### 8.3.2	M1 evaluation

There are two ETSI standards – EN 301442 (non-GEO systems with any channel bandwidth) and 302574-2 (GEO systems with the channel bandwidth >1MHz) – that are applicable to the M1 devices due to the 1.4MHz channel bandwidth. Referring specifically to EN 302574-2, it does not define any specific emission limits within or outside the S-band boundaries and thus should not create any additional restrictions. As of EN 301442, it defines an additional mask that is applied to any channel within the S-band and to the 2MHz offset from the band edges. Figure 8.3.2-1 below shows the graphical interpretation of the M1 1.4MHz mask and the ETSI mask.

Figure 8.3.2-1: Superposition of the ETSI mask (pink) with 3GPP M1 mask (blue).

While Figure 8.3.2-1 may give the impression that we need to define extra power back-off for the M1 devices, additional power back-off simulations indicate that existing MPR limits are sufficient. As can be seen from Figure 8.3.2-2, both DFT-s-OFDM and CP-OFDM do not require extra power back-off margins.

Figure 8.3.2-2: Power back-off simulations for the emission limits inside the S-band (1.4MHz, DFT-s-OFDM and CP-OFDM)