3GPP TSG-RAN WG4 Meeting#118		R4-2600235

Gothenburg, Sweden, Feb 9th - 13th, 2026

Agenda item:	8.3.1

Source: 	Spreadtrum, UNISOC

Title: 	Views on 6G UE RF output power requirements

Document for:	Discussion

Introduction

In RAN4#117 meeting, a WF [1] about 6G general RF and UE RF was approved. In this contribution, we continue to provide some views on output power requirements.

Discussion

Single carrier output power requirements

- Agreements for power class - Further study step size between power classes. Encourage further inputs considering e.g. - Benefits/drawbacks of potential finer granularity between power classes - Granularity for MPR/A-MPR requirement definition - Output power tolerance - REFSENS requirement impact - Potential regulatory restrictions - Flexibility for implementation - Further study power class for multi-TX case (TxD, UL-MIMO) - Consider benefits and drawbacks for - Defining power class to be agnostic of number of Tx chains and/or PAs - Considering that each chain and/or PA should be able to reach maximum output power - Consider impact of transparent vs. non-transparent TxD - Further study how to enable fully utilizing PA capabilities considering e.g. - Relaxation or removal of upper tolerance without sacrificing output power accuracy - Power boosting - Maximum power capability reporting - Removing the nominal power limit in the PCMAX upper bound - Further study need for default power class and default power class values considering e.g. - Network UL coverage / UL link budget at different frequencies/ System performance - Impact on REFSENS definition for FDD bands - Maximum channel bandwidth - Cell planning from the operators - Potential regulatory output power restrictions - Agreements for configured maximum output power - Further study SAR solution considering - P-MPR as baseline below 6 GHz - Whether/how SAR or MPE applies at higher than 6GHz and applicable solutions - Achieving more clarity on UE output power - Further study potential UL duty cycle requirement to optimize component size/cost considering - Use cases and deployment scenarios as well as network needs - Further review the 5G configured Tx power equation parameters considering at least the following - ∆TC,c : can it be removed, considering also new wider CBW and FFS on whether and where to accommodate this relaxation in requirements. - ∆TRxSRS: should it be separated from general Pcmax equation as it only addresses SRS transmission and FFS on how it can be accommodated. - ∆Tib: can the requirement be removed and accommodated to per band power capability and FSS on whether and where to accommodate this relaxation in requirements. ΔPPowerClass: Is it still needed in some cases even if not associated with SAR

For NR FR1, power class definition which is applied in single carrier, UL MIMO, CA, ENDC and Tx diversity. The step of power class is 3dB whatever the scenario. With the increased power class, especially above for PC1.5, it is more difficult for UE implementation to achieve a 3dB step. With the increase of power, the efficiency of PA will gradually decrease. The UE may waste more than 2x power consumption with the power class increase 3dB. In our understanding, in the next years, the transmission power of the terminal can’t always be in the afternoon at the step of 3dB, which is a great challenge to the power consumption and heat dissipation of the UE. At the same time, the power of PC1.5 is 29dBm，the power of PC1 is 31dBm, the step is 2dB. Therefore, regarding the present situation and future challenges, we can define the step as 2dB (or other values) above PC1.5 in 6GR. If we define the finer granularity between power classes above PC1.5, the granularity for MPR/AMPR requirement definition will be finer. We can try to find an approach like MSD to define a look up table to derive the requirements.

Proposal 1: Regarding the present situation and future challenges, we can define the step as 1dB or 2dB) above PC1.5 in 6GR. Try to find an approach like MSD to define a look up table to derive the requirements, including MPR/AMPR, REFSENS and other requirements.

In 5G, 1Tx, 2Tx, 3Tx and 4Tx are all supported and specified in spec. In order to fully use PA capability, tx diversity, full power mode and UL MIMO are specified. In the last meeting, the agreement was about to consider benefits and drawbacks for defining power classes to be agnostic of number of Tx chains and/or PAs. In our understanding, the pros are that it is helpful for network to obtain the UE capability of every single Tx chain to schedule for single layer or boosting the accuracy of beamforming for UL MIMO. In addition, the power class of a single Tx chain can be the baseline for UL MIMO/tx diversity/ UL CA. Obviously, it is still necessary to report multi-Tx chain UE power class to NW, otherwise, it may cause the scheduled which is not wanted by UE. Because even though every Tx chain can achieve the real transmission power. However, the power class of UL MIMO/UL CA cannot be thought of as the sum of every Tx chain.

Observation1: The pros are that it is helpful for network to obtain the UE capability of every single Tx chain to schedule for single layer or boosting the accuracy of beamforming for UL MIMO. The cons are more flexible for network to schedule, but it may cause the scheduled which is not wanted by UE.

Proposal 2: Tx diversity, full power mode and UL MIMO in 5G can be reused in 6GR. UE report power class of every Tx chain to NW can be considered.

Regarding the impact of transparent vs non-transparent TxD, the critical difference between transparent and non-transparent is whether the receiver needs to know the specific mode of tx diversity. For transparent TxD, it is the only mode in 5G NR. It is simple to design and compatible.  And it is stable when the scenery is high speed/ channel changes rapidly. However, it is impossible to obtain the gain of array but the diversity gain. For non-transparent TxD, it can obtain the array gain by channel self-adaption and multiplexing gain. However, it is necessary for receiver to know the precoding scheme and feedback the information like PMI, and the performance relies on the quality of the feedback for CSI. In summary, it is more complex to obtain the gain of array. In our understanding, from UE implementation perspective, it is simpler to reuse the transparent TxD.

Proposal 3: Transparent TxD is still only kept in 6GR.

It is observed that there is a complex and rigmarole formula to describe about configured maximum output power in 5G.The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass + ΔPPowerBoost}

From RAN4 perspective, we can consider which part of parameters can be remove or optimized. Even though we cannot remove all of the parameters, we can simplify the spec in 6GR. For ∆TC,c, which refers to the note3 in Table 6.2.1-1 in TS 38.101-1, it is the relaxation requirements due to the power flatness. The power flatness is related to the bandpass filter. If the frequency in downlink band and uplink band is close or the bandwidth is large. It is observed that it is necessary based on the current filter technology. We can optimize this value rather than remove this value.

Proposal 4: ∆TC,c can be optimized rather than remove it in 6GR.

For ΔPPowerClass, it is too complex to describe in spec, which is mainly used for the power fallback of duty cycle SAR. Duty cycle SAR will be remove if possible, ΔPPowerClass can be considered to remove.

Proposal 5: ΔPPowerClass can be considered to remove if Duty cycle SAR will be removed.

For ΔTIB,c is the additional tolerance for serving cell c for NR CA, SUL and ENDC. ENDC will not be supported in 6GR. The purpose of ΔTIB,c is to consider the additional insertion loss due to the band combination. It is observed that ΔTIB,c are all under 1dB. If the band group concept is introduced in 6GR, the number of band combinations will decrease, thus the analysis of ΔTIB,c will be easier. However, supporting the intra-group CA due to the request of operators’ is needed, then the addition insertion loss is existing due to the band or band group combination. In our understanding, ΔTIB,c needs to be considered. Of course, in order to reduce the workload for ΔTIB,c, we can define a simple approach, such as setting two levels of value of ΔTIB,c based on some rules. In summary, we can improve the ΔTIB,c requirement rather than remove it.

Proposal 6: ΔTIB,c needs to be considered in 6GR. Simplification of ΔTIB,c requirement can be FFS.

∆TRxSRS is applied during SRS transmission occasions with usage in antenna switching. SRS antenna switching is detected and used for downlink channel estimation. ∆TRxSRS is only used in transmit SRS in uplink to calculate the PCMAX. In our understanding, it can be separated from general PCMAX equation. However, if PCMAX,SRS can be configured independently, whether PRACH/PUCCH needs to be configured independently.

Proposal 7: For ∆TRxSRS, it can be separated from general PCMAX equation (e.g., PCMAX,SRS).Whether PRACH/PUCCH needs to be set independently align to the RAN1 spec can be FFS.

In 5G, FDD and TDD in FR1 about default power class are all PC3. Others power classes like PC2, PC1.5 and PC5 are also supported. Compared to 5G, according to the RAN guidance, around 7GHz needs to keep the same network grid like 3.5GHz, which means that around 7GHz has the same target coverage as 3.5GHz. Around 7GHz has higher free loss compared to 3.5GHz, default power class to be PC2 is reasonable. For FDD, it is widely applied in IoT market. IoT devices are sensitive for power consumption, and the most of commercial market are PC3 for FDD. Hence, the default power class can still be PC3 for FDD bands. Regarding TDD bands in FR1, it can be further discussed.

Proposal 8: For FDD bands, PC3 can still be the default power class. For around 7GHz, PC2 can be the default power class. For TDD bands in FR1, the default class can be further discussed.

Currently, there are two approaches to accessing the electromagnetic radiation. They are SAR and MPE respectively. The full name of SAR is Specific Absorption Rate, which means electromagnetic waves absorbed or consumed by human tissues per unit mass, in units of W/Kg or MW/g. Refers to the measured value of energy absorption rate when a human body is exposed to an RF electromagnetic field. The full name of MPE is Maximum Permissive Exposure. MPE is under normal operation, the human body can be exposed to a certain form of ability (such as laser, radio frequency electromagnetic field, etc.) without immediately or delaying the highest level harmful to health, the unit of MPW is W/m2. In 5G, SAR is applied to FR1 bands to access the electromagnetic radiation, MPE is applied to FR2 bands to access the electromagnetic radiation. There are two approaches as SAR solutions. The one is duty cycle, which is specified in 3GPP. The other is P-MPR which is specified only for P-MPRc. For duty-cycle, it lacks flexibility for power control due to the power class fall back. UE usually use P-MPR rather than duty-cycle solution to address SAR by implementation. From our point of view, it is not necessary to consider duty cycle as a SAR solution in 6GR. We can keep P-MPR as a SAR solution.

Proposal 9: It is not necessary to consider duty-cycle for SAR solution in 6GR. We can keep P-MPR as a SAR solution in 6GR.

In 5G, SAR is applied to FR1 bands to access the electromagnetic radiation, MPE is applied to FR2 bands to access the electromagnetic radiation. Around 7GHz and around 15GHz will be introduced in 6GR, whether SAR or MPE is appropriate to assess the electromagnetic radiation for these new spectrum is needed to study. With the increase of frequency, the depth of electromagnetic wave penetrating conductor (including lossy dielectric such as human body) will become shallow, and the ability will mainly be concentrated in. In low frequency, electromagnetic waves can penetrate several centimetres and reach deep tissues such as the brain and eyes. It is necessary to pay attention to the distribution of energy in the body, such as SAR. In higher frequency, the penetration depth of electromagnetic wave is shallow, which cause local overheating of the skin and cornea, MPE is appropriate to assess. According to the IEEE Standards C951 in Annex B.1.3.1 General as follow, below 6GHz, SAR or MPE (PD) is applied metric, above 6GHz, MPE (PD) is applied metric. Therefore, SAR can be applied in below 6GHz, MPE (PD) can be applied in above 6GHz in 6GR.

Proposal 10: SAR can be applied in below 6GHz, MPE (PD) can be applied in above 6GHz in 6GR.

CA output power requirements

In 5G NR, for CA/DC, the total power class should be defined. RAN2 defines the capability of higherPowerLimit-r17 to achieve the full power for CA/DC, which can make every carrier/band realize the real max output power. Meantime, the capability can be reported by UE-powerclassPerBandPerBC-R17. Therefore, in our understanding, there is no need to define the total power class for band combinations from the point of view of RAN4 in 6GR.

As we all know, CA can be categorized to inter-band CA and intra-band CA. For intra-band CA power class, in 5G NR, it is different from the band power class. For example, the UE power class of n3 is PC2. The power class of CA_n3B is PC3. Because the PC2 of n3 can be realized by 1Tx or 2Tx. Similarly, the UE power class of n26 is PC2, the power class of CA_n26 (2A) is PC3. In the market, 2Tx of n3 and n26 for PC2 are usual. Hence, in some cases like these, the power class of intra-band CA cannot be the same as the band power class.

Whatever the inter-band CA or intra-band CA, for downlink CA with UL CA/UL single CC, UL CA with configuration, activation/deactivation, there is no explicit power class to distinguish these states from UE capability perspective. In TS 38.101, UE power class Table is different between downlink intra-band contiguous CA with a single uplink CC and uplink intra-band contiguous CA. However, there is no power class to distinguish between downlink intra-band contiguous CA with a single uplink CC and uplink intra-band contiguous CA in TS 38.306. For UL CA with configuration, activation and deactivation, similarly, there are different power classes for these states. However, there is no power class to distinguish and indicate. In order to improve the UL coverage and reduce the MPR, there is an IE to indicate whether the UE supports reduced MPR for single CC if single CC is activated for intra-band UL contiguous CA by mpr-ActiveCarrierEnh-r19. From this point of view, it is obvious that there is a difference between configuration and activation. Based on the above analysis, UE should be allowed to report actual and dynamic power class to realize the maximum output power for CA.

Proposal 11: Remove the total power class definition for band combinations in 6GR.

Proposal 12: The power class of intra-band CA is allowed to be different from the band power class.

Proposal 13: UE should be allowed to report actual and dynamic power class to realize the maximum output power for CA.

Conclusions

In this paper, we provide our initial views on 6G UE RF output power requirements, we have made the following proposals.

Proposal 1: Regarding the present situation and future challenges, we can define the step as 1dB or 2dB) above PC1.5 in 6GR. Try to find an approach like MSD to define a look up table to derive the requirements, including MPR/AMPR, REFSENS and other requirements.

Observation1: The pros are that it is helpful for network to obtain the UE capability of every single Tx chain to schedule for single layer or boosting the accuracy of beamforming for UL MIMO. The cons are more flexible for network to schedule, but it may cause the scheduled which is not wanted by UE.

Proposal 2: Tx diversity, full power mode and UL MIMO in 5G can be reused in 6GR. UE report power class of every Tx chain to NW can be considered.

Proposal 3: Transparent TxD is still only kept in 6GR.

Proposal 4: ∆TC,c can be optimized rather than remove it in 6GR.

Proposal 5: ΔPPowerClass can be considered to remove if Duty cycle SAR will be removed.

Proposal 6: ΔTIB,c needs to be considered in 6GR. Simplification of ΔTIB,c requirement can be FFS.

Proposal 7: For ∆TRxSRS, it can be separated from general PCMAX equation (e.g., PCMAX,SRS).Whether PRACH/PUCCH needs to be set independently align to the RAN1 spec can be FFS.

Proposal 8: For FDD bands, PC3 can still be the default power class. For around 7GHz, PC2 can be the default power class. For TDD bands in FR1, the default class can be further discussed.

Proposal 9: It is not necessary to consider duty-cycle for SAR solution in 6GR. We can keep P-MPR as a SAR solution in 6GR.

Proposal 10: SAR can be applied in below 6GHz, MPE (PD) can be applied in above 6GHz in 6GR.

Proposal 11: Remove the total power class definition for band combinations in 6GR.

Proposal 12: The power class of intra-band CA is allowed to be different from the band power class.

Proposal 13: UE should be allowed to report actual and dynamic power class to realize the maximum output power for CA.

- Reference

- R4-2522451, WF for [117][102] 6G general RF and UE RF, Qualcomm. TSG-RAN4#117 meeting