3GPP TSG RAN WG4 Meeting #118		R4-2600266

Gothenburg, Sweden, Feb 9th – 13th , 2026

Title: 	Discussion on UE RF Tx/Rx requirements for 6G study

Agenda Item:	8.3.2

Source: 	CATT

Document for:	Approval

1 Introduction

In the RAN4#117 meeting, a WF [1] on General RF and UE RF for 6G study was approved. In this paper, we’d like to discuss the issues for UE Tx/Rx requirements.

Topic #3: Tx requirements - Agreements - Further study possibility for relaxation and applicable measurement bandwidth for 1st MHz of SEM - Evaluate the benefit of increasing the measurement bandwidth - Consider different CBW - Encourage input on SEM regulation - Further study Tx requirements and impairment assumptions and requirements considering - LO is placed in the middle of the frequency span used for deriving requirements - …… - Whether to consider APT and/or ET PA - …… - Whether to consider wider OOB boundary - FFS on modifying SEM based on wider OOB boundary and evaluating MPR based on wider OOB boundary - Encourage inputs on improvements on MPR requirement framework as well as MPR/A-MPR evaluation process improvements

Topic #4: Rx requirements - Agreements - Further study reference sensitivity including NF for different bands and/or frequency sub-ranges considering at least - Whether NF and implementation margin can be reduced taking into account implementation complexity - Encourage companies to provide numerical evaluations - Consider impact on other Rx requirements which have been defined relative to reference sensitivity - Study alternatives of determining UL RB allocation size and position for FDD bands considering at least following options - 1) UL allocation is set so that Tx noise is substantially below Rx noise floor - 2) Evaluation of component performance and Tx noise in both main and diversity receivers with the aim to define practical ways to define UL allocation - For both options - Consider impact to RSD for different power classes - Consider that REFSENS should meaningfully test UEs minimum performance - Further study whether ACS, blocking and wideband intermodulation test parameters can be more unified below 2700 MHz and above 3300 MHz and whether there is a need to define narrowband blocking requirement. - The plan for MSD related study - Step 1: Discuss the MSD and the related requirements, including if MSD will be introduced in 6G in 6G UE RF thread - Step 2: Based on the outcome of step 1, it is further discussed in UE RF thread if OTA test, should be considered. - Step 3: if the answer is Yes in step 2, the feasibility and other aspects, e.g. cost, of OTA test will be discussed under 6G testability and OTA thread. - Step 4: if the feasibility is confirmed in step 3, OTA measurement discussion will start. It is FFS if this should be discussed in 6G UE RF or OTA thread. - NOTE: the discussion plan in OTA thread and UE RF thread should be coordinated. - Further study how to define meaningful and useful MSD requirements with the target of having them widely available 6G day1 considering also usefulness for infra and operators. Further study possibilities to simplify and improve MSD framework considering e.g. - Strive to use 5G requirements as baseline - Simplifying minimum requirement organization using frequency grouping and impairment orders - RF parameter assumptions - Further study benefits and usefulness of MSD reporting schemes as an addition to MSD minimum requirements - Encourage further input from infra and operators on what kind of reporting could be useful and potentially taken into account in scheduling, e.g. reporting MSD separately when UE is transmitting and not transmitting

2 Discussion

2.1 UE Tx impairments and unwanted emission framework

In the last meeting, companies are encouraged to study the possibility for relaxation and applicable measurement bandwidth for 1st MHz of SEM

According to the Table 6.6.2-1 of TR 36.803 and LTE discussion paper [2], the requirements of 1st MHz of SEM came from FCC regulation FCC Title 47 part 24.238 and FCC Title 47 part 24.238.

### § 24.238 - Emission limitations for Broadband PCS equipment. The rules in this section govern the spectral characteristics of emissions in the Broadband Personal Communications Service. (a) Out of band emissions. The power of any emission outside of the authorized operating frequency ranges must be attenuated below the transmitting power (P) by a factor of at least 43 + 10 log(P) dB. (b) Measurement procedure. Compliance with these rules is based on the use of measurement instrumentation employing a resolution bandwidth of 1 MHz or greater. However, in the 1 MHz bands immediately outside and adjacent to the frequency block a resolution bandwidth of at least one percent of the emission bandwidth of the fundamental emission of the transmitter may be employed. A narrower resolution bandwidth is permitted in all cases to improve measurement accuracy provided the measured power is integrated over the full required measurement bandwidth (i.e. 1 MHz or 1 percent of emission bandwidth, as specified). The emission bandwidth is defined as the width of the signal between two points, one below the carrier center frequency and one above the carrier center frequency, outside of which all emissions are attenuated at least 26 dB below the transmitter power. (c) Alternative out of band emission limit. Licensees in this service may establish an alternative out of band emission limit to be used at specified band edge(s) in specified geographical areas, in lieu of that set forth in this section, pursuant to a private contractual arrangement of all affected licensees and applicants. In this event, each party to such contract shall maintain a copy of the contract in their station files and disclose it to prospective assignees or transferees and, upon request, to the FCC. (d) Interference caused by out of band emissions. If any emission from a transmitter operating in this service results in interference to users of another radio service, the FCC may require a greater attenuation of that emission than specified in this section. [67 FR 77192, Dec. 17, 2002] AUTHORITY: 47 U.S.C. 154, 301, 302a, 303, 309 and 332. SOURCE: 58 FR 59183, Nov. 8, 1993, unless otherwise noted. Redesignated at 59 FR 18499, Apr. 19, 1994. CITE AS: 47 CFR 24.238 § 22.917 - Emission limitations for cellular equipment. The rules in this section govern the spectral characteristics of emissions in the Cellular Radiotelephone Service. (a) Out of band emissions. The power of any emission outside of the authorized operating frequency ranges must be attenuated below the transmitting power (P) by a factor of at least 43 + 10 log(P) dB. (b) Measurement procedure. Compliance with these rules is based on the use of measurement instrumentation employing a reference bandwidth as follows: (1) In the spectrum below 1 GHz, instrumentation should employ a reference bandwidth of 100 kHz or greater. In the 1 MHz bands immediately outside and adjacent to the frequency block, a resolution bandwidth of at least one percent of the emission bandwidth of the fundamental emission of the transmitter may be employed. A narrower resolution bandwidth is permitted in all cases to improve measurement accuracy, provided that the measured power is integrated over the full required reference bandwidth (i.e., 100 kHz or 1 percent of emission bandwidth, as specified). The emission bandwidth is defined as the width of the signal between two points, one below the carrier center frequency and one above the carrier center frequency, outside of which all emissions are attenuated at least 26 dB below the transmitter power. (2) In the spectrum above 1 GHz, instrumentation should employ a reference bandwidth of 1 MHz. (c) Alternative out of band emission limit. Licensees in this service may establish an alternative out of band emission limit to be used at specified band edge(s) in specified geographical areas, in lieu of that set forth in this section, pursuant to a private contractual arrangement of all affected licensees and applicants. In this event, each party to such contract shall maintain a copy of the contract in their station files and disclose it to prospective assignees or transferees and, upon request, to the FCC. (d) Interference caused by out of band emissions. If any emission from a transmitter operating in this service results in interference to users of another radio service, the FCC may require a greater attenuation of that emission than specified in this section. [67 FR 77191, Dec. 17, 2002, as amended at 82 FR 17583, Apr. 12, 2017] AUTHORITY: 47 U.S.C. 154, 222, 303, 309, and 332. SOURCE: 59 FR 59507, Nov. 17, 1994, unless otherwise noted. CITE AS: 47 CFR 22.917

| CBW | -13dBm per 1%*CBW | Comparison between 1st MHz emission and -10dBm/MHz (1 ~ 5MHz) |
| --- | --- | --- |
| 1.4MHz | -10dBm/30kHz=5dBm/MHz | +15dB |
| 3MHz | -13dBm/30kHz=2dBm/MHz | +12dB |
| 5MHz | -13dBm/50kHz = 0dBm/MHz | +10dB |
| 10MHz | -13dBm/100kHz=-3dBm/MHz | +7dB |
| 20MHz | -13dBm/200kHz=-6dBm/MHz | +4dB |
| 40MHz | -13dBm/400kHz=-9dBm/MHz | +1dB |
| ≥50MHz | -24dBm/30kHz=-9dBm/MHz | +1dB |

It can be observed that the 1st MHz emission for 1.4MHz is most relaxed. Comparing between 1.4MHz and 40MHz, the 1st MHz emission can be varied in 14dB range. As the 1st MHz emission for smaller CBW can guarantee the coexistence based on the real deployment, it’s unclear why we tight this requirements for the larger CBWs.

Proposal 1: to relax the 1st MHz emission requirements for 6GR SEM considering the following options.

Option 1: -13dBm/30kHz in the 1st MHz for CBW equal to and larger than 3MHz.

Option 2: -13dBm/50kHz in the 1st MHz for CBW equal to and larger than 5MHz.

Option 3: -13dBm/200kHz in the 1st MHz for CBW equal to and larger than 20MHz.

Option 4: Other options are not precluded.

For the Tx impairments from Quadrature Modulation in the transceiver, carrier leakage, image rejection, CIM3 and CIM5 will be further studied. These factors will affect the UL EVM. In current specification, the UL EVM requirements, carrier leakage and image leakage requirements are shown below.

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 |
| QPSK | % | 17.5 |
| 16 QAM (Mandatory in LTE) | % | 12.5 |
| 64 QAM (Mandatory in NR) | % | 8 |
| 256 QAM (Mandatory in 6GR?) | % | 3.5 |

| IQ Image | dB | -28 | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
| --- | --- | --- | --- | --- |
|  |  | -25 | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm | Carrier leakage frequency (NOTES 4, 5) |
|  |  | -25 | 0 dBm ≤ Output power ≤ 10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power < 0 dBm |  |
|  |  | -10 | -40 dBm ≤ Output power < -30 dBm |  |

In NR phase, 64QAM was approved as UE mandatory capability. In the 6GR phase, if it’s agreed to mandatorily support 256QAM, the carrier leakage and image leakage requirements above 10dBm output power should be adjusted to meet total 3.5% EVM budget of UE Tx chain. Current -28dBc IQ image and carrier leakage above 10dBm output power are not enough to meet total 3.5% EVM budget of UE Tx chain.

Generally, RAN4 assume 60dB CIM3 and 70dB CIM5 for MPR simulation. As new frequency range should be studied, e.g. around 7GHz and 15GHz, RAN4 need to discuss whether 60dB CIM3 and 70dB CIM5 for MPR simulation are still valid or necessary for MPR simulation in around 7GHz and 15GHz.

Proposal 2: For UE Tx impairments, the following parts are proposed.

P2-1: In the 6GR phase, if it’s agreed to mandatorily support 256QAM, the carrier leakage and image leakage requirements above 10dBm output power should be adjusted to meet total 3.5% EVM budget of Tx chain.

P2-2: RAN4 need to discuss whether 60dB CIM3 and 70dB CIM5 for MPR simulation are still valid or necessary for MPR simulation in around 7GHz and 15GHz.

2.2 UE Rx requirements

As the maximum Channel bandwidth for 6GR may reach 200MHz or 400MHz and minimum Channel bandwidth may be 3MHz/5MHz/10MHz, the REFSENS requirements will be tested with a lot of different channel bandwidths. In NR, we have the following different channel bandwidths for different SCS.

15kHz: 3MHz, 5MHz, 7MHz, 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 35MHz, 40MHz, 45MHz, 50MHz. (12 CBWs)

30kHz: 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 35MHz, 40MHz, 45MHz, 50MHz, 60MHz, 70MHz, 80MHz, 90MHz, 100MHz (14 CBWs)

This leads to a huge RMC tables and test burden for UE. RAN4 can consider to refer the BS method and just specify some limited fixed reference channels for REFSENS testing.

Proposal 3: referring to the BS REFSENS definition, it’s recommended to only specify some limited fixed reference channels for UE REFSENS testing.

In the last meeting, RAN4 has the following WF for FDD band REFSENS.

- Study alternatives of determining UL RB allocation size and position for FDD bands considering at least following options - 1) UL allocation is set so that Tx noise is substantially below Rx noise floor - 2) Evaluation of component performance and Tx noise in both main and diversity receivers with the aim to define practical ways to define UL allocation

We can consider option 1 to set some proper configurations so that Tx noise is substantially below Rx noise floor, e.g. appropriate UL RB allocation, lower output power and using TDM scheduling between UL slot and DL slot.

Proposal 4: For FDD band REFSENS, some proper configurations could be set so that Tx noise is substantially below Rx noise floor, e.g. appropriate UL RB allocation, lower output power and using TDM scheduling between UL slot and DL slot.

3 Summary

2.1 UE Tx impairments and unwanted emission framework

Proposal 1: to relax the 1st MHz emission requirements for 6GR SEM considering the following options.

Option 1: -13dBm/30kHz in the 1st MHz for CBW equal to and larger than 3MHz.

Option 2: -13dBm/50kHz in the 1st MHz for CBW equal to and larger than 5MHz.

Option 3: -13dBm/200kHz in the 1st MHz for CBW equal to and larger than 20MHz.

Option 4: Other options are not precluded.

Proposal 2: For UE Tx impairments, the following parts are proposed.

P2-1: In the 6GR phase, if it’s agreed to mandatorily support 256QAM, the carrier leakage and image leakage requirements above 10dBm output power should be adjusted to meet total 3.5% EVM budget of Tx chain.

P2-2: RAN4 need to discuss whether 60dB CIM3 and 70dB CIM5 for MPR simulation are still valid or necessary for MPR simulation in around 7GHz and 15GHz.

2.2 UE Rx requirements

Proposal 3: referring to the BS REFSENS definition, it’s recommended to only specify some limited fixed reference channels for UE REFSENS testing.

Proposal 4: For FDD band REFSENS, some proper configurations could be set so that Tx noise is substantially below Rx noise floor, e.g. appropriate UL RB allocation, lower output power and using TDM scheduling between UL slot and DL slot.

Reference

[1] R4-2522451	WF on [117][102] 6G general RF and UE RF, Feature lead (Qualcomm)

[2] R4-071080, Alternative LTE Spectrum Emission Mask (SEM) methodology, Motorola