3GPP TSG RAN WG4 Meeting #118		R4-2600289

Gothenburg, Sweden, Feb 9th – 13st, 2026

Title: 	Discussion on improvement of radiated performance metric in 6G OTA

Agenda Item:	8.11.1

Source: 	CATT

Document for:	Discussion

1	Introduction

In 6G Testability and OTA related discussions, RAN4 has reached some general agreements. Meantime, many issues in the radiated performance metric aspects still require further discussion. In this paper we are sharing our views on the radiated performance metrics for NTN OTA, SISO OTA and MIMO OTA in 6G.

2	Discussion

2.1	NTN radiated performance metric

As described in the WID document of TRP_TRS_MIMO_OTA_Ph3 WI, the study on OTA performance metrics for FR1-NTN bands was initiated based on the following considerations:

In addition, the current 3GPP RF Requirements for FR1-NTN bands are only based on conductive tests. However, there is considerable variability in antenna implementations and resulting performance for NTN devices available in the market. Additionally, the antenna of NTN devices are primarily designed for satellite tracking with beamforming, which significantly differs from traditional terrestrial networks (TN). Consequently, the conventional FR1 UE OTA performance metrics, such as TRP and TRS, which assume an omni-directional antenna pattern, may not be suitable for NTN devices. As a result, it is necessary to define new performance metrics and corresponding test methodologies for NTN devices in Rel-19.

For NTN device, clearly radiated performance metric more closely reveals the real-world usage scenarios. In 6G, RAN4 may evaluate the pros and cons to adopt radiated performance metric as primary metric for FR1 NTN bands.

Proposal 1:	For FR1 NTN bands in 6G, RAN4 consider evaluating the pros and cons to adopt radiated performance metric as primary metric.

As for FR2 NTN bands, the defined performance requirements and test metric could be summarized as below:

Table 1 Radiated performance requirements and test metrics for Ku-band and Ka-band

|  | Characteristics | Test metric |
| --- | --- | --- |
| Tx | maximum output power: EIRP, TRP | EIRP(Link=Tx beam peak direction, Meas=Link angle), TRP |
|  | Transmit OFF power | TRP (Link=TX beam peak direction, Meas=TRP grid) |
|  | Transmit ON/OFF time mask | EIRP(Link=TX beam peak direction, Meas=Link angle) |
|  | Frequency Error |  |
|  | Error vector magnitude |  |
|  | Occupied bandwidth |  |
|  | Spectrum emission mask | TRP (Link=TX beam peak direction, Meas=TRP grid) |
|  | Adjacent channel leakage ratio |  |
|  | Spurious Emissions |  |
|  | On-axis spurious requirement | EIRP(Link=TX beam peak direction, Meas=Link angle) |
|  | Off-axis spurious requirement |  |
|  | “Carrier-on” and “Carrier-off” states |  |
| Rx | OTA reference sensitivity level | EIS (Link=RX beam peak direction, Meas=Link Angle) |
|  | Maximum input level |  |
|  | Adjacent channel selectivity |  |
|  | Blocking characteristics |  |

Some characteristics requirements are defined based on TRP test metric. For FR2 NTN UE (VSAT) antennas which are always sky-pointing, only measuring the upper-hemisphere performance should be sufficient. Just as the partial radiated metrics defined for FR1 NTN scenario 2 in [2], FR2 NTN performance requirements could adopt PRP metric in place of TRP metric in 6G.

Proposal 2:	For FR2 NTN bands in 6G, adopt partial radiated metrics in place of total radiated metric.

2.2	SISO OTA radiated performance metric

FR1 handheld UEs could support up to PC2 with 1Tx antenna. FWA could even supports PC1 with 1Tx antenna. With the development of industry, more and more bands are supporting high power class. In 6G,we should confront this trend and study the OTA methodologies for high power class verification. To avoid redundant testing, UE only need to test the highest supported power class.

Proposal 3:	Study the OTA test methodologies for high power class for FR1 UEs. And only performance related with the highest supported power class need to be verified.

2.3	Multi Tx radiated performance metric

In FR1 OTA test aspects, the development of test methodology and performance requirements for UL SISO is much more advanced than UL TxD. Actually in UE conformance conductive testing, for a FR1 single carrier, the UE will either perform the UL SISO or the UL TxD test case based on its own capability, but never does the both.

Taking MOP testing as an example, the test applicability for SISO test case 6.2.1 and TxD test case 6.2G.1 in [3] are extracted as below:

<SISO MOP test case> 6.2.1 UE maximum output power …… 6.2.1.2 Test applicability This test case applies to all types of NR Power Class 1 UE release 15 and forward. This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity and don't support Redcap. <TxD MOP test case> 6.2G.1 UE maximum output power for Tx Diversity …… 6.2G.1.2 Test applicability This test case applies to all types of NR Power Class 1.5 FWA UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

Observation 1：In FR1 conducted conformance testing, UL SISO and TxD test cases are in parallel. UE will perform either SISO test case or TxD test case for each characteristics based on its own capability.

As mentioned in section 2.2, the needs for testing higher power class is growing with the development of industry. Some UE could only transmit PC2 via 2Tx antennas. And PC1.5 could only be achieved via TxD (single port single layer scenario). Before initiating the study of OTA methodology for high-power related mechanisms, the OTA testing for TxD should be studied first. Thus in 6G, the study of TxD radiated performance metric should be considered as high priority.

Observation 2：Developing the TxD test methodology is the essential first step for validating higher power classes such as PC1.5

Proposal 4:	In 6G, the study of TxD radiated performance metric should be considered as high priority.

3	Conclusion

In this paper, we submit following proposals on the consideration of OTA performance metric improvements in 6G:

Proposal 1:	For FR1 NTN bands in 6G, RAN4 consider evaluating the pros and cons to adopt radiated performance metric as primary metric.

Proposal 2:	For FR2 NTN bands in 6G, adopt partial radiated metrics in place of total radiated metric.

Proposal 3:	Study the OTA test methodologies for high power class for FR1 UEs. And only performance related with the highest supported power class need to be verified.

Observation 1：In FR1 conducted conformance testing, UL SISO and TxD test cases are in parallel. UE will perform either SISO test case or TxD test case for each characteristics based on its own capability.

Observation 2：Developing the TxD test methodology is the essential first step for validating higher power classes such as PC1.5

Proposal 4:	In 6G, the study of TxD radiated performance metric should be considered as high priority.

Reference

[1]	RP-252187	WI on FR1 TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3

[2]	TR 38.870 Technical Specification Group Radio Access Network; Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS) (Release 19)

[3]	TS 38.521-1 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone