3GPP TSG-RAN WG4 Meeting #118	                                                                                            R4-2600294

Göteborg, SE, 9th –13th Feb, 2026

Source: 	CATT

Title: 	Discussion on UE RF requirements for Ku band HD-FDD VSAT

Agenda item:	7.6.4.1

Document for:	Approval

- Introduction

In RAN4 #117 meeting, the WF [1] for NTN and IoT NTN phase 2 HD-FDD VSAT has been approved. However, there remain some issues need to be discussed. This paper shares some views on NTN and IoT NTN phase 2 HD-FDD VSAT.

- Discussion

In RAN4 #117 meeting, a WF for HD-FDD VSAT has been approved, as shown below:

- Topic #1: UE RF requirements for HD-FDD for Ku band with FR1 numerology ### Sub-topic 1-1: Issues for HD-FDD requirements #### Issue 1-1-1: Regulation requirements for fixed VSAT (type 2 and 3) - WF - Reflect the 50 dBW maximum EIRP limit for NTN VSAT operating in NTN CEPT Ku-band(s) 14.25 – 14.5 GHz from ECC Decision(03)04 in the Maximum output power limits for Fixed VSAT table. - The requirements related to NS_203N, NS_204N, NS_207N, and NS_208N apply to fixed VSAT. - The off-axis EIRP requirements related to NS_205N apply to NTN Ku fixed VSAT operating in CEPT regions. - Further investigate the on-axis spurious, off-axis spurious and receiver off-axis gain pattern requirements for NTN Ku fixed VSAT operating in CEPT regions, with consideration of EN 301 428, EN 302 448 at least. #### Issue 1-1-2: Transmit ON/OFF time mask for HD-FDD - Agreement - Reuse the existing Ku-band FR1 FDD Transmit ON/OFF time mask requirement for HD-FDD #### Issue 1-1-3: Treatment on HD-FDD specifications. - Agreement - No SAN RF impact has been observed by introducing HD-FDD VSAT. #### Issue 1-1-4: EIRP requirements for HD-FDD - Agreement - Reuse the existing FDD VSAT minimum EIRP requirements for HD-FDD for VSAT types 2, 3, 5 and 6 - Reuse the existing FDD VSAT maximum EIRP requirements for HD-FDD for VSAT types 2, 3, 5 and 6 #### Issue 1-1-5: EIS requirements for HD-FDD - Agreement - Reuse the existing FDD VSAT EIS requirements for HD-FDD for VSAT types 2, 3, 5 and 6 - Reuse the existing FDD VSAT maximum input level for HD-FDD for VSAT types 2, 3, 5 and 6 ### Sub-topic 1-2: Issues for VSAT with small antennas (e.g., 20x20cm) #### Issue 1-2-3: New VSAT type - WF - Study potential new VSAT type(s) for small antenna with the following candidates: - VSAT that support LEO only - VSAT that support both GSO and LEO #### Issue 1-2-4: System parameters alignments for VSAT with small antenna - WF - Take the following VSAT parameters as a starting point for 20x20cm electronically steered panel, other values are not precluded 1; VSAT Antenna Characteristics. 1.1; Antenna pattern; TR 38.921. 1.2; Element gain (dBi) (Note 1); [3.5]. 1.3; Y axis / Z axis element 3 dB beam width of single element (degree); 90° for Y axis 90° for Z axis. 1.4; Y axis / Z axis element front-to-back ratio (dB); 30 dB. 1.5; Antenna polarization (Note 5); Circular (RHCP or LHCP). 1.6; Antenna array configuration (Y axis × Z axis)(Note 3); [15x15], [16x16], [17x17] elements. 1.7; Number of supported polarizations, P; 1. 1.8; Y axis / Z axis radiating element spacing; [0.5] lambda. 1.9; Array Ohmic loss (dB) (Note 1); 2. 1.10; Conducted power (before Ohmic loss) per antenna element (dBm) (Note 2); [4-15]. 1.11; Maximum coverage angle in the horizontal plane (degrees); 0~360 degrees. 1.12; Vertical coverage range (degrees) between beam direction and normal direction; 0~60 degrees. 1.13; Normal direction; Toward X+ axis. 1.14; Noise Figure; [6] dB. Note 1: The element gain in row 1.2 includes the loss given in row 1.9. Note 2: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 3: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 4: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). Note 5: Antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization evaluations are conducted by converting to linear polarization. #### Issue 1-2-5: System parameters alignments for SAN aspects - WF - Take the following satellite parameters as a starting point, other values are not precluded Satellite orbit; GEO; LEO-1200; LEO-600. Satellite altitude; 35786 km; 1200 km; 600 km. Satellite EIRP density; Ku-band (i.e. 11 GHz for DL); 43 dBW/MHz; 13 dBW/MHz; 7 dBW/MHz. Satellite RX max Gain; Ku-band (i.e. 14 GHz for UL); 44 dBi; 38.5 dBi; 38.5 dBi. #### Issue 1-2-6: Assumptions for VSAT with small antenna for LEO - WF - Study antenna size with 20×20cm with the following candidates: - 17 × 17 antenna elements - 16 × 16 antenna elements - Other antenna elements are not precluded. #### Issue 1-2-7: New VSAT with small antenna to support LEO - WF - Encourage more companies to analyze the following RF requirements based on the updated system parameters (Issue 1-2-4 & 5): - Minimum peak EIRP - Maximum EIRP - Maximum TRP - EISREFSENS_50MHz #### Issue 1-2-8: New VSAT with small antenna to support both GSO and LEO - WF - Further study on how to support both GSO and LEO for VSAT with small antennas size with the following consideration as the starting point - Antenna size around 30cm × 30cm - Consideration of small channel BW support (e.g. 3, 5MHz) #### Issue 1-2-9: ACLR and ACS for the new VSAT with small antenna - Agreement - Reuse the existing Ku-band ACLR and ACS requirements for small antenna VSAT. #### Issue 1-2-10: Regulations for the new VSAT with small antenna - Agreement - Consider whether to exclude the non-conformance testable “off-axis gain limit“ and “Rx antenna off-axis performance” requirements for new VSAT types with small antenna for LEO only and for both GSO and LEO or not.

2.1 General

The WID is aimed to introducing NTN HD-FDD for Ku band in phase 1. Firstly, the applicability of HD-FDD VSAT requirements should be discussed. Considering the demands of stakeholders for HD-FDD VSAT, the introduction of the HD-FDD architecture is actually mainly aimed at reducing the implementation difficulty of the NTN VSAT, which does not imply a reduction in the NTN VSAT capability. Meanwhile, considering that the requirements defined for Ku band in FR1 are all OTA requirements, while currently OTA requirements do not include content of RedCap. Therefore, when defining requirements for HD-FDD VSAT, only the impact caused by the HD-FDD feature should be considered, rather than regarding HD-FDD VSAT as a RedCap device by default.

Observation 1: For NTN VSAT operating in Ku band, the HD-FDD VSAT is not a RedCap VSAT.

2.2 HD-FDD VSAT requirements

During the previous meeting, RAN4 discussed the regulation related requirements for fixed VSAT. Several companies indicated that most regulation related requirements could be reused, covering NS_203N to NS_208N. In general, we consider this approach to be reasonable. The signalling reflects the regulatory requirements from both the FCC and the ECC. For FCC related regulation requirements, including NS_203N, NS_204N, NS_207N, and NS_208N, since they do not specify VSAT type, we believe they can be directly applied to fixed VSAT.

Meanwhile, regarding ECC related regulation requirements (NS_205N and NS_206N), some regulation requirements that explicitly applies only to mobile VSAT. However, it is worth noting that these requirements were defined by adopting the more stringent values applicable to either all VSAT or mobile VSAT. Therefore, we do not foresee any potential harm in extending their application to fixed VSAT.

Proposal 1: The regulatory requirements associated with NS_203N through NS_208N could be reused for the fixed VSAT operating in the n248 and n247.

- Conclusion

This contribution provides our initial views on RF requirements for HD-FDD VSAT. The following proposals are concluded as follows:

Observation 1: For NTN VSAT operating in Ku band, the HD-FDD VSAT is not a RedCap VSAT.

Proposal 1: The regulatory requirements associated with NS_203N through NS_208N could be reused for the fixed VSAT operating in the n248 and n247.

- References

- R4-2522970, Way Forward for [117][330] NR_IoT_NTN_Ph2_Part2, MTK, RAN4 #117