3GPP TSG RAN4 Meeting #118							R4-2600386

Gothenburg, Sweden, 9th – 13th February 2026

Source:	Nokia

Title:	6GR System parameters - Modulation

Agenda item:	8.2.2

Document for:	Approval

Introduction

Under 6G system parameters AI for Modulation following text was captured into WF [1].

- Recommendation from FL based on inputs for RAN4#117

  - Evaluation Cases:

    - Case 1 (Baseline): Existing NR modulations (BPSK to 256QAM) with new 6G PA model(s).

    - Case 2 (Higher-order modulation):

      - UL 1024QAM: Primarily focusing on FWA UE implementation feasibility.

        - Already agreed to start parallel study in last RAN4 meeting

      - DL 4096QAM: Focusing on both BS and UE implementation feasibility.

        - Whether and when to consider it as an optional feature for study, pending on further RAN4 discussion and decision

    - Case 3 (Constellation shaping): Defer detailed evaluation until RAN1 conclusions are stable.

  - Evaluation assumptions:

    - PA Model: Depends on the discussion progress on 6G PA model. This is a foundational assumption for all modulation studies.

    - EVM budget: Define a clear EVM budget for higher order modulation (UL 1024QAM, DL 4096QAM), considering all impairment sources (PA non-linearity, I/Q imbalance, phase noise, CFR, etc.).

    - Scenarios and frequencies: Focus evaluations on agreed scenarios (TBD, like Urban Macro and indoor hotspot), across agreed frequencies (TBD, like ~700 MHz, 2 GHz, and 7 GHz).

    - Bandwidth: Consider wider channel bandwidths (TBD, e.g., 200 MHz)

- WF

  - FFS in next meeting

Discussion

Both LTE (4G) and NR (5G) use OFDM-based waveforms with uniform QAM modulation for both UL and DL. Hence, there is already a long experience in utilizing these modulations with these waveforms for several use cases and frequency bands with higher order multi-antenna processing.

RAN1#122 in August 2025 made the following agreements on modulation for 6G:

Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

The agreement marked a positive step forward, however, the phrase "as basis for study" lacks practical clarity regarding the status of the mentioned modulation schemes for 6G. It does not explicitly delineate whether these modulations are intended as supported baseline functionalities or solely as benchmarks for future evaluations. Nevertheless, the consistent inclusion of "FFS: Enhancements and other modulation schemes" as a sub-bullet implicitly suggests that these modulations are indeed presupposed as supported baseline modulations for 6G. Therefore, we advocate that further study on modulation should concentrate on optional additions and enhancements to these assumed baseline modulations.

Proposal 1:

- For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported for data channel

- For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported for CP-OFDM for data channel

- For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported for DFT-s-OFDM for data channel

- FFS: Enhancements and other modulation schemes

For 6G devices, the UE’s baseline power class is expected to increase. The same is valid for the number of receive antennas at the base station. This means, compared with 5G NR, we may observe an improved SNR at the base station which makes uniform 1024QAM in UL a useful enhancement compared to 5G NR.

Proposal 2: Support uniform 1024QAM in Uplink.

On the other hand, 4096QAM in DL will strongly drive the computation complexity and the power consumption of the R-ML MIMO detection at the UE. It remains to be justified that such increase in implementation cost and power consumption is reasonable.

EVM requirements for various QAM modulation schemes are shown in Table 1. Notice how the EVM requirements are more stringent, the higher the modulation order is. EVM requirement for 4096QAM would then be far beyond that of 1024QAM.

Table 1: EVM requirements for BS type 1-C and BS type 1-H carrier.

Figure 1 shows the throughput of 4096QAM compared with 1024QAM for different EVM values (at Tx and Rx) for a code rate 0.9. We notice:

- For Tx-EVM=Rx-EVM=0%, a minimum SNR of approximately 32 dB is required such that the throughput of 4096QAM exceeds that of 1024QAM.

- At the same non-zero EVM values, the required SNR is further increased.

Figure 1: Throughput of 4096QAM vs. 1024QAM for different EVM values (Tx and Rx) for TDL-D 10 ns. 4GHz, CBW=100MHz, scs=30kHz, full RB allocation, low correlation, Type A mapping, start symbol 1, pos1. HARQ enabled.

Observation 1: Uniform 4096QAM in DL drives computation complexity and the power consumption of R-ML MIMO detection at the UE side.

Observation 2: Very low EVM is required for uniform 4096QAM in DL.

Conclusion

In this contribution we present our view on 6GR modulation. The following proposals and observations have been made:

Proposal 1:

- For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported for data channel

- For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported for CP-OFDM for data channel

- For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported for DFT-s-OFDM for data channel

- FFS: Enhancements and other modulation schemes

Proposal 2: Support uniform 1024QAM in Uplink.

Observation 1: Uniform 4096QAM in DL drives computation complexity and the power consumption of R-ML MIMO detection at the UE side.

Observation 2: Very low EVM is required for uniform 4096QAM in DL.

- 4	References

R4-2522450, WF on 6GR system parameter, RAN4#117, Feature lead