# 3GPP TSG-RAN WG4 Meeting #118	R4-2600453

# Gothenburg, Sweden, Feb. 09-13, 2026

Source: 	Xiaomi

Title: 	Discussion on RRM core requirements of E-UTRA TN to NR NTN handover enhancements

Agenda Item:	7.15.2

Document for:	Discussion

# 1Introduction

The Rel-20 WI of E-UTRA TN to NR NTN handover enhancements [1] has been approved in RANP#108. Based on the arrangement in the TU budget, RAN4 discussion on the related RRM requirements can start from this February meeting.

Assumptions - GSO (Geo Synchronous Orbit) and NGSO (Non-Geo Synchronous Orbit). NGSO includes Low Earth Orbit (LEO) and Medium Earth Orbit (MEO) - Earth fixed tracking area. Quasi-Earth fixed & Earth moving cells for NGSO - NTN-FR1 band is considered in priority - UEs with GNSS capability - Support of Multiple SMTCs is not within the scope Objectives - Specify necessary enhancements on inter-RAT measurements to support E-UTRA TN to NR NTN handover in RRC connected mode [RAN2] - This includes the definition of necessary satellite information to be provided by E-UTRA TN for NR NTN neighbor cell measurement (e.g. polarization information, ephemeris data, etc.) in NR specific measurement configurations. - Note: For this objective, the solutions shall be reusing the existing signalling for inter-RAT measurements to NR in TS 36.331 and the related satellite assistance information specified in TS 38.331 for NR NTN. - Specify necessary RRM requirements for mobility scenario of "E-UTRA TN to NR NTN" in CONNECTED mode. [RAN4]

According to the WID, RAN4 is responsible for defining the RRM requirements for mobility scenario of "E-UTRA TN to NR NTN" in CONNECTED mode. In this contribution, we will provide our initial consideration.

- Discussion

From RRM requirements point of view, to support the mobility scenario of "E-UTRA TN to NR NTN" in CONNECTED mode, RAN4 has to specify both related measurement and handover requirements. Based on the assumption in the WID, NTN-FR1 band is considered in priority. The discussion on FR1-NTN should be considered firstly.

Proposal 1: RAN4 to define the following requirements for the mobility scenario of "E-UTRA TN to NR NTN" in CONNECTED mode:

- Requirements for inter-RAT NR Measurements for FR1-NTN

- Requirements for E-UTRA TN to NR FR1-NTN handover

For inter-RAT NR Measurements for FR1-NTN, the requirements for NR inter-frequency measurements for FR1-NTN defined in TS 38.133 section 9.3C could be taken as baseline, as the target cell is NTN cell. However, it should be noticed that the UE GNSS may be switched OFF before the first time of NTN carrier measurement. Similar as the discussion on TN to NTN cell reselection scenario, the GNSS time to first fix should be taken into consideration. From our side, RAN4 could reuse the agreements achieved for TN to NTN cell reselection:

#### Issue 4-1: TN to NTN cell reselection Agreement in RAN4#109: - Define requirements on TN to NTN cell reselection. - Define core requirements for GNSS ON and GNSS switch OFF to ON, no test case. - No specific value for the GNSS time to first fix to be define for the case of GNSS switch OFF to ON.

Proposal 2: For inter-RAT NR Measurements for FR1-NTN, RAN4 to take the requirements for NR inter-frequency measurements for FR1-NTN defined in TS 38.133 section 9.3C as baseline, and study the GNSS status impact.

For E-UTRA TN to NR FR1-NTN handover, the requirements for NR SAN FR1 – NR SAN FR1 Handover defined in TS 38.133 section 6.1C.1.2 could be taken as baseline. Similarly, the GNSS time to first fix should be considered. Basically, the handover requirements are divided into known and unknown cases. In our understanding, for known case, UE have prior information on the target NTN carrier, there is no need to consider the GNSS time. However, for unknown case, the requirements should additionally take the GNSS time to first fix into consideration.

Proposal 3: For E-UTRA TN to NR FR1-NTN handover, RAN4 to take the requirements for NR SAN FR1 – NR SAN FR1 Handover defined in TS 38.133 section 6.1C.1.2 as baseline, and study the GNSS status impact.

- Conclusion

Proposal 1: RAN4 to define the following requirements for the mobility scenario of "E-UTRA TN to NR NTN" in CONNECTED mode:

- Requirements for inter-RAT NR Measurements for FR1-NTN

- Requirements for E-UTRA TN to NR FR1-NTN handover

Proposal 2: For inter-RAT NR Measurements for FR1-NTN, RAN4 to take the requirements for NR inter-frequency measurements for FR1-NTN defined in TS 38.133 section 9.3C as baseline, and study the GNSS status impact.

Proposal 3: For E-UTRA TN to NR FR1-NTN handover, RAN4 to take the requirements for NR SAN FR1 – NR SAN FR1 Handover defined in TS 38.133 section 6.1C.1.2 as baseline, and study the GNSS status impact.

- Reference

- RP-252890, Revise WID on E-UTRA TN to NR NTN handover enhancements, CATT