# 3GPP TSG-RAN WG4 Meeting #118	R4-2600458

# Gothenburg, Sweden, 9-13 February, 2026

Agenda Item:	8.2.2

Source: 	Xiaomi

Title: 	View on 6GR modulation orders

Document for:	Discussion

- Introduction

According to the endorsed 6G work arrangement from RAN4 chair (R4-2511652), RAN4 divides 6G study into 11 separate topic areas in initial stage. System parameter is one of major agenda with following candidate scope:

In this contribution, our preliminary view on modulation orders are provided.

- Discussion

In RAN4#116bis, RAN4 has following agreements:

#### Modulation Agreement - RAN4 evaluation work focus on the feasibility study and RF requirements impact - To establish the foundational evaluation framework in RAN4 firstly when no solid progress and inputs from RAN1 - Identify the main affected requirements for modulation evaluations - The existing 5G NR requirements will serve as the baseline, which are subject to future updates based on RAN4’s 6G UE RF discussions. - Align on the evaluation assumptions. - Study on how to align on the transmitter chain model, including PA model, for consistent evaluations on the modulation. - RF evaluation could be done firstly for 5G supported modulations with new assumptions for 6G study, such as assumed new spectrum, CBW, new PA models, etc. Co-ordination with 6G UE RF study is needed. - Both link-level and system-level simulations should be performed as usual for high-order modulations study done by RAN4 in prior releases, pending on the progress of RAN1. - Model and evaluate the performance and the implementation complexity of higher-order modulations, e.g. 1024QAM on the UL and/or new constellations - For high order QAM with uniform constellation, e.g. 1024QAM on the UL, RAN4 can work concurrently with RAN1 studies. - For new non-uniform constellation, the evaluation in RAN4 should depend on RAN1 progress and request.

The major work expected from RAN4 on modulation order/waveform is feasibility study with associated RF requirements impact e.g., MPR/Power boosting, Tx EVM, and applicable scenarios (DL/UL, device type, and frequency ranges). Without sufficient input from RAN1 on basic waveform and modulation order structure, it’s hard for RAN4 to have efficient discussion. Also, the evaluation work has dependency on RF requirements assumption and PA model used.

The progress from RAN1 on modulation orders till Nov meeting’25 is summarized in Annex. The key agreements are summarized in below observations.

Observation 1: RAN1 discussion on modulation orders can be categorized into three areas:

- CAT 1: NR modulation orders with uniform constellation are supported in 6G as baseline

  - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel

  - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel

  - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel

  - Note: Above NR modulation orders served as basis and reference for performance evaluation in RAN1

- CAT 2: Higher modulation orders with uniform constellation are considered pending on performance evaluation

  - For DL, 4K QAM is on evaluation

  - For UL, 1K QAM is on evaluation

  - For assumed TX/RX EVM, before receive any concrete numbers from RAN4, companies can provide their assumptions. One example can be 6dB tighter than the EVM of 1K QAM for DL and 256QAM for UL.

- CAT 3: Constellation shaping schemes including geometric shaping (GS) and Probabilistic shaping (PS)

  - RAN1 carry on evaluation for complexity, power consumption, and performance benefits compared to traditional one

Observation 2: RAN1 start on evaluation work for CAT 2(uniform high modulation orders) and CAT3 (constellation shaping schemes including geometric shaping (GS) and probabilistic shaping (PS).

- Both link level evaluation and system level evaluation are considered in RAN1

  - start with link-level evaluation first with throughput/BLER as target evaluation metric

- RAN1 already agreed basis evaluation assumption including MIMO configuration in BS and UE side, and receiver processing assumption

Observation 3: Tight cooperation with RAN4 is required for RAN1 evaluation especially for PA modelling, Tx EVM assumption and MPR.

Similar as waveform evaluation, RAN4 shall work closely with RAN1 for evaluation work to avoid duplicated work across different WGs with clear working split between RAN4 and RAN1 with considering technical expertise/responsibility from each WGs.

Proposal 1: RAN4 work closely with RAN1 to avoid duplicated evaluation work across different WGs with clarification of responsibility of each WG

- RAN4 focused on evaluation on Transmitter side for achievable Tx EVM and MPR reduction

- RAN1 evaluation focused on the throughput gain/SNR gain in receiver side by taking RAN4 input into account including achievable Tx EVM, Rx EVM and MPR

- The simulation assumption between RAN1 and RAN4 shall be well aligned e.g., BS/UE antenna modelling, fading channel profile, MIMO configuration, and operating frequency, CHBW.

The most relevant RF requirements for modulation order is Tx EVM and MPR; especially for Tx EVM both performance benefits and implementation feasibility needs to be considered. When starting the evaluation work, RAN4 also needs to carefully consider the budget of Tx EVM and Rx EVM on high modulations.

Proposal 2: RAN4 evaluation focus on MPR and Tx EVM aspects by considering performance benefits and implementation feasibility.

On potential RAN4 expected work, we believe the expected work in RAN4 for different cases are different. For CAT 1 existing NR modulation orders with uniform constellation, existing RAN4 requirements can be considered as baseline; and any potential improvement or trade-off between MPR/Tx EVM requirements can be further discussed in UE RF thread as no feasibility study required. For CAT 2 (Higher modulation orders with uniform constellation including DL 4K QAM and UL 1K QAM), RAN1 evaluation work require RAN4 input on Tx EVM/Rx EVM and MPR considering realistic achievable performance from implementation perspective to evaluate realistic performance gain with high modulation orders compared w/o those modulation orders.

For CAT3 Constellation shaping schemes (PS and GS), RAN1 work is on initial stage on detailed shaping algorithm/schemes which is brand new for RAN4. It’s better to wait for sufficient progress from RAN1 to start RAN4 work. PS-QAM uses a distribution matcher with nonuniform distribution to map the encoded bits to the constellation points. Performance gain may be obtained to map inner constellation points with higher probability to achieve better power efficiency. For probability shaping, RAN4 needs to evaluate the impact to Tx EVM and MPR and it’s pending on detailed distribution matcher method which is still on discussion in RAN1 with several options. GS-QAM is to generate a non-rectangular constellation, also known as nun-uniform constellation (NUC) to pack more densely and closer to a hypersphere. Clearly RAN4 expected to be involved to evaluate the impact on Transmitter side and receiver side for GS-QAM.

Figure 1 Constellation patterns of 2D-NUC 256QAM for code rates 10/15, 11/15, 12/15, 13/15

Proposal 3: Postpone modulation order evaluation until sufficient progress reached in RAN1/RAN4 e.g., start from Q2’ 26

Proposal 4: RAN4 expected work for different cases can be considered separately

- For CAT 1 (NR modulation orders with uniform constellation): RAN4 can further evaluate Tx EVM or MPR requirements improvement/relaxation by taking existing NR requirements as baseline (NO feasibility study required, this work is not urgent and shall be handled in UE RF thread instead of system parameter thread)

- For CAT 2 (Higher modulation orders with uniform constellation including DL 4K QAM and UL 1K QAM): RAN4 focus on the evaluation of achievable Tx EVM/Rx EVM and MPR assumption in UL side

  - Taking as first priority as RAN1 required input from RAN4 on achievable Tx EVM/Rx EVM and MPR assumption

- For CAT 3 Constellation shaping schemes (PS and GS): RAN4 need to wait sufficient progress from RAN1 to evaluate potential impact on Tx EVM, MPR and other potential requirements impact

  - Wait for sufficient progress reached in RAN1 on detailed shaping algorithm

Proposal 5: Taking NR uniform modulation orders as basis for 6GR

- For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for data channel

- For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for CP-OFDM for data channel

- For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for DFT-s-OFDM for data channel.

On supporting high modulation orders, the status of existing NR situation can be summarized in below table:

| Modulation orders | Tx EVM in FR1 | Applicable scenario |
| --- | --- | --- |
| QPSK | 17.5 % | DL/UL (mandatory) |
| 16QAM | 12.5 % | DL/UL (mandatory) |
| 64QAM | 8 % | DL/UL (mandatory) |
| 256QAM | 3.5 % | Mandatory for DL Optional for UL |
| 1024QAM | 2.5 % for below 4.2GHz 2.8 % for above 4.2GHz | Only applicable for DL and target for stationary wireless scenarios e.g., FWA scenario Optional |

To supporting DL 4K QAM and UL 1K QAM with benefits compared to existing NR modulation orders, below 1% Rx EVM in UE side is required for DL 4K QAM, and around 2% ~4% Tx EVM is required for UL 1K QAM.  With initial survey for available RF component performance in NR commercial smartphones, those Tx/Rx EVM are extremely challenge with trade-off on power reduction/power efficiency.  We also share the similar observation from Skyworks in [4], the situation in WIFI and gNB side can’t be directly applied to UE due to applicable power consumption, power control, device form factor limitation and applicable RF components difference.

To evaluate the achievable Tx/Rx assumption, RAN4 can further analyse the Tx EVM/Rx EVM budget with different contributors and required MPR reduction as similar in LTE/NR [3].

Observation 4: The conclusion from gNB and WIFI for 1K QAM shall not applied to UE due to the difference from following aspects power consumption/power control, device form factor limitation, compliant RF requirements and applicable RF components.

Observation 5: Based on initial survey, it’s extremely challenge to achieve desirable Tx EVM and MPR performance to get benefits for UL 1KQAM and DL 4K QAM for handheld UE (e.g. RAN1 assume 6dB tightened than existing UL 256QAM, and DL 1K QAM).

Proposal 6: RAN4 needs to evaluate the limitation of achievable Tx EVM and MPR performance for DL 4KQAM and UL 1KQAM with associated device type assumption for high modulation (e.g., FWA only) and applicable operating frequency.

Proposal 7: RAN4 also needs to evaluate the possibility of supporting 1K QAM for handheld UE in 6GR.

Proposal 8: For RAN4 evaluation for high modulation orders on achievable Tx EVM and Rx EVM, following table can be considered on UL 1024QAM, DL 4K QAM:

| EVM contributor |  | MPR assumption 1 |  | MPR assumption 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | EVM | SNR (dB) | EVM | SNR (dB) |
| Tx EVM | PA |  |  |  |  |
|  | Transmitter |  |  |  |  |
|  | Phase noise |  |  |  |  |
|  | IQ imbalance |  |  |  |  |
|  | Total Tx EVM |  |  |  |  |
| Rx EVM budget |  |  |  |  |  |
| Total Tx+ Rx EVM |  |  |  |  |  |

- Conclusion

In this contribution, our preliminary view on waveform are provided.

Observation 1: RAN1 discussion on modulation orders can be categorized into three areas:

- CAT 1: NR modulation orders with uniform constellation are supported in 6G as baseline

  - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel

  - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel

  - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel

  - Note: Above NR modulation orders served as basis and reference for performance evaluation in RAN1

- CAT 2: Higher modulation orders with uniform constellation are considered pending on performance evaluation

  - For DL, 4K QAM is on evaluation

  - For UL, 1K QAM is on evaluation

  - For assumed TX/RX EVM, before receive any concrete numbers from RAN4, companies can provide their assumptions. One example can be 6dB tighter than the EVM of 1K QAM for DL and 256QAM for UL.

- CAT 3: Constellation shaping schemes including geometric shaping (GS) and Probabilistic shaping (PS)

Observation 2: RAN1 start on evaluation work for CAT 2(uniform high modulation orders) and CAT3 (constellation shaping schemes including geometric shaping (GS) and probabilistic shaping (PS).

- Both link level evaluation and system level evaluation are considered in RAN1

  - start with link-level evaluation first with throughput/BLER as target evaluation metric

- RAN1 already agreed basis evaluation assumption including MIMO configuration in BS and UE side, and receiver processing assumption

Observation 3: Tight cooperation with RAN4 is required for RAN1 evaluation especially for PA modelling, Tx EVM assumption and MPR.

Proposal 1: RAN4 work closely with RAN1 to avoid duplicated evaluation work across different WGs with clarification of responsibility of each WG

- RAN4 focused on evaluation on Transmitter side for achievable Tx EVM and MPR reduction

- RAN1 evaluation focused on the throughput gain/SNR gain in receiver side by taking RAN4 input into account including achievable Tx EVM, Rx EVM and MPR

  - On high modulation orders,

- The simulation assumption between RAN1 and RAN4 shall be well aligned e.g., BS/UE antenna modelling, fading channel profile, MIMO configuration, and operating frequency, CHBW.

Proposal 2: RAN4 evaluation focus on MPR and Tx EVM aspects by considering performance benefits and implementation feasibility.

Proposal 3: Postpone modulation order evaluation until sufficient progress reached in RAN1/RAN4 e.g., start from Q2’ 26

Proposal 4: RAN4 expected work for different cases can be considered separately

- For CAT 1 (NR modulation orders with uniform constellation): RAN4 can further evaluate Tx EVM or MPR requirements improvement/relaxation by taking existing NR requirements as baseline (NO feasibility study required, this work not urgent and shall be handled in UE RF thread instead of system parameter thread)

- For CAT 2 (Higher modulation orders with uniform constellation including DL 4K QAM and UL 1K QAM): RAN4 focus on the evaluation of achievable Tx EVM/Rx EVM and MPR assumption in UL side

  - Taking as first priority as RAN1 required input from RAN4 on achievable Tx EVM/Rx EVM and MPR assumption

- For CAT 3 Constellation shaping schemes (PS and GS): RAN4 need to wait sufficient progress from RAN1 to evaluate potential impact on Tx EVM, MPR and other potential requirements impact

  - Wait for sufficient progress reached in RAN1 on detailed shaping algorithm

Proposal 5: Taking NR uniform modulation orders as basis for 6GR

- For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for data channel

- For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for CP-OFDM for data channel

- For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for DFT-s-OFDM for data channel.

Observation 4: The conclusion from gNB and WIFI for 1K QAM shall not applied to UE due to the difference from following aspects power consumption/power control, device form factor limitation, compliant RF requirements and applicable RF components.

Observation 5: Based on initial survey, it’s extremely challenge to achieve desirable Tx EVM and MPR performance to get benefits for UL 1KQAM and DL 4K QAM for handheld UE (e.g. RAN1 assume 6dB tightened than existing UL 256QAM, and DL 1K QAM).

Proposal 6: RAN4 needs to evaluate the limitation of achievable Tx EVM and MPR performance for DL 4KQAM and UL 1KQAM with associated device type assumption for high modulation (e.g. FWA only) and applicable operating frequency.

Proposal 7: RAN4 also needs to evaluate the possibility of supporting 1K QAM for handheld UE in 6GR.

Proposal 8: For RAN4 evaluation for high modulation orders on achievable Tx EVM and Rx EVM, following table can be considered:

| EVM contributor |  | MPR assumption 1 |  | MPR assumption 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | EVM | SNR (dB) | EVM | SNR (dB) |
| Tx EVM | PA |  |  |  |  |
|  | Transmitter |  |  |  |  |
|  | Phase noise |  |  |  |  |
|  | IQ imbalance |  |  |  |  |
|  | Total Tx EVM |  |  |  |  |
| Rx EVM budget |  |  |  |  |  |
| Total Tx+ Rx EVM |  |  |  |  |  |

Reference

[1] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] R4-2514650,“WF on 6GR system parameter”, Feature Lead (Huawei)

[3] R4-166954, “WF on MPR\AMPR simulation assumptions for UL 256QAM”, Huawei, Hisilicon, Qualcomm, CMCC, Nokia, Skyworks, Ericsson

[4] R4-2521792, “On UL/DL high order modulation for 6G”, Skyworks

Annex: RAN1 related agreements

RAN1#122

Agreement

- For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel

  - FFS: Enhancements and other modulation schemes

- For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel

  - FFS: Enhancements and other modulation schemes

- For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel

  - FFS: Enhancements and other modulation schemes

RAN1#122bis

Agreement

For 6GR constellation shaping evaluation for CP-OFDM, and improved MCS table, the proposed scheme will be compared with non-shaping with NR MCS table. The evaluation and comparison should consider at least the following:

- BLER performance under AWGN channel (at least for performance calibration)

  - 1st transmission (baseline) and with HARQ re-transmission

- BLER performance under fading channel with fixed MCS

  - 1st transmission (baseline   m) and with HARQ re-transmission

- Throughput performance with link adaptation (adaptive MCS and rank) under fading channel

  - Needs to provide assumptions on rate adaptation (e.g., target BLER for 1st transmission, maximum # of retransmissions)

- Transmitter and receiver complexity (e.g., shaping/deshaping, demapper), latency, parallelism implementation, and storage requirements,

- Other KPI not excluded, such as PAPR, EVM, MPR/A-MPR

- Expected spec impact

- FFS detailed assumption of constellation shaping and improved MCS table

- System level evaluation can be done after link level evaluation.

Agreement

For 6GR constellation shaping study, proponent is encouraged to provide details for the PS/GS schemes considered for evaluation and comparison, including at least the following

- Probabilistic shaping for CP-OFDM and DFT-s-OFDM

  - Use the list of spectrum efficiencies in NR MCS table as starting point, and provide constellation (including normalization), coding rate and target probabilistic distribution for each SE

    - If multiple coding rate and target probabilistic distribution pairs are provided for each SE, how to switch between them

  - Relationship between shaping and FEC, coded bits to modulation symbol mapping, and other modules (such as scrambling, interleaving), in transmit and receive chains. How to handle HARQ retransmission

  - PS algorithm details (for example, source coding based, channel coding based, etc) and parameters (such as block length, rate loss)

- Geometric shaping for CP-OFDM and DFT-s-OFDM

  - Use the list of spectrum efficiencies in NR MCS table as starting point, and provide target constellation shape (including normalization) (1D-NUC, 2D-NUC, QAM-CS, etc) for each SE

    - If multiple constellation shapes are provided for each SE, how to switch between them

  - GS mapping details, such as bit to constellation point mapping and shape

  - Relationship with other blocks (such as scrambling, interleaving). How to handle HARQ retransmission

Agreement

For link level simulation for modulation evaluation, companies are encouraged to evaluate with the following assumptions and should report the exact scheme evaluated.

- channel configuration, including Channel profiles,Tx/RX antenna settings

- For MIMO scenario: SU-MIMO or MU-MIMO, follow agenda item 11.2 for MIMO when available.

- Precoder assumption

  - Close loop MIMO (reciprocal beamforming (e.g., SVD, SLR/RZF, etc.), codebook based)

    - Realistic CSI/SRS/AP-SRS periodicity and delay, and SRS chanEst assumptions,

    - or genie beamforming

  - Open loop MIMO

- Receiver assumption (for MIMO): LMMSE (baseline) for UL, rML or LMMSE for DL

- LLR demapper: Max-log (baseline) or Log-MAP

- Channel estimation: Realistic (baseline) or ideal

- Other assumptions: Channel coding NR-LDPC (baseline), PxSCH bandwidth, SCS, FD interleaver used or not, 5GNR BICM interleaver usage

- Note: For MIMO, SIMO, MISO and SISO are included when possible

Agreement

For 6GR constellation shaping evaluation for DFT-s-OFDM, and improved MCS table, the proposed scheme will be compared with non-shaping with NR MCS table. In addition to what has been agreed for CP-OFDM in earlier agreement, the evaluation and comparison should further consider at least the following:

- PAPR/CM of the resulting waveform

- EVM, MPR/A-MPR

Agreement

For the study of uniform 4096QAM for DL and uniform 1024QAM for UL, need to study performance (assuming realistic channel estimation, time/freq synchronization assumption, phase noise assumption, etc), complexity/power consumption, requirements, benefit/necessity under applicable scenarios, associated restrictions, and challenges (such as EVM requirement, PAPR increase, MPR or A-MPR increase under realistic PA model).

- FFS: How to involve RAN4 early

- FFS: Shaping of higher order modulation

- System level evaluation can be done after link level evaluation.

RAN1#123

Agreement

For PS/GS fixed MCS performance reporting for 10% BLER (other target x% BLER can also be reported), adopt the following format for simulation as a starting point for result reporting.

| NR reference |  | Scheme A (e.g, PS, 1D-NUC, 2D-NUC etc) |  |  |
| --- | --- | --- | --- | --- |
| SE | (Mod order, coding rate) | SE point specific parameters | Baseline (uniform QAM) SNR to reach target BLER | Gain/loss in dB wrt NR baseline at target BLER x% |
| SE x | (modOrder, coding rate) | … |  |  |
| SE y | (modOrder, coding rate) | … |  |  |
| SE z | (modOrder, coding rate) | … |  |  |
| SE point independent assumptions |  | Common assumptions for the scheme simulated, including channel type (AWGN, SISO, SIMO, MIMO) and antenna configuration, number of spatial layers, number of RB allocated, TB size, shaping algorithm used (including block length), freq domain interleaver applied or not, receiver assumption, precoding assumption, realistic channel estimation, etc |  |  |

Note: For NR MCS reference, since NR has multiple MCS tables, it is not enough to provide the MCS index. Instead, need to provide the (modulation order, coding rate) pair for the simulated SE

Note: For SE point specific parameters:

- For GS, this can be a pointer to the constellation used for this SE point

- For PS, this can be a constellation size, coding rate and shaping parameter used for this SE point

Note: Other metrics (at least complexity) will be merged into the same table with other columns, if details of the metrics are agreeable.

Note: For AMC study, if possible, we can use the same table format

Agreement

On how to evaluate complexity, storage requirement, delay and parallelism/serialism for PS and GS compared with uniform QAM.

- For PS

  - The demapper complexity is compared with uniform QAM demapper complexity

    - Can report the demapper complexity with PS and the demapper complexity of NR MCS with the same spectrum efficiency, and the ratio of the complexities

      - Also report the number of spatial layers, dm-algorithm used and the receiver type (e.g., LMMSE or rML), and fixed point assumed or floating point assumed.

  - The Distribution Matcher (DM)/Distribution De-Matcher (DDM) complexity and/or storage requirement as a function of the DM algorithm used (ESS, CCDM, etc), precision of fixed point implementation, block length, and the number of bit levels shaped per symbol

    - For complexity, can report the complexity normalized by the number of information bits

      - As a reference, can also report the computation complexity of LDPC decoding with 10 iterations.

    - For storage requirement, can report the overall storage needed for DM/DDM for supporting all MCS in the MCS table and all shaping related parameters

    - DM and DDM complexity and storage will be counted separately

  - The DM/DDM processing delay, parallelism/serialism, as a function of DM design and block length, and their impact to throughput

- For GS,

  - The demapper complexity is compared with uniform QAM demapper complexity

    - Can report the ratio of GS demapper complexity over the uniform QAM demapper complexity

      - Also report if 1D-NUC or 2D-NUC is used, # of spatial layers, and the receiver type (e.g., LMMSE or rML)

    - Also need to report the assumption on complexity counting, e.g, fixed point assumed or floating point assumed

  - The storage requirement for storing all the constellations corresponding to the MCS indices in the MCS table, as a function of precision of constellation point storage

  - Processing delay and parallelism/serialism, if applicable, and their impact to throughput

Note: the complexity is represented by the numbers of comparison, addition/subtraction, and multiplication/division operations, normalized by the number of information bits.

Note: For complexity as a function of SE point, will add a column in the already agreed performance reporting table.

Note: For complexity/storage not as a function of SE point, will add a row in the already agreed performance reporting table.

Note: Spec impact will be separately evaluated, include BICM, scrambling, etc

Note:

For 4K uniform QAM DL and 1K uniform QAM UL link level performance study, the following format can be used for performance reporting.

| (modOrder, coding rate*1024, SE) | Assumed TX/RX EVM | Channel 1 | Channel 2 | Channel 3 |
| --- | --- | --- | --- | --- |
|  |  | SNR to achieve target BLER | SNR to achieve target BLER | SNR to achieve target BLER |
| (10, 900.5, 8.7939) for DL evaluation (8, 916.5, 7.1602) for UL evaluation | Legacy EVM for 1K QAM for DL and 256QAM for UL respectively |  |  |  |
| (10, 948, 9.2578) for DL evaluation (8, 948, 7.4063) for UL evaluation | Legacy EVM for 1K QAM for DL and 256 QAM for UL respectively |  |  |  |
| SE1 |  |  |  |  |
| SE2 |  |  |  |  |
| SE3 |  |  |  |  |
| Other parameters |  |  |  |  |

- For assumed TX/RX EVM, before we receive any concrete numbers from RAN4, companies can provide their assumptions. One example can be 6dB tighter than the EVM of 1K QAM for DL and 256QAM for UL.

- Other parameters include: Channel estimation assumption (genie or realistic), channel configurations (AWGN, SISO, SIMO, MIMO and TX/RX antenna configurations, channel types, number of spatial layers,), assumed residual freq offset range, number of allocated RBs, etc

- Two highest MCS points in DL 1K QAM and UL 256QAM in NR added in the table for comparison.

- This is preliminary result and not intended for TR

Note: For high order uniform QAM for DL 4K QAM and UL 1K QAM, to provide the UPT with the high order QAM (DL 4K QAM and UL 1K QAM) over the UPT without the high order QAM under the assumed deployment scenario.

Agreement

To evaluate the proposal to allow a single spectrum efficiency point to be supported by multiple MCS entries (with different modulation order and coding rate combinations with uniform QAM or with different shaping parameters, coding rate, and constellation size combinations for PS and different coding rate and constellation combinations for GS).

- When providing results, to provide the following information

  - Details on the design of MCS table with overlapping MCS entries and expected size of MCS table, including performance comparison of designs with the same expected size of MCS table

  - Performance benefit under different channel and rank assumptions

    - As baseline, provide performance with legacy MCS table up to 256 QAM

      - Can additionally provide performance with legacy MCS table up to 1K QAM

    - For PS/GS, provide performance allowing each SE point to be mapped to one or more MCS entries

      - For PS/GS, provide performance allowing each SE point to be mapped to only one MCS entry (from the set of one or more MCS entries)

    - For uniform QAM, provide performance allowing each SE point to be mapped to one or more MCS entries

  - MCS selection mechanism across multiple MCS corresponding to the same spectrum efficiency.

    - If UE feedback is needed for gNB to select between multiple MCS entries corresponding to the same SE, provide details on what is to be fed back

- FFS: How different MPR for different modulation order is captured in the simulation for uplink

- FFS: How different EVM for different modulation order is captured in the simulation

- For the purpose of this study, the same set of SE points as in legacy uniform QAM table will be used as starting point.

- When reporting performance, also report other assumptions, including channel type (AWGN, SISO, SIMO, MIMO) and antenna configuration, number of spatial layers, number of RB allocated, TB size, shaping algorithm used (including block length), freq domain interleaver applied or not, receiver assumption, precoding assumption, realistic channel estimation, etc

- To propose how to align shaping parameters or how to align coding rate for facilitating comparison.

Agreement

For DFT-s-OFDM, further study how/whether Net Gain over uniform QAM can be achieved by PS/GS.