3GPP TSG-RAN WG4 Meeting #118							   R4-2600501

Gothenburg, Sweden, 9th – 13th February, 2026

Agenda Item: 	8.2.4

Source: 	MediaTek Inc.

Title: 	Smallest maximum CBW for low-tier devices

Document for: 	Discussion

# 1	Introduction

Reducing the maximum bandwidth supported by the UE is being considered as a method for constraining the complexity of the lowest tier device for 6GR. RAN4 has also discussed the potential benefits to enabling SAW-less designs. This document further discusses those aspects.

# 2	3GPP status

Both RAN1 & RAN plenary have made progress on these discussions, and the key related agreements are captured below.

RAN#110 Agreement:

- 6GR supports the operation (but not required to be optimized for performance) in a minimum spectrum allocation of 3MHz with a 15kHz SCS

Note: the following agreement made in RAN1#123 still holds, with the clarification that the bandwidth in Opt 1 below is assumed to be at least 5MHz with a 15kHz SCS.

RAN1#123 Agreement

If the minimum spectrum allocation is 3MHz with 15kHz SCS for 6GR,

- Opt1: Design of the common signals/channels (at least for SSB) for initial access by assuming bandwidth larger than 3MHz, which is applicable to any spectrum allocations with adjustment, if applicable

- Opt2: A single design of the common signals/channels (at least for SSB) for initial access by assuming minimum spectrum allocation as target bandwidth 3MHz, which is applicable to any spectrum allocations

- Regarding the smallest maximum UE bandwidth as discussed in the following RAN1 agreement, Opt 1 is excluded. Aim to conclude by RAN plenary no later than RAN#112 (June 2026).

  - RAN1 and RAN4 is tasked to continue providing more analysis accordingly.

  - Companies are encouraged to provide more analysis at RAN plenary particularly regarding the use cases, requirements, economy of scale, etc.

RAN1#122 Agreement

- Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

  - Opt1: 3MHz

  - Opt2: 5MHz

  - Opt3: 10MHz

  - Opt4: 20MHz

  - FFS: the UL bandwidth may be different to the DL bandwidth

  - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

  - FFS: whether RF and BB UE BW are same or different

# 3	Enabling true reduced device complexity

The fundamental component for reducing true device complexity for the end customer is to ensure sufficient economies of scale for a given device. However, considering just reduction of Bill of Materials (BOM) can constrain the potential applications and use cases of such a device, and therefore negatively impact that scale.

Another important aspect to enable scale and usability is to ensure that the same device can be a global device that is able to connect to networks in different geographical regions. This drives a need for the device to support different frequency bands and somewhat drives a need for ease of integration with networks. The UE would also need to be able to operate in different system BWs, i.e. 3MHz upwards.

Observation 1: Economies of scale and broad applicability both in terms of use cases and deployments/spectrum are key for the success of 6G low-tier devices, as a trade-off with BOM reduction.

Proposal 1: Economies of scale and ease of network integration to be considered as part of any analysis of the benefits of smallest maximum BW for low-tier devices.

Proposal 2: The UE should support all specified CBWs up to the smallest maximum CBW defined for a given SCS.

# 4	Bill Of Materials (BOM) and size reduction

BOM and size should of course be considered as a trade-off with the above aspects, but some balanced consideration is required. In this section we evaluate aspects that may impact device BOM and size from the UE RF perspective.

## 4.1	UE maximum supported BW

It was shown in eRedCap studies (3GPP TR 38.865) that the savings yielded by reducing the maximum supported bandwidth from 20MHz to 5MHz was small vs a RedCap HD-FDD device. It should also be noted that there are other components for BOM reduction, such as processing time relaxation and peak rate reduction, that may reduce the magnitude of that saving.

Observation 2: BW reduction from 20MHz to 5MHz has small benefit on overall device complexity.

## 4.2	Duplex filter reduction/removal

Reducing the need for the duplex filter in FDD bands for a global SKU device supporting multiple FDD bands can have significant benefits in terms of complexity reduction. HD-FDD operation rather than FD-FDD can achieve that goal for the lowest tier device, and we recommend that it is supported by 6GR.

Proposal 3: HD-FDD should be supported by the lowest tier 6GR device for FDD bands.

## 4.3	SAW filter reduction/removal

We agree with the proponents of [4] that if a device supports HD-FDD, then if it still needs to support multiple SAW filters for multiple bands to fulfil certain spurious emission coexistence requirements at UE Tx side, then this would be unwelcome from a device BOM perspective. However, enabling a partial or fully SAW-less UE design does not necessarily require the smallest maximum bandwidth to be limited to cope with the worst case.

SAW filters for UE Tx

For UE Tx, where we have observed that SAW filters may be more prevalent, we consider the following possible alternatives:

- A relaxation of the spurious emission requirement for UE cross-band coexistence.

  - Further analysis of the coexistence impact would be required here, e.g. identifying whether anything has changed since the analysis in Rel-8 which led to the existing limits.

- Restrict the operated UL bandwidth by the UE or UL RB allocation in certain strict coexistence scenarios when the UE is located close to the band edge.

  - RAN4 would need to consider the cross-band coexistence cases that would require particular attention.

- A-MPR for device in certain frequency locations that operates a wider CBW/RB allocation.

Observation 3: There are different approaches to allowing SAW-less UE Tx operation that do not require constraining the maximum supported CBW of the UE.

Proposal 4: Considering 20MHz CBW as the smallest maximum supported CBW for UL and DL for 15kHz and 30kHz SCS, study further the impact of partial or fully SAW-less designs in UE Tx side, with analysis on the following mitigation approaches:

- Relaxation of the spurious emission requirement for UE cross-band coexistence.

- Restricting the operated UL bandwidth by the UE or UL RB allocation in certain strict coexistence scenarios when the UE is located close to the band edge.

- A-MPR for device in certain frequency locations that operates a wider CBW/RB allocation.

SAW filters for UE Rx

We are open to study further the need/impact of SAW filters on the UE Rx requirements. Our initial analysis suggests that (unlike for uplink) bandwidth or RB allocation scaling may not help in case a SAW filter would be needed to fulfil specific stringent Rx blocking requirements for specific bands. It would be useful to further evaluate which bands may have stringent blocking requirements, and whether those requirements are needed for the lowest tier device.

Proposal 5: RAN4 to consider further Rx blocking and associated impacts to enabling fully or partially SAW-less Rx designs for the lowest tier 6GR device.

# 5	Conclusions

Observation 1: Economies of scale and broad applicability both in terms of use cases and deployments/spectrum are key for the success of 6G low-tier devices, as a trade-off with BOM reduction.

Proposal 1: Economies of scale and ease of network integration to be considered as part of any analysis of the benefits of smallest maximum BW for low-tier devices.

Proposal 2: The UE should support all specified CBWs up to the smallest maximum CBW defined for a given SCS.

Observation 2: BW reduction from 20MHz to 5MHz has small benefit on overall device complexity.

Proposal 3: HD-FDD should be supported by the lowest tier 6GR device for FDD bands.

Observation 3: There are different approaches to allowing SAW-less UE Tx operation that do not require constraining the maximum supported CBW of the UE.

Proposal 4: Considering 20MHz CBW as the smallest maximum supported CBW for UL and DL for 15kHz and 30kHz SCS, study further the impact of partial or fully SAW-less designs in UE Tx side, with analysis on the following mitigation approaches:

- Relaxation of the spurious emission requirement for UE cross-band coexistence.

- Restricting the operated UL bandwidth by the UE or UL RB allocation in certain strict coexistence scenarios when the UE is located close to the band edge.

- A-MPR for device in certain frequency locations that operates a wider CBW/RB allocation.

Proposal 5: RAN4 to consider further Rx blocking and associated impacts to enabling fully or partially SAW-less Rx designs for the lowest tier 6GR device.

# 6	References

- [1] RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator)

- [2] R4-2522450, WF for 6GR system parameter (RAN4#117), Feature lead (Huawei, HiSilicon)

- [3] Meeting report, 3GPP TSG RAN#110 meeting

- [4] R4-2522049, Views on general and UE RF of 6GR, Sony (RAN4#117)