# 3GPP TSG-RAN WG4 Meeting #118	                              R4-2600536                                                     Gothenburg, Sweden, Feb. 09-13, 2026

Agenda Item:	6.11.4

Source: 	Apple

Title: 			 Demodulation and/or CSI reporting requirements for CSI prediction

Document for:	Discussion

Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved. This contribution provides discussion on demodulation requirements for CSI prediction.

Discussion

Issue 1-1: Codebook to be used in the tests Agreement: - In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined - use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions Issue 1-2: Requirement baseline Agreement: - Requirement to be introduced for Dopler domain basis N4=1 - N4>1 can be further discussed in the future if needed Issue 1-4: Test metric Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/ML using the precoders configured according to the UE reports, - trnd is the throughput measured at SNRue_AI/ML with random precoding. - trnd is obtained with Rel-15 type I single panel codebook - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4

According to the revised WID RP-243244, CSI prediction (one-sided model) is one of the objectives for WI, and RAN4 need to specify necessary RAN4 requirements. In previous meeting, it was agreed to use relative throughput as requirement metric for CSI prediction. Reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook.

For CSI prediction, it was agreed that AI/ML based performance requirement should not be worse than the legacy, and FFS which legacy requirement/feature are relevant.

Agreements in RAN4#112bis Issue 1-4: AI/ML Performance Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

During RAN4#115, the following agreement was reached:

Issue 1-1: Requirement considerations

Agreement:

The performance of AI based CSI prediction should not be worse than R16 etypeII.

- The exact test procedure and configurations are FFS

- The impact of different timeline between AI and R16 etype II should be also carefully analysed

Issue 1-3: Doppler handling

Agreement:

  - De-prioritize 100Hz Doppler in R19

    - Consider 100Hz Doppler in the future release if needed

During RAN4#117 the following agreement was reached:

Agreements in main session:

Issue 1-2: Averaging of γ

Agreement: Averaging of γ will not be specified.

Issue 1-3: Parameters for the test cases

Agreement:

    - FDD 2Rx: 16 CSI-RS ports, TDLA30-20, MCS19

    - TDD 4Rx: 16 CSI-RS ports, TDLA30-20, MCS17

    - FFS: FDD 2Rx: 16 CSI-RS ports, TDLC300-20, MCS X (for generalization purpose)

Issue 1-4: Simulation results

Agreement:

Regarding CSI prediction, it is agreed to define the requirements based on the worst one, which is no worse than non-AI sample and hold, based one, in the current simulations without removing the outliers

In the context of AI/ML-based CSI prediction, defining meaningful and fair performance requirements is fundamentally more complex than in legacy PMI testing. Unlike traditional PMI reporting, where the algorithm behavior is deterministic and independent of deployment scenarios, the performance of AI/ML models is scenario-dependent, tightly linked to the training dataset and inference conditions. Factors such as antenna configuration, UE speed (Doppler), observation window length, and prediction window length directly affect prediction accuracy and generalization. This poses a challenge for RAN4, as the current legacy testing framework assumes a single PMI requirement is sufficient to represent conformance across all conditions.

While non-static channel conditions (e.g., changing SNR or fading profiles over time) offer a more realistic and comprehensive test of model generalization, they introduce significant complexity and ambiguity, especially for a group like RAN4 that has not previously handled time-varying test conditions. Defining the appropriate dynamics, reference models, and reproducible baselines would require extensive new work and agreement, which is not feasible within the limited timeline of Rel-19.

Therefore, to ensure AI/ML model robustness and performance while remaining within practical constraints, it is essential that RAN4 focuses on well-defined static test conditions for Rel-19. However, unlike legacy PMI tests, multiple static test cases must be defined, each reflecting a different scenario dimension. This allows the test framework to reflect real-world variability in AI/ML model behavior without requiring non-static (time-varying) setups, thereby ensuring a balance between test coverage and feasibility.

On the use of MCS X for generalization
Using an additional MCS point (MCS X) for “generalization purpose” does not address the core technical challenges associated with AI/ML model generalization and evolution. Generalization in AI/ML-based CSI prediction is driven primarily by variations in channel statistics and system context, rather than by the specific MCS value. Key factors that fundamentally affect model generalization include channel model and delay spread (e.g., TDL-A vs TDL-C/D), Doppler spectrum and mobility profile, SNR and interference distributions, antenna and CSI-RS port configurations, spatial correlation and angular spread, timeline alignment between prediction and application, etc. If an AI/ML model demonstrates robust performance across these dimensions, changing MCS alone does not meaningfully stress generalization or model evolution. Therefore, adding MCS X is unlikely to provide insight into real-world generalization capability.

Do not introduce additional Test case for generalization purpose. Different test with different channel model/ MCS doesn’t meaningfully test generalization.

Performance monitoring

The WI on AI/ML-based CSI prediction use case is specifying the reporting of performance. First, the delay and accuracy of performance monitoring metric is discussed in RAN4#115 [1].

Agreement: Monitoring reporting delay and accuracy: Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric: - BM-Case1 and possibly BM-Case 2 o Performance monitoring type 1 option 2 - If it proves to be not possible to define, then reverse the agreement - Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirement. If it would be in Rel-19, a monitoring delay requirement is needed. CSI prediction: - CSI prediction, the details are subject to RAN1 decision - RAN4 will further study the feasibility to specify the corresponding delay and/or accuracy requirements for UE sided monitoring

In RAN4 agreement, monitoring reporting is under RAN1’s scope. RAN1#120-bis and RAN1#121 have made the agreements as follows.

RAN1#120-bis Agreement For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric.

In RAN1 agreement, SGCS is agreed the metric of CSI prediction using UE-side model for performance metric type 3. In Type 3, the UE computes the performance metrics, reports them to the network, which then determines whether to initiate functionality fallback to legacy CSI reporting.

RAN1#121 Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, - one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCS is reported - wideband frequency granularity - only for one prediction instance configured by NW - per layer, and the total number of layers is the same as the reported RI in the associated inference report - Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’ - Each SGCS value is quantized with 4-bit - 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale. - One codepoint is used to indicate the value range of (0 0.3] - Only one monitoring resource set is configured in the CSI-ReportConfig - The one configured time instance (i.e. f-th doppler domain unit in one inference report) for SGCS calculation is configured in the CSI-ReportConfig for monitoring, for N4>1 Agreement For CSI prediction using UE-side model, support single UCI part with the following report format (i.e., CSI field mapping order) among a set of SGCSs. SGCS1 #1. SGCS1 #2. …. SGCS1 #v. SGCS2 #1. SGCS2 #2. …. SGCS2 #v. - SGCS1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI, - SGCS2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCSi #k is the SGCS of k-th layer of i-th SGCS where k ={1, …., v}, and i ={1, 2}. - v is the value of the reported RI in the associated inference report.

During RAN4#116 the following agreements were reached:

Agreement:

RAN4 will introduce requirements for Type 3 performance monitoring for CSI prediction

FSS which requirements are to be introduced.

Issue 1-2: Requirement baseline for monitoring

Agreement:

Introduce the following requirements for CSI performance monitoring:

- Reporting delay

  - FFS how the delay is defined

- Reporting accuracy

  - FFS on whether accuracy requirement can be defined/checked

- Mapping table (38.1.3.3 section 10)– To be checked whether RAN1 captures or RAN4.

- If it is not feasible to define reporting accuracy requirement or test it, RAN4 will send an LS to RAN1 to inform RAN1 about this

During RAN4#116bis the following agreements were reached:

Issue 1-1: Requirements for performance monitoring

Agreement:

No performance monitoring delay requirements are specified in RAN4 for CSI prediction.

Issue 1-7: Doppler and MCS

Agreement:

Introduce tests for 20Hz Doppler with MCS [17or19] with 16Tx ports and 2Rx in FDD

Introduce test for TDD with the details FFS

Issue 1-8: Generalization

Agreement:

Test setup for the generalization will be further discussed based on the following options

- Option 1: Introduce multiple tests with different MCS, which will lead to different SNR points.

- Option 2: Introduce multiple tests with different channel models. The details of the channel models are FFS.

- Option 3: the combination of option 1 and 2.

It is FFS on how to quantify the generalization performance.

Issue 1-9: Activation

Agreement:

No activation requirement in RAN4 spec for CSI prediction will be introduced.

- Note: it is a general understanding that UE has to be ready to start measurements for inference after sending RRC reconfiguration complete. No details will be specified in RAN4.

Based on the RAN1#121 agreement, the requirements for CSI prediction using UE-side models and reporting contents of UE-assisted performance monitoring are defined.

For CSI prediction using UE-side models, the agreement specifies that two types of SGCS values are calculated and reported. The first type, SGCS1, is based on the predicted CSI and the ground truth CSI. The second type, SGCS2, is based on the ground truth CSI and the non-predicted CSI corresponding to the latest CSI-RS transmission occasion not later than the CSI reference resource of the inference reporting instance.

However, for the network to rely on these predictions, it must continuously monitor the prediction quality and decide when to trust the reported CSI or revert to legacy CSI reporting. Today there is no standardized, verifiable way to ensure that a UE-reported similarity metric truly reflects the accuracy of its prediction. Without clearly defined accuracy requirements and corresponding test procedures, a malfunctioning or mis-implemented predictor can silently degrade link adaptation, scheduling, or mobility performance while still presenting apparently healthy CSI reports to the network.

Issues with Only SGCS1
SGCS1 measures the similarity between the UE’s predicted CSI and the UE’s own ground-truth CSI, i.e., CSI derived from standard CSI-RS measurements on the configured reference resource. Although SGCS1 provides a direct view of prediction quality in principle, the network cannot directly observe how the UE determines its ground truth or performs the similarity computation. This creates room for errors or even manipulation, for example, the UE could inadvertently use outdated channel samples, or, in a worst case, use an internally simulated (“genie”) channel, producing a high SGCS1 even when the actual prediction is inaccurate. If RAN4 were to rely only on SGCS1, it would leave operators exposed to silent failures, mis-implementation, or intentional misreporting that could compromise overall system performance.

Dual-Metric Safeguard with SGCS1 and SGCS2
The standardized introduction of SGCS2, defined as the similarity between the same ground truth and the most recent non-predicted CSI measurement, provides a critical second layer of protection. Under realistic, dynamic channel conditions, a functioning AI/ML predictor should consistently yield SGCS1 values significantly higher than SGCS2, reflecting the genuine performance gain of prediction over legacy CSI. Conformance and network monitoring can therefore look not only at absolute SGCS1 thresholds but also at the relationship between SGCS1 and SGCS2. Any large inconsistency or unexpected correlation (e.g., SGCS1 not appreciably exceeding SGCS2) flags potential mis-reporting or degraded predictor performance.

In the CSI prediction use case, the UE runs an on-device AI/ML model to estimate future or missing CSI and reports both the predicted CSI and a performance monitoring metric, typically the SGCS.  The purpose of SGCS is to allow the network (or test equipment) to monitor whether the model’s predictions remain accurate over time and to trigger fallback or model deactivation if performance degrades.

However, the core issue is that the test equipment has no access to the ground truth SGCS.

Therefore, there is no direct way to verify whether a reported SGCS truly reflects the model’s real accuracy.
A malfunctioning, outdated, or misconfigured model might still report high SGCS values if the UE computes it incorrectly or with outdated measurements.
Defining normative RAN4 requirements directly on SGCS is of limited practical value for both conformance testing and field operation, because SGCS depends on UE-internal ground truth that is not observable by the network or test equipment. As a result, high SGCS values cannot be independently verified and may not reliably reflect true prediction accuracy, creating risk of silent failures or mis-implementation. In addition, CSI prediction performance is already evaluated end-to-end through relative throughput–based requirements, which directly capture the system impact of prediction errors on link adaptation and scheduling. From a RAN4 perspective, throughput-based metrics provide a verifiable, system-relevant validation of prediction performance, whereas SGCS is difficult to interpret and validate in both lab and live networks. Therefore, RAN4 should avoid setting standalone normative requirements on SGCS and instead rely on end-to-end performance metrics, using SGCS only as an optional, non-normative monitoring or diagnostic indicator.

Do not define accuracy requirements on SGCS, since it relies on UE-internal ground truth that cannot be independently verified. CSI prediction is already validated via PMI reporting requirement.

Conclusion

This contribution provides discussion on CSI prediction. The proposals captured are:

- Do not introduce additional Test case for generalization purpose. Different test with different channel model/ MCS doesn’t meaningfully test generalization

- Do not define accuracy requirements on SGCS, since it relies on UE-internal ground truth that cannot be independently verified. CSI prediction is already validated via PMI reporting requirement

Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface