3GPP TSG-RAN4 Meeting  #118	R4-2600575

Gothenburg, Sweden, Feb. 09-13, 2026

Agenda item:	8.2.2

Source:	Apple

Title:	On 6G system parameters – Modulation

Document for:	Discussion

# 1	Introduction

A WF on 6G system parameters was agreed [1] at the last meeting, including the following aspects on modulation:

# Topic #2: Modulation - Recommendation from FL based on inputs for RAN4#117 - Evaluation Cases: - Case 1 (Baseline): Existing NR modulations (BPSK to 256QAM) with new 6G PA model(s). - Case 2 (Higher-order modulation): - UL 1024QAM: Primarily focusing on FWA UE implementation feasibility. - Already agreed to start parallel study in last RAN4 meeting - DL 4096QAM: Focusing on both BS and UE implementation feasibility. - Whether and when to consider it as an optional feature for study, pending on further RAN4 discussion and decision - Case 3 (Constellation shaping): Defer detailed evaluation until RAN1 conclusions are stable. - Evaluation assumptions: - PA Model: Depends on the discussion progress on 6G PA model. This is a foundational assumption for all modulation studies. - EVM budget: Define a clear EVM budget for higher order modulation (UL 1024QAM, DL 4096QAM), considering all impairment sources (PA non-linearity, I/Q imbalance, phase noise, CFR, etc.). - Scenarios and frequencies: Focus evaluations on agreed scenarios (TBD, like Urban Macro and indoor hotspot), across agreed frequencies (TBD, like ~700 MHz, 2 GHz, and 7 GHz). - Bandwidth: Consider wider channel bandwidths (TBD, e.g., 200 MHz) - WF - FFS in next meeting

In this contribution, we continue the discussion on modulation based on the WF above.

# 2	Discussion

## 2.1 6G baseline

We propose to consider the existing 5G modulation schemes as a starting point for 6G, because they have been evaluated from the standpoint of both performance gain and implementation feasibility:

- UL: up to 256 QAM

- DL: up to 1024QAM

Furthermore, as 6G is going to introduce new spectrum such as ~7GHz and wider CBW such as 200MHz, we agree with the views in [2] that the modulation schemes can be studied with 200MHz CBW in ~7GHz to identify any issues or challenges, including the potential use of a new PA model.

Proposal 1: It is proposed to consider the following modulation schemes as a starting point for 6G:

- UL: up to 256 QAM

- DL: up to 1024QAM

Proposal 2: The modulation schemes are studied with 200MHz CBW in ~7GHz to identify any issues or challenges, including the potential use of a new PA model.

## 2.2 Higher-order modulation (UL 1K QAM and DL 4K QAM)

UL 1024QAM

As stated in the WF, the targeted UE type is FWA UE. Compared to handheld UEs, FWA UE has less form factor or power consumption constraints. Therefore, the starting point of EVM requirements can be the BS EVM requirement for DL 1024QAM, which is 2.5% for 4.2GHz or below and 2.8% for above 4.2GHz.

Then we discuss the major implementation sources that contribute to EVM. On the BS side, major sources are LO phase noise, clipping noise caused by CFR (Crest factor reduction) algorithm, and PA non-linearity. With some power backoff and proper clipping, it was shown that it is feasible to achieve the required EVM.

On the UE side, we note the following EVM breakdown for UL 256QAM and 64QAM as shown in [3].

Table 1 EVM budget for 256QAM and 64QAM

| Non-linearity source | 256QAM |  | 64QAM |  |
| --- | --- | --- | --- | --- |
|  | % | C/N | % | C/N |
| PA | 1.50% | 36.5 | 4.00% | 28.0 |
| Transmitter nonlinearity | 1.34% | 37.4 | 3.37% | 29.5 |
| LO Phase noise | 1.78% | 35.0 | 2.24% | 33.0 |
| IQ Image | 2.24% | 33.0 | 5.62% | 25.0 |
| Transmitter total | 3.50% | 29.1 | 8.00% | 21.9 |

In our view, the EVM contributing sources for UL 1024QAM remain the same. The question is to what extent they can be further reduced. For example, the LO phase noise contribution may have to be lowered to around 1%, which would require significant PLL improvement and optimization. Similarly, it is also challenging to further improve PA linearity. The IQ image requirement also needs to be tightened to around 40dB, which is much tighter than the current RAN4 requirement of 25dB.

Proposal 3: It is proposed to limit the support of UL 1024QAM to FWA UE and further discuss how the existing 2.5% or 2.8% EVM requirements can be met.

DL 4096QAM

We ran some link-level simulation to see the crossover SNR of 4096QAM compared to 1024QAM, as shown below. It can be seen that depending on the channels, AWGN/TDL-A/TDL-D, the crossover SNR is 30dB or higher.

We understand that Wi-Fi 7 supports 4096QAM with an EVM requirement of -38dB or 1.26%. However, since Wi-Fi and cellular services have different application scenario, the first issue to address is in the real cellular deployments, how likely it is to observe an SNR 30dB or higher, where 4096QAM starts to show gain over 1024QAM for single layer transmission. When Tx EVM requirement for 256QAM were discussed, the small cell deployments scenarios were targeted where BS transmits at a low power and have room to apply power backoff if needed to meet the EVM requirement. For DL 1024QAM, even lower BS transmit power is expected because of the tightened BS EVM requirement. For DL 4096QAM, would this mean the cell coverage is further reduced compared to 1024QAM due to the further tightened BS EVM requirement, say 1.26%? It is better to consider the deployment scenarios for 4096QAM. Furthermore, for small cell deployments, the receive SINR at the UE may also suffer from the inter-cell interference, which makes evaluation of real field SINR critical in understanding if there is performance gain.

Proposal 4: It is proposed to understand the potential gain of supporting 4096QAM in real field.

On implementation feasibility, besides the difficulty on the BS side, we would like to emphasize this places a huge burden on the UE side to reduce RX EVM to meet the 1.26% EVM requirement, assuming the BS TX EVM and UE RX EVM are similar. Note that the UE RX EVM consists of the following sources:

- Rx LO phase noise

- Rx non-linearity

- ADC quantization noise

- I/Q imbalance

- Carrier leakage (DC offset)

- Carrier frequency offset

It is worth highlighting that to reduce LO phase noise which may be the bottleneck, a great deal of effort is needed based on our investigation. And even with improved phase noise, it is still challenging to improve other sources.

Observation 1: It is very challenging to support UE RX EVM around 1.26%.

## 2.3 EVM split between BS and UE

The current specification assumes a symmetric EVM split between BS and UE, which does not reflect the significant hardware capability differences between UE and BS. The BS hardware benefits from superior PA linearity, lower phase noise, and relaxed size and power constraints, while UEs face fundamental limitations due to battery, thermal, and cost constraints. An asymmetric EVM split where BS Tx EVM requirements are tightened and UE Tx EVM requirements are relaxed would better utilise the different hardware capabilities of BS and UE while preserving overall system performance. This approach could improve UL coverage through reduced MPR and facilitate industry-wide adoption of higher-order modulations such as 1024QAM and 4096QAM. The detailed technical analysis and specific proposals are provided in our contribution [4].

# 3	Conclusions

In this contribution, we discuss 6G modulation and make the following proposals.

Proposal 1: It is proposed to consider the following modulation schemes as a starting point for 6G:

- UL: up to 256 QAM

- DL: up to 1024QAM

Proposal 2: The modulation schemes are studied with 200MHz CBW in ~7GHz to identify any issues or challenges, including the potential use of a new PA model.

Proposal 3: It is proposed to limit the support of UL 1024QAM to FWA UE and further discuss how the existing 2.5% or 2.8% EVM requirements can be met.

Proposal 4: It is proposed to understand the potential gain of supporting 4096QAM in real field.

Observation 1: It is very challenging to support UE RX EVM around 1.26%.

# 4	References

R4-2522450, "WF for 6GR system parameter," Feature lead (Huawei, HiSilicon)

R4-2520964, "Views on 6G modulation," MediaTek

R4-2520426, "Discussion on 6GR modulation," CMCC

R4-2600148, "Views on 6G joint UE-BS RF and spectrum aggregation," Apple