3GPP TSG-RAN WG4 Meeting #118	R4-2600654

Gothenburg, Sweden, Feb. 09-13, 2026

Agenda Item:	7.9.3

Source:	CAICT

Title:	Discussion on 	Inter-vendor training collaboration and interoperability for two-sided AI/ML models

Document for:	Discussion

1	Introduction

In last meeting, moderator provided a comprehensive list of topics for discussion. In this contribution, we provide our views on selective topics.

# Summary of issues to consider for RAN4#118 The following is not intended as an agreement, but is a summary of topics for which contributions are invited for RAN4#118 in order to make progress: - Scalability options: - Whether to consider 8, 64, 128 ports - TX port configurations - Sub-band configurations - Eventually, CSI payload sizes for the final model - Method to achieve scalability - Evaluation of the model structure variations with swapped normalization and MLA/MLP and/or additional layers - Proposals for model hyperparameters - Further discussion of quantization for the final model - Confirmation or proposals for encoder complexity limit - Proposals for decoder complexity limit - Whether to consider robustness when selecting the final hyperparameters set - Whether to consider rank 3 and 4 in the dataset - How to generate the dataset - Dataset alignment (whether to do, which metrics) - Performance requirement design - Proposals for how to capture the model and the dataset in 3GPP (format, how to store etc.) - LCM requirements in case there is sufficient RAN1/2 progress

2	Discussion

2.1 Scalability options: Sub-band configurations

In last meeting, there was no agreement achieved on sub- band size configuration. In our consideration, RAN4 could take the simulation assumption in TR38.843 as starting point. The simulation bandwidth is 10/20MHz for 15kHz SCS and 40MHz for 30kHz SCS, the detailed sub-band configuration is presented in following table.

| Simulation BW | SCS | Sub-band size | Number of sub-bands |
| --- | --- | --- | --- |
| 10MHz | 15kHz | 4, 8 | 13, 7 (last one is partial size) |
| 20MHz | 15kHz | 8, 16 | 14 (last one is partial size), 7 (last one is partial size) |
| 40MHz | 30kHz | 8, 16 | 14 (last one is partial size), 7 (last one is partial size) |

Thus, we suggest to consider 6 or 13 sub-bands with identical sub-band size as initial assumption for further scalability evaluation.

Proposal 1: Suggest to consider 6 or 13 sub-bands with identical sub-band size as initial assumption for further scalability evaluation.

2.2 How to generate the dataset

In previous RAN4 meeting, it was agreed that unless significant issues are identified, RAN4 would focus only on discussing scalability issues and options that had already been agreed upon in RAN1. In RAN1, the scalability of CSI compression (Case 0) was primarily evaluated across the following dimensions: the number of Tx ports, payload sizes, and bandwidth settings—including variations in both the number of sub-bands and sub-band sizes. Based on our understanding, since the dataset includes only the target CSI (i.e., the input to the encoder), the payload size does not affect the fundamental assumptions of the dataset.

The dataset construction can be structured along three key dimensions: the number of Tx ports, the configuration of {number of sub-bands, sub-band size}, and the channel models. RAN4 could specify a minimal set of candidate values for each dimension to cover typical test configurations. The global dataset would then be built by considering all combinations of these dimensions. For instance, if M candidate values are defined for Tx ports, N for {number of sub-bands, sub-band size}, and P for channel models, the total number of dataset combinations would be M × N × P.

On size of each dataset, the method used to construct mixed dataset in the study item could be taken for reference, at least 10K samples per contributing company to ensure balanced representation and the minimum total size should exceed a threshold, e.g., 100K samples.

Proposal 2: As starting point, the dataset construction can be structured along three key dimensions: the number of Tx ports, the configuration of {number of sub-bands, sub-band size}, and the channel models. RAN4 could specify a minimal set of candidate values for each dimension to cover typical test configurations. The global dataset would then be built by considering all combinations of these dimensions.

2.3 Dataset alignment

Based on the experience of R19 evaluation, the discrepancy of dataset distribution would highly affect the performance of the AI/ML models. Thus, RAN4 could use the R19 CNN-based CSI encoder/decoder or Transformer-based CSI encoder/decoder defined in RAN1 as a benchmark to test performance on each company’s dataset. If the performance is consistent across datasets, they can be deemed as comparable.

Proposal 3: Considering to use the R19 CNN-based CSI encoder/decoder or Transformer-based CSI encoder/decoder defined in RAN1 as a benchmark to test performance on each company’s dataset.

2.4 how to capture the model and the dataset in 3GPP

Regarding the model, the discussion can be divided into model structure and model weight. The model structure can be documented in the specification using text descriptions and diagrams/ flowcharts, along with essential hyperparameters. The model weights can be stored on the 3GPP FTP server and referenced in the specification via URLs. The weight files should be stored in mainstream formats that are widely supported and can be easily loaded by common machine learning tools.

Regarding the dataset, we suggest that the specification should include a descriptive paragraph outlining its key features. The actual dataset files can be hosted in a designated directory on the 3GPP FTP server and referenced via URLs in the specification.

Proposal 4: The model structure of test decoder and reference encoder can be documented in the specification using text descriptions and diagrams/ flowcharts, along with essential hyperparameters.

Proposal 5: The model weights of test decoder and reference encoder can be stored on the 3GPP FTP server and referenced in the specification via URLs. The weight files should be stored in mainstream formats that are widely supported and can be easily loaded by common machine learning tools.

Proposal 6: The specification should include a descriptive paragraph outlining key features of the dataset. The actual dataset files can be hosted in a designated directory on the 3GPP FTP server and referenced via URLs in the specification.

Conclusion

In this contribution, inter-vendor training collaboration and interoperability for two-sided AI/ML models are discussed with following proposals.

Proposal 1: Suggest to consider 6 or 13 sub-bands with identical sub-band size as initial assumption for further scalability evaluation.

Proposal 2: As starting point, the dataset construction can be structured along three key dimensions: the number of Tx ports, the configuration of {number of sub-bands, sub-band size}, and the channel models. RAN4 could specify a minimal set of candidate values for each dimension to cover typical test configurations. The global dataset would then be built by considering all combinations of these dimensions.

Proposal 3: Considering to use the R19 CNN-based CSI encoder/decoder or Transformer-based CSI encoder/decoder defined in RAN1 as a benchmark to test performance on each company’s dataset.

Proposal 4: The model structure of test decoder and reference encoder can be documented in the specification using text descriptions and diagrams/ flowcharts, along with essential hyperparameters.

Proposal 5: The model weights of test decoder and reference encoder can be stored on the 3GPP FTP server and referenced in the specification via URLs. The weight files should be stored in mainstream formats that are widely supported and can be easily loaded by common machine learning tools.

Proposal 6: The specification should include a descriptive paragraph outlining key features of the dataset. The actual dataset files can be hosted in a designated directory on the 3GPP FTP server and referenced via URLs in the specification.