3GPP TSG- RAN4 Meeting # 118	R4-2600753

# Gothenburg, Sweden, Feb. 09-13, 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev |  | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TC for SSB based L1-RSRP measurement on SDL SCell for UE supporting LB CA via switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LBCA_Sw-Perf |  |  |  |  |  | Date: |  |  | 2026-1-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Specify TC for SSB based L1-RSRP measurement on SDL SCell for UE supporting LB CA via switching. |  |  |  |  |  |  |  |  |
| s |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify TC for SSB based L1-RSRP measurement on SDL SCell for UE supporting LB CA via switching. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No to verify SSB based L1-RSRP measurement on SDL SCell for UE supporting LB CA via switching. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New A.6.6.4.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

A.6.6.4.X	SSB based L1-RSRP measurement on SDL SCell for UE supporting LB CA via switching

A.6.6.4.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE configured with LowBandCA-Switching-r19 makes correct reporting of L1-RSRP measurement on SDL SCell. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.1. There are two cells in the test, the FR1 FDD PCell (Cell 1) and FR1 SDL SCell. The supported test configurations for NR PCell are shown in table Table A.6.6.4.X.1-1. Supported test configurations for NR SDL SCell are shown in table Table A.6.6.4.X.1-1A.

Table A.6.6.4.X.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test for NR PCell

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.4.X.1-1A: Applicable NR configurations for FR1 SSB based L1-RSRP test for NR SLD SCell

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, SDL duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

A.6.6.4.X.2	Test parameters

The test parameters are given in table A.6.6.4.X.2-1 and table A.6.6.4.X.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically on the SDL SCell. The switching pattern is partially overlapped with SSBs resource occasions on SDL SCell. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs on the SDL SCell.

Table A.6.6.4.X.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| RF Channel Number | 1 |  | 1,2 |
| Active PCell | 1 |  | Cell 1 |
| Configured activated SDL SCell | 1 |  | Cell 2 |
| CP length | 1 |  | Normal |
| DRX | 1 |  | OFF |
| Cell 2 timing offset to cell1 | 1 | s | 0 |
| Time alignment error between cell2 and cell1 | 1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. |
| SSB ARFCN | 1 |  | freq1 |
| Duplex mode | 1 |  | FDD on PCell; SDL on SCell |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD on PCell; SR.1.1 SDL on SCell |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD on PCell; CR.1.1 SDL on SCell |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD on PCell; CCR.1.1 SDL on SCell |
| SSB configuration | 1 |  | SSB.3 FR1 |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1 |  | SMTC.1 |
| switching pattern | 1 |  | {1111111111000000000000000000001111111111} |
| gapDurationPCelltoSCell | 1 |  | 3 |
| gapDurationSCelltoPCell | 1 |  | 3 |
| TRS Configuration | 1 |  | TRS.1.1 FDD on PCell TRS.1.1 SDL on SCell |
| reportConfigType | 1 |  | periodic |
| reportQuantity | 1 |  | ssb-Index-RSRP |
| Number of reported RS | 1 |  | 2 |
| L1-RSRP reporting period | 1 | slot | 40 |
| T1 | 1 | s | 5 |
| T2 | 1 | s | 1 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.X.2-2: SSB specific test parameters on SDL SCell

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.6.6.4.X.3	Test Requirements

The UE shall send L1-RSRP report on SDL SCell every 40 slots. No later than 640 ms plus 40 slots from the beginning of time period T2, UE shall send L1-RSRP report on SDL SCell including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of change>