3GPP TSG-RAN WG4 Meeting # 118	R4-2600756

Gothenburg, Sweden, Feb. 09-13, 2026

Title: 	Discussion on impacts for RRM measurement prediction

Source: 	Huawei, HiSilicon

Agenda item:	7.12.3

Document for:	Discussion

Introduction

The revised WID on AI/ML for mobility in NR agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI The objectives of this work item focused on AI/ML-assisted mobility in RRC_CONNECTED mode, wherein RRM measurement prediction and measurement event prediction could be applied. Only functionality-based LCM procedures are supported. In general, the RRM measurement configuration and reporting frameworks in RRC layer are taken as baseline. The objectives are as the following: - Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: - Inference input report for at least RRM sub-case 2 of intra-frequency temporal domain case A for L3 cell prediction - Inference input report for RRM sub-case 5 of all scenarios for L3 beam level prediction - Network sided data collection for all scenarios and all RRM sub-cases of both L3 cell level and L3 beam level prediction, at least - UE is configured to log RRM measurement results. - UE supports periodical logging and L3 event triggered logging. - UE can indicate logged data availability. Note 4: All scenarios in this objective refer to intra-frequency domain case A and case B, inter-frequency prediction and spatial domain prediction. Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible. - For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

This contribution focuses on RRM measurement prediction in AI/ML mobility.

Discussion

Simulations for measurement prediction

In RAN4#117 meeting, the below procedure is agreed to perform simulations for measurement prediction.

- Companies to generate ideal L1-RSRP dataset from the SLS assumption. - Note: The dataset is filtered based on SINR value (i.e., -6dB < SINR). - Use the LLS assumptions to generate L1-RSRP difference as the baseband errors based on one shot measurement. - Use the RF error model assumption to generate RF errors. - Add the measurement error into the ideal SLS dataset in 1st step to derive the dataset with measurement errors. - Both baseband errors and RF errors are included. - Measurement error assumptions follow 2nd step and 3rd step. - Note: this is based on option A for RF error addition. If using option B, the RF error will be added after L3 filtering in 5th step. - Add L1 filtering and L3 filtering to the dataset with added measurement error generated from the 4th step. - Use subset samples of the dataset generated from the 5th step for training models. - Use the other subset samples of the dataset with added measurement error (non-overlapped samples with the subset dataset in 6th step) to derive the dataset for inference. - Use the dataset with measurement error in 7th step for inference to evaluate the performance metrics (predicted L3-RSRP). Note: L1-RSRP in 1st and 2nd step means the L1 RSRP measurement without L1 filtering.

In the simulation, some points need further discussion and clarification.

The way to add BB error

BB error stems from UE channel estimation. For each measurement sample (Layer1 RSRP), the channel is different, so BB error shall be added independently per sample, i.e., point A.

- System-level simulation yields SINR of each target cell at each measurement occasion (per 40ms)

- Link-level simulation yields the CDF curves of BB errors corresponding to each SINR level.

- At the corresponding SINR level, generate a random number following a truncated Gaussian distribution with (50%, max[5%,95%]). Each generated random number (i.e., BB error) is then added to corresponding L1 measurement point.

Proposal 1: The way to add BB error in simulations:

- System-level simulation yields SINR of each target cell at each measurement occasion (per 40ms)

- Link-level simulation yields the CDF curves of BB errors corresponding to each SINR level.

- At the corresponding SINR level, generate a random number following a truncated Gaussian distribution with (50%, max[5%,95%]). Each generated random number (i.e., BB error) is then added to corresponding L1 measurement point.

The way to add RF error

RF error stems from non-ideal RF component. in last meeting, companies proposed that RF error is added independently before L1 filtering. If the RF error is time-varying, it will be averaged after L1 and L3 filtering, and the impact of RF error will be underestimated.

We observed that when the UE moves from the cell center toward the cell edge, the RSSI changes. Once RSSI exceeds a certain threshold, the UE will adjust the AGC gain which will affect AGC LNA. Consequently RF error will change accordingly. However, it should be noted that the RF error does not vary at every measurement occasion (i.e., it is not time‑varying). Therefore, we recommend assuming that the RF error remains constant over a period of time (e.g., 10 seconds). Starting from the 11th second, the RF error changes and follows a truncated Gaussian distribution with (0, 2.5).

As the RF error is not timing-varying, and from UE implementation perspective, RF error occurs at the sampling points. We can agree to add the RF error to the L1 samples.

Proposal 2: RF error remains constant over a period of time (e.g., 10 seconds). Starting from the 11th second (just an example), the RF error changes and follows a truncated Gaussian distribution with (0, 2.5).

Benchmark for comparison

Agreement: - Only align the LLS assumption, the baseband error is added directly on ideal dataset by companies. Agreement for the simulation only: - The baseband and RF errors should be considered in training dataset (for both input and output), model input during inference. - For the benchmark for comparison, both with and without error can be considered.

As it is agreed in RAN4#117 meeting that the baseband error is added directly on ideal dataset. It states that during data training, both BB error and RF error are added both in input and output. In other words, the standard answer takes the BB error and RF error into account. Then the purpose of AI training is solely to achieve more accurate prediction measurement results, not to reduce BB and RF errors.

In testing dataset, RAN4 is to discuss what kinds of the benchmark for comparison shall be:

- Option 1: No any error for the benchmark

The final accuracy result not only reflects the prediction error, but also includes the impact of the BB error and RF error.

- Option 2: Only RF error is considered for the benchmark

The reason to introduce option 2 is that if the ground truth is the reported RSRP under high SINR, the BB error can be negligible. If the real SINR is not high enough, the BB error is still present

In option 2, the final accuracy result reflects the prediction error and the impact of the BB error and RF error.

- Option 3: Both BB error and RF error are considered for the benchmark

In option 3, sole prediction accuracy by AI model is evaluated.

Proposal 3: In testing dataset, RAN4 is to discuss what kind(s) of the benchmark for comparison shall be:

Option 1: No any error for the benchmark

Option 2: only RF error is considered for the benchmark

Option 3: Both BB error and RF error are considered for the benchmark

Preliminary Measurement Results

In annex, the simulation assumptions are provided. Based on simulation assumptions, the simulation results are given in Table 1 and Table 2, Figure 1 and Figure 2.

Table 1: Delta RSRP (dB) for AI prediction in FR1 (UE speed=30km/h)

| Benchmark for comparison | 5th%-iles | 50th%-iles | 95th%-iles |
| --- | --- | --- | --- |
| Option 1 in proposal 3 | -2.97 | 0.07 | 2.88 |
| Option 2 in proposal 3 | -3.98 | 0.10 | 3.72 |
| Option 3 in proposal 3 | -1.88 | 0.17 | 1.81 |

Table 2: Delta RSRP (dB) for AI prediction in FR1 (UE speed=90km/h)

| Benchmark for comparison | 5th%-iles | 50th%-iles | 95th%-iles |
| --- | --- | --- | --- |
| Option 1 in proposal 3 | -3.52 | 0.01 | 3.50 |
| Option 2 in proposal 3 | -4.34 | -0.07 | 4.39 |
| Option 3 in proposal 3 | -2.57 | -0.12 | 2.41 |

Figure 1. CDF Analysis of Delta RSRP for AI prediction in FR1

Methodology of predicted RSRP accuracy

As we know, in legacy non-AI, RAN4 simulate the measurement samples and corresponding measurement accuracy, and determine requirements based on trade-off between measurement delay and accuracy. Therefore we think the methodology of determining the principle of prediction accuracy in AI/ML mobility shall be discussed in core requirements phase.

To facilitate discussion, the high-level measurement model in TS38.300 without AI is duplicated as below.

Figure 9.2.4-1: Measurement Model [in TS38.300]

In TR38.744, 3 sub-use cases for cell-level measurement prediction are considered.

3 sub-use cases are considered for cell-level RRM measurement prediction: Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

In sub-use case 1, the prediction error is generated from actual L1 beam level predicting L1 beam level measurement (prediction is executed at point A1).

In sub-use case 2, the prediction error is generated from actual L3 beam level predicting L3 cell level measurement (prediction is executed at point C).

In sub-use case 3, the prediction error is generated from actual L1 beam level predicting L3 beam level measurement (measurement result at point A1 predicting measurement result at Point C).

In the three cases, the inference model is used at different position in the measurement process. The AI prediction model algorithm would be various due to different input and output. Although in RAN4#116bis it is agreed to evaluate sub-case 2, L3-L3 measurement prediction as a starting point for both FR1 and FR2 [2], other sub-cases would be discussed latter. So one fundamental issue is inevitably still going to be touched upon: whether RAN4 to define one unified cell level prediction accuracy requirement or to define prediction accuracy for each sub-use case/scenario.

Agreement: - As starting point, evaluate sub-case 2, L3-L3 measurement prediction as baseline for both FR1 and FR2. - Interested companies can also provide results for other sub-cases. - It is not precluded to add additional sub-case for FR2 in future meetings. - Further discuss how to define requirements based on the available simulation results.

- If use-case specific prediction accuracy is defined, UE needs to know which prediction accuracy is to be satisfied. The scenario and sub-use case shall be known to UE. This scheme is related with RAN2 discussion.

- If it is expected to define one unified prediction accuracy, there are two different approaches. As per the RAN2 simulation results in SI phase, majority of the companies observe that RRM sub-use case 2 demonstrates higher prediction accuracy than RRM sub-use case 1 and RRM sub-use case 3 at least with short PW length for FR2 intra-frequency temporal domain case A.

  - Approach 1: RAN4 can select one sub-use case which has proper prediction accuracy. For example, use prediction accuracy of sub-use case 2 as the final accuracy result. UE shall satisfy this accuracy regardless which kind of sub-use case is actually applied in UE side.

  - Approach 2: RAN4 can determine prediction accuracy to cover all sub-use cases.

Proposal 4: For prediction accuracy, RAN4 need to discuss:

- Whether to define prediction accuracy for each sub-use case/scenario, or

- Whether to define one unified cell level prediction accuracy

- to accommodate all sub-use cases/scenario, or

- by selecting one typical and proper sub-use case/scenario

Regarding to whether the prediction accuracy shall at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurement, it is also need to be further discussed in RAN4 WI.

Proposal 5: RAN4 to discuss whether the prediction accuracy has the same or different accuracy requirements that are defined for the non-AI/ML RRM measurement.

Potential core requirements

As per the progress of beam management in R19 AI/ML air interface WI, the following related core requirements are to be specified for AI/ML beam management:

Prediction report delay - For aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay - The reporting delay includes the inference delay - For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+time for the first available reporting occasion Measurement period for inference - observation period for prediction: (observation period is the amount of time during which UE samples the reference signals to make 1 prediction report) - For case 1 reuse M,N,P from legacy measurement requirements - For case 2, use T*M, N P (M,N,P same as legacy measurement requirement) - FFS on T value, T can also be 1. T can also be based on capability - This observation period is to be used for the prediction delay requirement Activation delay - UE has to be ready to start measurements for inference after sending RRC reconfiguration complete - Inference delay (might include also model loading depending on UE implementation) will be included in the inference reporting delay - This applies to all reporting schemes

We think the core requirements of R20 AI/ML mobility can reuse the conclusion of beam management in R19 AI air interface as a starting point.

For prediction report delay, for aperiodic report, the overall prediction reporting delay can include measurement delay, reporting delay where the reporting delay includes the inference delay. For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+time for the first available reporting occasion.

Proposal 6: In AL/ML mobility, prediction reporting delay can be defined as core requirement and the definition can refer to the conclusion of beam management in Rel-19 NR_AIML_air.

In RAN4#116bis meeting, RAN4 agreed to discuss the RRM requirements for the below scenarios [3]:

Agreement: RAN4 to discuss the potential RRM requirements for cell-level based prediction for the following scenarios: Evaluation scenario; Methodology. FR1 to FR1 intra-frequency temporal domain case A; intra-cell. FR1 to FR1 intra-frequency temporal domain case B; Intra-cell. FR1 to FR1 inter-frequency (frequency domain) (co-located case); Inter-cell. FR2 to FR2 intra-frequency temporal domain case A; Intra-cell. FR2 to FR2 intra-frequency temporal domain case B; Intra-cell. FFS whether to discuss the requirements for the following scenario: Evaluation scenario; Methodology. FR2 to FR2 inter-frequency (frequency domain) (co-located case); Inter-cell.

In the following, we discuss the potential core requirements regarding with prediction measurement delay.

Agreements in RAN2#126 Chairman notes 7 To agree on methodology of Intra_F_C_T_Case A as following: Intra-frequency intra-cell temporal domain case A prediction is done by predicting measurement result(s) in prediction window based on measurement results in observation window of the same cell for both FR1_to_FR1 and FR2_to_FR2. FFS aligning the prediction window 8 To agree on methodology of Intra_F_C_T_Case B: Intra-frequency intra-cell temporal domain prediction is done by predicting sub set measurement (case B) instances in temporal domain of the same cell for both FR1_to_FR1 and FR2_to_FR2. Several measurement reduction rates should be aligned among companies. The detail values for measurement reduction rate are FFS.

We know temporal domain case A and case B have different working mechanism. For both case A and case B, OW (observation window) and PW (prediction window) are introduced [2].

Figure 5.2.1-1 Example of intra-frequency temporal domain case A [TR38.744]

In intra-cell temporal domain prediction case A, observation window is a duration UE performs actual measurement. Within an observation window, the measurement period shall follow the legacy measurement period. The prediction window is a duration UE predicts measurement. Observation window and prediction window will slide. It means that the instances in previous prediction window would become part of the observation window when UE perform measurement in predicted instances. It shall be noted that the observation window length depends on model performance, channel environment and observation length, etc.

The below observations for FR2 intra-frequency temporal domain case A are made in RAN2 [TR 38.744]:

- Higher UE speed correlates with decreased prediction accuracy

- Longer PW length correlates with decreased prediction accuracy

- Increasing the OW length can improve the prediction accuracy, especially when the OW length is relatively short. However, once the OW length exceeds a certain value, further increase of the OW length does not yield significant benefit.

To guarantee prediction accuracy for FR2 intra-frequency temporal domain case A, we think limited doppler shift (corresponds to limited UE speed), limited PW length and proper OW length shall be set.

Proposal 7: For intra-frequency temporal domain prediction case A, RAN4 to discuss whether observation window and prediction window length can be defined as core requirements.

In FR1 intra-frequency temporal domain prediction case B, the measurement results in PW are predicted and these time instances are completely skipped by UE. RAN2 provide two examples duplicated as below, and example 2 is recommended as baseline for evaluation.

Figure 5.2.1-2 Example 1 of intra-frequency temporal domain case B [TR38.744]

Figure 5.2.1-3 Example 2 of intra-frequency temporal domain case B [TR38.744]

Measurement Reduction Rate in temporal domain (MRRT) is used for case B in RAN2 evaluation work [TR38.744].

MRRT = skipped measurement time instances / total measurement time instances

It is observed that,

- Under different MRRT (e.g., 1:1, 2:3), the prediction accuracy would be different. Increasing MRRT would result in decreased prediction accuracy.

- Regarding example1 in figure 5.2.1-2 and example 2 in Figure 5.2.1-3, MRRT for both examples are the same, that is 0.5. However the prediction error may be not the same, even with the same MRRT value. Taking an extreme example, following the predication strategy like example 1, using100 measurements in OW predict 100 measurements in PW, MRRT is still 0.5. However the prediction accuracy of the sample in the farthest instance in PW would be degraded. In other words, even under the same MRRT setting, different measurement skipping patterns can result in different prediction accuracy.

Therefore for intra-frequency temporal domain prediction case B, RAN4 to discuss whether observation window and prediction window length with certain MRRT can be defined as core requirements.

Proposal 8: For intra-frequency temporal domain prediction case B, RAN4 to discuss whether observation window and prediction window length with certain MRRT can be defined as core requirements.

For inter-frequency inter-cell frequency domain measurement, the idea is to leverage the RSRP result on one cell to infer the RSRP on another cell in another frequency. The prediction accuracy in frequency domain would be evaluated and defined in RAN4 RRM. With different channel correlation coefficient between two inter-frequency carriers, the measurement prediction accuracy is different. In RAN2, only co-located scenario is considered where the cell to be measured and the cell to be predicted are located in the same sector/site with the same antenna configuration. RAN2 had some simulations and observed that

- the correlation between different frequency in FR1 has high correlation, e.g., correlation coefficient is ~0.9;

- the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy.

Proposal 9: For inter-frequency inter-cell frequency domain prediction, RAN4 to discuss whether only RSRP prediction accuracy under certain channel correlation coefficient is to be specified.

It is agreed model activation delay is to be defined for beam management in R19 AI/ML air interface. For RSRP prediction, the similar requirement can be defined. However we think the requirement is not crystal clear in R19 AI/ML beam management, such as what’s the exact components shall be included in activation delay. RAN4 can wait for more progress in R19 AI/ML air interface beam management.

Proposal 10: For RRM measurement prediction, whether and how to define activation delay can wait for more progress in R19 AI/ML air interface beam management.

Ground truth definition for RSRP Accuracy

Definition of L3 RSRP predication accuracy is agreed in TR38.744.

Both absolute accuracy and relative accuracy of predicted L3-RSRP are considered as the candidate metrics for RRM measurement prediction use cases for intra-frequency scenario and inter-frequency scenarios. The absolute accuracy of predicted L3-RSRP is defined as: - Absolute accuracy of predicted L3-RSRP = reported predicted L3-RSRP – ground truth of L3-RSRP. The relative accuracy of predicted L3-RSRP for intra-frequency prediction is defined as: - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported L3-RSRP of cell 2) – (ground truth of L3-RSRP of cell 1 – ground truth of L3-RSRP of cell 2), - cell 1 and cell 2 are on the same frequency - the reported L3-RSRP of cell 2 can be measured or predicted. Editor's Note: The relative accuracy of predicted L3-RSRP for beam level measurements may be further discussed during WI phase depending upon RAN2 progress. The relative accuracy of predicted L3-RSRP for inter-frequency prediction is defined as:- - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported L3-RSRP of cell 2) – (ground truth of L3-RSRP of cell 1 – ground truth of L3-RSRP of cell 2), - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1; - the reported L3-RSRP of cell 2 can be measured or predicted. NOTE: It is not precluded to update the definition based on further RAN2 progress in WI phase

It is agreed that for the ground truth of L3 RSRP in FR2, reported L3 RSRP measurement under sufficient high SNR is taken as a baseline.

The ground truth definition for L3 RSRP accuracy in FR1 is still not decided. There are two alternatives in TR38.744.

The ground truth of L3-RSRP in FR1 is based on: - Alt1: The transmitted or reception power - Alt2: The reported measurement value under certain conditions.

- Alternative 1: Ground truth for FR1 is based on the TE transmitted or reception power.

Channel conditions

In legacy FR1 conducted measurement accuracy test, UE antenna is directly connected to TE. The channel environment is AWGN.  As there is no channel fluctuation in time domain, TE can determine the ideal L3-RSRP result. For AI/ML based measurement prediction, to verify AI prediction capability, AWGN channel is not proper, and fading channel is to be employed.

Observation 1: Fading channel is to be employed for AI mobility measurement prediction.

The measurement time instances alignment between UE and TE

The time instance of ground truth of L3 RSRP shall be aligned with that of the predicted L3 RSRP. Then TE shall have the information on the specific time instance when UE performs prediction. As discussed above, for either intra-frequency temporal domain case B and case A, although the configuration of PW, OW and skipping pattern are known to TE, the specific position in time domain of UE obtaining a predicted result is unknown to TE.

Observation 2: For either intra-frequency temporal domain case B and case A, although the configuration of PW, OW and skipping pattern are known to TE, the specific position in time domain of UE obtaining a L3 predicted result is unknown to TE.

Number of samples of L1 measurement

In legacy L3 measurement, the measurement accuracy is specified at point A and no L1 filter is applied. For AI/ML measurement prediction, the L3 RSRP measurement is specified at point C. Then the ground truth of L3 RSRP result shall be determined by the linear average with multiple L1 measurement samples. Although the current L3 RSRP measurement period is defined based on 5 L1 samples, the actual number of L1 samples is up to UE implementation.

Observation 3: The number of L1 measurement samples is unknown to TE.

Based on the above analysis, the factors of measurement time instances alignment between UE and TE and number of samples of L1 measurement are obstacles to use ground truth definition Alt1.

Proposal 11: If the ground truth in FR1 is defined based on the TE transmitted or reception power (Alter 1), the following aspects are obstacles:

- The specific position in time domain of UE obtaining a L3 predicted result is unknown to TE.

- The number of L1 measurement samples is unknown to TE.

- Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions

In general, no big obstacles are observed if the ground truth for FR1 is defined based on UE reported value (Alt 2). One tricky point is that UE may cheat, as UE also know its reported value. The cheating issue is in parallel under discussion in R19 AI interface beam management test. We can wait more progress on R19 AI interface beam management

Proposal 12: The ground truth in FR1 can be defined based on the reported measurement value under certain conditions.

- how to avoid UE cheating can wait the progress in R19 AI interface beam management test.

Conclusion

The contribution provides analysis on RRM measurement prediction in AI/ML mobility. The following proposals are proposed:

Proposal 1: The way to add BB error in simulations:

- System-level simulation yields SINR of each target cell at each measurement occasion (per 40ms)

- Link-level simulation yields the CDF curves of BB errors corresponding to each SINR level.

- At the corresponding SINR level, generate a random number following a truncated Gaussian distribution with (50%, max[5%,95%]). Each generated random number (i.e., BB error) is then added to corresponding L1 measurement point.

Proposal 2: RF error remains constant over a period of time (e.g., 10 seconds). Starting from the 11th second (just an example), the RF error changes and follows a truncated Gaussian distribution with (0, 2.5).

Proposal 3: In testing dataset, RAN4 is to discuss what kind(s) of the benchmark for comparison shall be:

Option 1: No any error for the benchmark

Option 2: only RF error is considered for the benchmark

Option 3: Both BB error and RF error are considered for the benchmark

Proposal 4: For prediction accuracy, RAN4 need to discuss:

- Whether to define prediction accuracy for each sub-use case/scenario, or

- Whether to define one unified cell level prediction accuracy

- to accommodate all sub-use cases/scenario, or

- by selecting one typical and proper sub-use case/scenario

Proposal 5: RAN4 to discuss whether the prediction accuracy has the same or different accuracy requirements that are defined for the non-AI/ML RRM measurement.

Proposal 6: In AL/ML mobility, prediction reporting delay can be defined as core requirement and the definition can refer to the conclusion of beam management in Rel-19 NR_AIML_air.

Proposal 7: For intra-frequency temporal domain prediction case A, RAN4 to discuss whether observation window and prediction window length can be defined as core requirements.

Proposal 8: For intra-frequency temporal domain prediction case B, RAN4 to discuss whether observation window and prediction window length with certain MRRT can be defined as core requirements.

Proposal 9: For inter-frequency inter-cell frequency domain prediction, RAN4 to discuss whether only RSRP prediction accuracy under certain channel correlation coefficient is to be specified.

Proposal 10: For RRM measurement prediction, whether and how to define activation delay can wait for more progress in R19 AI/ML air interface beam management.

Observation 1: Fading channel is to be employed for AI mobility measurement prediction.

Observation 2: For either intra-frequency temporal domain case B and case A, although the configuration of PW, OW and skipping pattern are known to TE, the specific position in time domain of UE obtaining a L3 predicted result is unknown to TE.

Observation 3: The number of L1 measurement samples is unknown to TE.

Proposal 11: If the ground truth in FR1 is defined based on the TE transmitted or reception power (Alter 1), the following aspects are obstacles:

- The specific position in time domain of UE obtaining a L3 predicted result is unknown to TE.

- The number of L1 measurement samples is unknown to TE.

Proposal 12: The ground truth in FR1 can be defined based on the reported measurement value under certain conditions.

- how to avoid UE cheating can wait the progress in R19 AI interface beam management test.

Reference

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- TR38.744

- R4-2514827, WF on NR_AIML_Mob_RRM, OPPO

Annex- Simulation assumptions

The System level simulation parameters for FR1 are used in our simulation are provided as below.

| Parameters | Value for FR1 |
| --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and optional LOSsoft; Inter-frequency correlation model is optional. without UErotation,Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage. |
| System BW | 20MHz |
| UE speed | 30, 90km/h |
| UE distribution | 100% outdoor |
| BS Antenna Configuration | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 1TX beams are assumed. |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi |
| UE Antenna Configuration | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) 1RX beam is assumed |
| UE Antenna radiation pattern | Omni-direction |
| BS Tx Power | 44dBm |
| Maximum UE Tx Power | 23dBm |
| BS receiver Noise Figure | 5dB |
| UE receiver Noise Figure | 9dB |
| Inter site distance | 500m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5m |
| Spatial consistency | companies report one of the spatial consistency procedures: - Procedure B in TR38.901 |
| UE trajectory model | Option 1 in 38.843 section 6.3.1. |
| UE trajectory boundary processing model | Companies report which of the following models they used: boundary-terminated model |
| Sampling period | 40ms |

Table 2.2-2 (Table 5.2.1.1-2)

| L3 filtering parameter | value |
| --- | --- |
| FR1 FilterCoefficient | 4, 0 |

Table 2.2-3 (Table 5.2.1.1-3)

| Measurement period | value |
| --- | --- |
| FR1 to FR1 intra-frequency case A | 200ms |

Table 2.2-4 (Table 5.2.1.1-4)

| Consolidation parameter | value |
| --- | --- |
| nrofSS-BlocksToAverage for FR1 | 1 |
| nrofSS-BlocksToAverage for FR2 | 3 |
| absThreshSS-BlocksConsolidation for FR1 | -110dbm |
| absThreshSS-BlocksConsolidation for FR2 | -110dbm |

Table 2.2-5 Other parameters

| Parameters | Value for FR1 |
| --- | --- |
| L1 filtering | 5 samples Option 1: sliding Option 2: non-sliding |
| Observation window | 200ms |
| Prediction window | 200ms |

Table 2.2-6: 3-Sector BS antenna radiation pattern for above 6GHz (TR 38.802 Table A.2.1-6)

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8dBi |

Table 2.2-7: UE antenna radiation pattern model 1 (TR 38.802 Table A.2.1-8)

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |

Note: are in local coordinate system.