# 3GPP TSG-RAN WG4 Meeting #118                              R4-2600861

# Gothenburg, Sweden, Feb. 09-13, 2026

Agenda Item:	4.31.2

Source: 	CMCC

Title: 			Discussion on RRM core requirement and testing framework for beam management

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on RRM core requirements for beam management.

- Discussion

Issue 2-1: Measurement period for inference – case 2 Agreement: For BM-case 2 measurement period for inference, RAN4 only specifies the requirements for M=1. - K: number of RS transmission occasions, which can be measured per Rx beam. - M:CSI reference resource occasions Issue 2-2: TCI State Switching Clarifications Agreement - RAN4 to confirm that the statement ‘TCI state remains detectable’ means side condition for detectable is met throughout the time period. - The term ”predictable” won’t be used in the description of the side condition. Issue 2-6: Value of x Agreement: - Regarding the metric for beam ID only prediction, where top-1 predicted beam(s) are reported, RAN4 requirements are specified based on the Y% of probability that the ground truth RSRP of the predicted beam is larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – X dB - X and Y are TBD based on the simulations Regarding the metric for RSRP+beam ID prediction, only RSRP accuracy requirement will be defined.

One open issue is relative RSRP accuracy. As duplicated as below, there is definition of relative RSRP accuracy for case 1, but there are still some FFS.

Previous agreements on RSRP accuracy Issue 2-2: Absolute RSRP accuracy Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams Issue 2-3: Relative RSRP accuracy Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

One FFS for relative RSRP accuracy for case 1 is whether reported L1-RSRP can be measured RSRP. RAN1 agreed that for report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. It is clear that measured RSRP are not allowed to report.

RAN1 agreements in RAN1#120 Agreement For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Note: how to capture it in the spec is a separate discussion.

Proposal 1: for relative RSRP accuracy, it is proposed that the reported L1-RSRP cannot be measured RSRP, since RAN1 agreed that the RSRP of predicted beam(s)in the report of inference results is the predicted RSRP which is based on AI/ML output.

Another FFS is whether beam index n owns the largest reported value. As discussed above, RAN1 agree that only largest predicted RSRP is reported as absolute RSRP, other RSRP are reported as differential predicted RSRP with reference to the largest predicted RSRP. With the agreements, it is reasonable that beam index n owns the largest reported value.

Proposal 2: for relative RSRP accuracy, it is proposed that beam index n owns the largest reported value.

Based on above consideration, for case 1, the relative RSRP accuracy is proposed as (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value.

Proposal 3: for BM case 1, it is proposed that

- The relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]

BM case 2 is temporal downlink beam prediction for Set A of beams based on the historic measurement results of Set B of beams. The model output is the L1-RSRP prediction of furture N time instances with K beams for each time instances. According to RAN1 agreements on the inference report for BM case2, RSRP #1 of time instance #1 is absolute RSRP; and the remaining RSRP are differential RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1 of time instance #1. Time instance #1 corresponds to the time instance indicated by the time instance indicator. Since multiple time instances are consdiered, the relative accuracy for BM case2 need to be updated to reflect different time instance. Based on above consideration, the relative accuracy for BM case2 is proposed as (predicted L1-RSRP of beam index i for time instance m - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i for time instance m - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value, 1<=m<=M where M is the number of time instance.

Proposal 4: for BM case 2, it is proposed that

- The relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i for time instance m - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i for time instance m - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value among all the predicted beams. 1<=m<=M where M is the number of time instance

- Conclusion

This contribution provides discussion on RRM core requirements for beam management. The proposals are:

Proposal 1: for relative RSRP accuracy, it is proposed that the reported L1-RSRP cannot be measured RSRP, since RAN1 agreed that the RSRP of predicted beam(s)in the report of inference results is the predicted RSRP which is based on AI/ML output.

Proposal 2: for relative RSRP accuracy, it is proposed that beam index n owns the largest reported value.

Proposal 3: for BM case 1, it is proposed that

- The relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]

Proposal 4: for BM case 2, it is proposed that

- The relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i for time instance m - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i for time instance m - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value among all the predicted beams. 1<=m<=M where M is the number of time instance

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2522430, WF on [117][112] R19 AI for air interface, Qualcomm Incorporated

- R4-2514631, WF on [116bis][112] R19 AI for air interface, Qualcomm Incorporated

- R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm Incorporated

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm Incorporated

- R4-2505105, WF on requirements for AI/ML air interface normative work, Qualcomm Incorporated

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface