3GPP TSG-RAN WG4 Meeting #118                                            R4-2600891

Gothenburg, Sweden, Feb. 09 – 13, 2026

Source: 	Huawei, HiSilicon

Title: 	On UE output power requirements for 6G

Agenda Item:	8.3.1

Document for:	Approval

Introduction

Further to our previous contributions, we share our views on the open issues for UE RF as listed in the WF [1].

Discussion

Single carrier output power

Issue 1: Step size between power classes

RAN4 Agreement - Agreements for power class - Further study step size between power classes. Encourage further inputs considering e.g. - Benefits/drawbacks of potential finer granularity between power classes - Granularity for MPR/A-MPR requirement definition - Output power tolerance - REFSENS requirement impact - Potential regulatory restrictions - Flexibility for implementation

Based on the previous discussions, the step size between power classes can be narrowed down to two options:

-	Opt1: 3dB,

-	Opt2: 1dB.

Option 1 is the well-established conventional way. The main drawback seems to be sometimes imposing artificial restriction on the allowed output power, since the nominal MOP (Ppowerclass) is included in the upper limit (PCMAX_H) of Pcmax,c.

Option 2 would allow small incremental power levels between conventional power classes to be reached, such as 27/28dBm between PC2 and PC1.5. One of the main challenges for option 2 is how to define the corresponding MPR/A-MPR requirements.

The gating factors for MPR include: SEM, ACLR and EVM. Similar MPR allowances may be defined for regions that are gated by the ACLR and EVM limits for a range of MOP levels. But for regions that are gated by the SEM, more MPR is usually needed with the increase of MOP.

The gating factors for A-MPR can be additional ACLR, additional SEM and most often additional emission requirements that are imposed on one side of the channel. Higher A-MPR is usually needed with the increase of MOP.

Observation 1-1: In summary, it seems very challenging to define only one set of MPR or A-MPR requirements that are applicable for all MOP levels (with 1dB step size). On the other hand, to define one set of MPR/A-MPR requirements for each MOP level would bring excessive overhead to the specs.

The impact to REFSENS requirements should also be studied. Usually, the amount of desens for single-carrier FDD or inter-band CA depends on the Tx power level. One option is to scale the REFSENS requirements based on Tx power level, e.g., based on LUT with Tx power dependent scaling factors. Another option is to define and verify REFSENS requirements based on limited number Tx power levels.

The tolerance for MOP is the allowance to address the Tx power variation caused by component variation during mass production, temperature variation during operation and etc, known as PVT (process, voltage and temperature) variables. Such variations could happen for any target power level, and similar ranges may be needed regardless of the nominal MOP level.

It’s also worth noting that the nominal MOP is usually achievable only for the waveform with specific modulation and RB allocation, such as DFT-s-OFDM with BPSK/QPSK in the inner region. For other waveforms, network needs PHR from the UE to know the achievable Tx power (Pcmax,c).

Observation 1-2: If MPR/A-MPR is non-zero, the max Tx power of a UE is always in the range of PCMAX_L and PCMAX_H. Reporting the MOP with finer granularity does not necessarily reduce the uncertainty on the UE’s max Tx power from the network perspective.

Proposal 1-1: Further study two options of the step size for power classes: 3dB and 1dB. For 3dB step size, study how to allow output power levels between MOP and MOP+3dB. For 1dB step size, study how to address the challenges of defining corresponding MPR/A-MPR requirements as well as REFSENS requirements.

Issue 2: Power class for multi-Tx

RAN4 Agreement - Agreements for power class - Further study power class for multi-TX case (TxD, UL-MIMO) - Consider benefits and drawbacks for - Defining power class to be agnostic of number of Tx chains and/or PAs - Considering that each chain and/or PA should be able to reach maximum output power - Consider impact of transparent vs. non-transparent TxD

In 5GNR, the MOP of a power class is agnostic of the number of Tx chains/PAs. For example, PC2 can be implemented via 1Tx, 2Tx or 3Tx and PC1.5 via 2Tx or 4Tx. However, different MPR requirements are defined for different number of Tx chains for the same power class. The drawback of this way is that RAN4 needs to spend a significant amount of time to compare MPR from other companies and this way makes specification more complicated.

For 6GR, it’s possible to simplify the specifications by defining the MPR requirements based on one Tx number, i.e., specifying MPR based on larger number of Tx per power class, such as 2Tx for PC2 and 4Tx for PC1.5. If requirements for even larger number of Tx’s are introduced later, additional relaxation for (limited number of) certain entries in the MPR table can be allowed and specified. Since MPR is allowed power reduction, it is not a problem in terms of MPR definition. For more specific, for smaller number of Tx’s, less MPR might be needed but it's unnecessary to define tighter requirements in the specs, since MPR for the largest Tx can cover that for smaller number of Tx’s. The potential drawback is, however, that the gain of a feature like power boosting or MPR reduction becomes blur. For example, if MPR for 4Tx is defined as 3 dB and actual MPR for 1Tx is 1 dB (but not defined). Later if MPR reduction of 0.5 dB by a certain feature is introduced, the actual reduction for 1Tx operation is not clear. The other thing is that if network considers side conditions of MPR such as length of RBs, MCS etc., in their scheduler, network has no choice but scheduling resources in a conservative way.

For multiple-Tx with different PA capabilities, such as PC2+PC3, what’s the applicable MPR/A-MPR if MOP of 27.8 dBm is allowed? This is a similar question that is faced when considering to define power classes with 1dB step size, e.g., what's the MPR/A-MPR for MOP=28dBm?

From the output power perspective, the following performance gain is expected for different PA configurations:

PC2 ≤ PC2 + PC3 ≤ PC2 + PC2.

For the sake of simplicity, if only MPR is considered, we have:

26dBm – MPRPC2, 1Tx ≤ 27.8dBm – MPRPC2+PC3 ≤ 29 – MPRPC1.5,2Tx

And some bounds for MPR PC2+PC3 can be obtained as follows, assuming the MPR for PC1.5 and PC2 are already defined:

MPRPC1.5,2Tx – 1.2dB ≤ MPRPC2+PC3 ≤ MPRPC2, 1Tx +1.8dB.

Proposal 1-2: It’s feasible to define power class related requirements based on one Tx number, provided that small adjustments for other Tx numbers are allowed, while drawback of this way in terms of concurrent usage of other feature like power boosting/MPR reduction and NW scheduling should be further discussed. For multi-Tx with different PA capabilities, the MPR requirements can be bounded by those for lower/higher power classes. The details are FFS.

Issue 3: How to enable fully utilizing PA capabilities

RAN4 Agreement - Agreements for power class - Further study how to enable fully utilizing PA capabilities considering e.g. - Relaxation or removal of upper tolerance without sacrificing output power accuracy - Power boosting - Maximum power capability reporting - Removing the nominal power limit in the PCMAX upper bound

As discussed earlier, the tolerance for MOP is the allowance to address the Tx power variation caused by PVT variations, and it could be part the regulation requirements, especially the upper tolerance. It’s preferred not to change the tolerance.

Power boosting has been adopted in 5G NR. The main drawback is the signaling overhead as well as the complexity and obscurity of the corresponding requirements defined in the specs, which makes it difficult to understand and implement.

Removing the MOP (Ppowerclass) limit in the PCMAX_H is a straightforward way to enable higher output power with little overhead. For example, the current upper bound for single carrier Tx power is:

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass + ΔPPowerBoost}.

By taking away Ppowerclass related constraint, the Tx power is only bounded by PEMAX, which is necessary to be kept in order for the network to have control. Alternatively, (Ppowerclass + [2] dB) may be used to ease the concern on “no upper limit”, i.e.,

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass + [2]dB}.

Proposal 1-3: Further study the following options:

- Allow higher Tx power by increasing the upper bound (PCMAX_H) by [2] dB, e.g., PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass + [2]dB};

- Simplify the requirements for power boosting, make them easy to understand and implement.

Issue 4: Default power class

RAN4 Agreement - Agreements for power class - Further study need for default power class and default power class values considering e.g. - Network UL coverage / UL link budget at different frequencies/ System performance - Impact on REFSENS definition for FDD bands - Maximum channel bandwidth - Cell planning from the operators - Potential regulatory output power restrictions

Regarding the FR1 default power class for 6G, there seems to be good consensus on PC2 for TDD bands and PC3 for FDD bands. To facilitate cell planning for operators and guarantee UL coverage, the default power class for a band means that the UE operating on this band should at least support the default power class. Lower power classes might be allowed for less-capable UEs, such as RedCap UEs in 5GNR or IoT UEs.

Proposal 1-4: The default power class for a band means that the UE operating on this band should at least support the default power class. It’s feasible to set the default power class to PC2 for TDD bands and PC3 for FDD bands. Lower power classes should be allowed for specific types of UEs, such IoT UEs.

For example, in order to fully utilize the large bandwidth of the ~7GHz band, higher Tx power is desired. The ~7GHz band is considered to be the key value-added band for 6GR. Owing to higher frequency, more antennas might be integrated into the handsets. Equipping two Tx chains with PC2 PAs enables ready support for PC1.5.

Proposal 1-5: Study and evaluate the performance benefits for different Power Classes for ~7GHz including PC1.5.

Issue 5: SAR and UL duty cycle

RAN4 Agreement - Agreements for configured maximum output power - Further study SAR solution considering - P-MPR as baseline below 6 GHz - Whether/how SAR or MPE applies at higher than 6GHz and applicable solutions - Achieving more clarity on UE output power - Further study potential UL duty cycle requirement to optimize component size/cost considering - Use cases and deployment scenarios as well as network needs

As pointed out by several companies, modern handsets are equipped with multiple radios including those for WiFi, Bluetooth and etc. These non-3GPP radios can operate simultaneously with 3GPP radio, and generate emissions that are absorbed by the human body. Given the dynamics of the multiple-radio operations, it’s better to let the UE to manage the SAR solution. Therefore, the widely deployed P-MPR solution should be the baseline. Other solutions can be studied, but they have to be implementable and verifiable, and also show clear advantages compared with P-MPR.

Proposal 1-6: Regarding solutions for meeting SAR/MPE/APD requirements, P-MPR should be the baseline. For UE thermal management, P-MPR can also be considered.

Issue 6: Configured maximum power equation parameters

RAN4 Agreement - Agreements for configured maximum output power - Further review the 5G configured Tx power equation parameters considering at least the following - ∆TC,c : can it be removed, considering also new wider CBW and FFS on whether and where to accommodate this relaxation in requirements. - ∆TRxSRS: should it be separated from general Pcmax equation as it only addresses SRS transmission and FFS on how it can be accommodated. - ∆Tib: can the requirement be removed and accommodated to per band power capability and FSS on whether and where to accommodate this relaxation in requirements.

The Tx power equation parameters such as: ∆TC,c, ∆TRxSRS, ∆Tib are used to address issues encountered in practical implementations, which are unlikely to disappear in 6GR. Where to specify them can be further studied.

Proposal 1-7: The relaxation introduced by the parameters of ∆TC,c, ∆TRxSRS, ∆Tib are still needed to accommodate practical implementation limitations. How to specify them can be further studied and improved.

CA output power

RAN4 Agreement - Strive to improve clarity on UE power ability under different UE condition (e.g. configured, activated, scheduled) in all output power requirements - Study how to maximize the UL transmission power capability for CA considering e.g. - feasibility of maintaining the ‘single carrier’ UE output power capability of a ‘primary’ or ‘anchor’ carrier at configuration/activation of the spectrum aggregation scheme - single active/configured UL carrier within a DL multi-carrier bandwidth should not increase allowed UE power reduction compared to single carrier - Whether CA power class it still needed for inter-band CA or power class per band or per band per BC is sufficient - Identify if any there is any motivation from RAN4 perspective to limit total output power and whether this needs to be communicated to RAN1. - Recognize that need for total power limit can also depend on RAN1 design of multicarrier schemes - Prioritize discussion for single carrier in RAN4#118

Observation 2-1: The following use cases can be considered under this sub-topic:

- 1: UE is configured with single-carrier UL and single-carrier DL on a band

- 2: UE is configured with DL CA (inter/intra/both) on ≥1 band(s) and

  - 2a: configured with single-carrier UL on one band or

  - 2b: configured with UL CA but only single-carrier activated or

  - 2c: configured with UL CA and all activated but only single-carrier scheduled

In the following, it’s assumed that the concept of RRC configuration/activation will remain valid for 6G.

Case 1 is the simplest to implement and the UE can potentially support the highest power class compared with other use cases. And in this case, a UE can potentially indicate the support of TxD and/or UL MIMO. In 5G NR, the power class is assumed to be the same regardless if UL MIMO is configured or not. Subject to the PA configuration, the UE can further indicate different ULFPTx modes.

Proposal 2-1: For the case (#1) when the UE is configured with single-carrier UL and single-carrier DL on a band, further study how to efficiently support TxD and ULFPTx, including UE RF requirements as well as capability signaling design, by considering but not limited to the underlying PA configuration as well as insertion loss difference between different Tx paths.

There had been prolonged discussion in RAN4 in Rel-18 regarding the power class for case 2a (i.e., when UE is configured with DL CA but single-carrier UL). It was proposed that the UE should mandatorily support the same power class as for case 1. Although desirable, many UE vendors preferred more flexibility for implementation. Technically speaking, the Tx insertion loss could become higher when more bands are aggregated in the DL. Additionally, the power consumption and thermal management caused by increased DL CA processing could also constrain the potential Tx power.

Proposal 2-2: For the case (#2a) when the UE is configured with DL CA but single-carrier UL, the power class per band could be considered if there are no strong concerns from UE vendors.

For case 2b, a UE could potentially support the same power class as for case 2a. The main concern is that the Tx power capability per carrier could change if another carrier is activated in the UL. It is noted that power control is performed not per band, but rather per carrier/CC. The UE may need to re-configure its PAs and LOs in order to support the additional carrier. For example, PC1.5 with 2Tx may be supported when only one carrier is activated, while the Tx power capability could downgrade to PC2 for each carrier if both are activated and each PA covers one carrier each.

Proposal 2-3: For the case (#2b) when a UE is configured with UL CA but with only single-carrier activated, the UE might be able to support the same power class as case #2a. However, as another UL carrier could be activated by the network at any time, it’s very challenging for the UE to decide its transceiver configuration during this status.

Case 2c is the most dynamic one, since the UL scheduling could switch between one carrier and two carriers in the matter of several slots. A UE could potentially support higher power class for the single-carrier if it can switch its PAs and LOs to the same carrier in time. Practically speaking, Tx switching time and DL interruption time might be needed.

Proposal 2-4: For the case (#2c) when a UE is configured with UL CA, both activated but only single-carrier scheduled, the UE may be able to support a higher power class than that for two scheduled carriers, provided that the UE is capable of Tx switching (for intra-band or inter-band). The RF requirements (including power class, Tx switching time, DL interruption, etc.) as well as capability signaling should be considered within the generic Tx switching framework, which is FFS.

For inter-band UL CA, the power class for the band combination as defined in NR might not be needed. Instead, the power class per band per BC is the key to determine the maximum total Tx power as well as the max Tx power per band under CA.

For Tx switching in 5G, it’s unclear what are the applicable power class capabilities for a band or for a BC after Tx switching is performed among multiple bands. This should be clarified when developing 6GR specifications.

Proposal 2-5: For inter-band UL CA, the power class per band per BC should be considered. For inter-band Tx switching, the applicable power class per band in a band pair should be further studied.

Conclusion

This contribution provides our considerations on UE output power for 6G.

Single carrier output power

Issue 1: Step size between power classes

Observation 1-1: In summary, it seems very challenging to define only one set of MPR or A-MPR requirements that are applicable for all MOP levels (with 1dB step size). On the other hand, to define one set of MPR/A-MPR requirements for each MOP level would bring excessive overhead to the specs.

Observation 1-2: If MPR/A-MPR is non-zero, the max Tx power of a UE is always in the range of PCMAX_L and PCMAX_H. Reporting the MOP with finer granularity does not necessarily reduce the uncertainty on the UE’s max Tx power from the network perspective.

Proposal 1-1: Further study two options of the step size for power classes: 3dB and 1dB. For 3dB step size, study how to allow output power levels between MOP and MOP+3dB. For 1dB step size, study how to address the challenges of defining corresponding MPR/A-MPR requirements as well as REFSENS requirements.

Issue 2: Power class for multi-Tx

Proposal 1-2: It’s feasible to define power class related requirements based on one Tx number, provided that small adjustments for other Tx numbers are allowed, while drawback of this way in terms of concurrent usage of other feature like power boosting/MPR reduction and NW scheduling should be further discussed. For multi-Tx with different PA capabilities, the MPR requirements can be bounded by those for lower/higher power classes. The details are FFS.

Issue 3: How to enable fully utilizing PA capabilities

Proposal 1-3: Further study the following options:

- Allow higher Tx power by increasing the upper bound (PCMAX_H) by [2] dB, e.g., PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass + [2]dB};

- Simplify the requirements for power boosting, make them easy to understand and implement.

Issue 4: Default power class

Proposal 1-4: The default power class for a band means that the UE operating on this band should at least support the default power class. It’s feasible to set the default power class to PC2 for TDD bands and PC3 for FDD bands. Lower power classes should be allowed for specific types of UEs, such IoT UEs.

Proposal 1-5: Study and evaluate the performance benefits for different Power Classes for ~7GHz including PC1.5.

Issue 5: SAR and UL duty cycle

Proposal 1-6: Regarding solutions for meeting SAR/MPE/APD requirements, P-MPR should be the baseline. For UE thermal management, P-MPR can also be considered.

Issue 6: Configured maximum power equation parameters

Proposal 1-7: The relaxation introduced by the parameters of ∆TC,c, ∆TRxSRS, ∆Tib are still needed to accommodate practical implementation limitations. How to specify them can be further studied and improved.

CA output power

Observation 2-1: The following use cases can be considered under this sub-topic:

- 1: UE is configured with single-carrier UL and single-carrier DL on a band

- 2: UE is configured with DL CA (inter/intra/both) on ≥1 band(s) and

  - 2a: configured with single-carrier UL on one band or

  - 2b: configured with UL CA but only single-carrier activated or

  - 2c: configured with UL CA and all activated but only single-carrier scheduled

Proposal 2-1: For the case (#1) when the UE is configured with single-carrier UL and single-carrier DL on a band, further study how to efficiently support TxD and ULFPTx, including UE RF requirements as well as capability signaling design, by considering but not limited to the underlying PA configuration as well as insertion loss difference between different Tx paths.

Proposal 2-2: For the case (#2a) when the UE is configured with DL CA but single-carrier UL, the power class per band could be considered if there are no strong concerns from UE vendors.

Proposal 2-3: For the case (#2b) when a UE is configured with UL CA but with only single-carrier activated, the UE might be able to support the same power class as case #2a. However, as another UL carrier could be activated by the network at any time, it’s very challenging for the UE to decide its transceiver configuration during this status.

Proposal 2-4: For the case (#2c) when a UE is configured with UL CA, both activated but only single-carrier scheduled, the UE may be able to support a higher power class than that for two scheduled carriers, provided that the UE is capable of Tx switching (for intra-band or inter-band). The RF requirements (including power class, Tx switching time, DL interruption, etc.) as well as capability signaling should be considered within the generic Tx switching framework, which is FFS.

Proposal 2-5: For inter-band UL CA, the power class per band per BC should be considered. For inter-band Tx switching, the applicable power class per band in a band pair should be further studied.

- References

[1] R4-2522451, WF for [117][102] 6G general RF and UE RF, FL (Qualcomm), RAN4#117

[2]	R4-2513305, Consideration on UE RF study for 6G, Huawei, HiSilicon, RAN4#116bis