# 3GPP TSG-RAN WG4 Meeting #118		                                                     R4-26xxxxx

# Gothenburg, Sweden, Feb. 09-13, 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on test cases for CSSF enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2026-01-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This draftCR is based on R4-2522842 endorsed in RAN4#117 meeting, and further update the capability signalling and parameter configuration in test case of CSSF enhancement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the corresponding signalings. - Correct the parameters in the tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for NR RRM phase 5 are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.6.2.X, A.6.6.2.X, A.7.6.2.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.4.6.2.X	EN-DC: event triggered reporting tests under non-DRX

A.4.6.2.X.1	Test purpose and Environment

The purpose of this test is to verify that the UE which supports interFrequencyMeas-Nogap-r16 and threeCarrierMeasWithoutGap-r19 makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search without measurement gap requirements in clause 9.3.9.

The UE is only required to pass one of the three tests in A.4.6.2.x for FR1 EN-DC, A.6.6.2.x for FR1 CA, A.7.6.2.Y for FR1 and FR2 CA.

A.4.6.2.X.2	Test parameters

In this test, LTE cell 1 as PCell in FR1 on LTE RF channel 1, NR cell 2 as PSCell in FR1 on NR RF channel 2, NR cell 3 as SCell in FR1 on NR RF channel 3. NR cell 4 as neighbour cell in FR1 on NR RF channel 4 which is in the same band as NR Cell 3.  The SSB of Cell 4 is completely within UE’s active BWP BW. The RBs containing SSB from Cell 3 and Cell 4 should be different in frequency location within the cell bandwidth.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration

T1, the UE shall not have any timing information of NR Cell 4.

UE needs to be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource

at every DRX cycle.

Table A.4.6.2.X.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE cell: LTE FDD FR1 cell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.4.6.2.X.2-2: General test parameters for inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 2 |  |
| NR RF Channel Number |  | Config 1 | 2,3,4 |  |
| E-UTRA RF Channel Number |  | Config 1 | 1 | One E-UTRAN FDD carrier frequency is used. |
| Active cell |  | Config 1 | LTE cell 1 (PCell), NR cell 2(PSCell), NR cell 3(SCell) |  |
| Neighbour cell |  | Config 1 | NR cell 4 |  |
| SMTC-SSB parameters on NR RF Channels |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 for FR1 and A.3.10.2 for FR2 |
| A3-Offset | dB | Config 1 | -6 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| A3-Threshold | dBm | Config 1 | -11 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3 s | Synchronous NR CA Synchronous EN-DC |
|  |  | Config 1 | 3s | Synchronous cells |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 1.5 |  |

Table A.4.6.2.X.2-3: Cell specific test parameters for inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Serving cell 1 |  | Serving cell 2,3 |  | Neighbour Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| TDD configuration |  |  | Config 1 | Not Applicable |  | Not Applicable |  | Not Applicable |  |
| BWchannel |  | MHz | Config 1 |  |  | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  |
| Data PRBs allocated |  |  | Config 1 |  |  | 52 |  | 52 |  |
| BWP BW |  | MHz | Config 1 |  |  | 52 |  | 52 |  |
| BWP configuration | Initial DL BWP |  | Config 1 |  |  | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 |  |  | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  | Config 1 |  |  | SR.1.1 TDD |  | - |  |
| measurement channel |  |  |  |  |  |  |  |  |  |
| RMSI CORESET Reference |  |  | Config 1 |  |  | CR.1.1 TDD |  | - |  |
| Channel |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 |  |  | CCR.1.1 TDD |  | - |  |
| SMTC configuration defined |  |  | Config 1 |  |  | SMTC.2 |  | SMTC.5 |  |
| in A.3.11.1 and A.3.11.2 |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 |  |  | 15 |  | 15 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 |  |  | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Noc |  | dBm/SCS | Config 1 |  |  | -98 |  |  |  |
| Ês |  | dBm/SCS | Config 1 |  |  | -94 |  | -Infinity | -91 |
| SSB_RP Note 3 |  | dBm/SCS | Config 1 |  |  | -94 |  | -Infinity | -91 |
|  |  | dB | Config 1 |  |  | 4 |  | -Infinity | 7 |
| Ês /Noc |  | dB | Config 1 |  |  | 4 |  | -Infinity | 7 |
| Io |  | dBm/9.36 MHz | Config 1 |  |  | -64.60 |  | -Infinity | -61.26 |
| Propagation Condition |  |  | Config 1 |  |  | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Void NOTE 3: SS B_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void NOTE 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 6: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 7: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |

A.4.6.2.X.3	Test Requirements

In this test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2

UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of Change 1>

<Start of Change 2>

A.6.6.2.X	SA event triggered reporting tests under non-DRX

A.6.6.2.X.1	Test purpose and Environment

The purpose of this test is to verify that the UE which supports interFrequencyMeas-Nogap-r16 and fr1-CA-NR-DC-r19 indicated via threeCarrierMeasWithoutGap-r19 makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search without measurement gap requirements in clause 9.3.9.

The UE is only required to pass one of the three tests in A.4.6.2.x for FR1 EN-DC, A.6.6.2.x for FR1 CA, A.7.6.2.Y for FR1 and FR2 CA.

A.6.6.2.X.2	Test parameters

In this test, NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 as SCell in FR1 on NR RF channel 2. NR cell 3 as neighbour cell in FR1 on NR RF channel 3 which is in the same band as NR Cell 1. The SSB of Cell 3 is completely within UE’s active BWP BW. The RBs containing SSB from Cell 1 and Cell 3 should be different in frequency location within the cell bandwidth. Supported test configurations are shown in table A.6.6.2.X.2-1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration

T1, the UE shall not have any timing information of NR Cell 3.

UE needs to be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource

at every DRX cycle.

Table A.6.6.2.X.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.2.X.2-2: General test parameters for inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test 1 | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1,2,3 |  |
| E-UTRA RF Channel Number |  | NA | NA |  |
| Active cell |  | Config 1 | NR cell 1 (PCell), NR cell 2(SCell), |  |
| Neighbour cell |  | Config 1 | NR cell 3 |  |
| SMTC-SSB parameters on NR RF Channels |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 for FR1 |
| A3-Offset | dB | Config 1 | -6 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| A3-Threshold | dBm | Config 1 | -11 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3 s | Synchronous NR CA Synchronous EN-DC |
|  |  | Config 1 | 3s | Synchronous cells |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 1.5 |  |

Table A.6.6.2.X.2-3: Cell specific test parameters for inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter |  | Unit | Test configuration | Serving cell 1 |  | Serving cell 2 |  | Neighbour Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  |  | Config 1 | FDD |  | FDD |  | FDD |  |
| TDD configuration |  |  | Config 1 | N/A |  | N/A |  | N/A |  |
| BWchannel |  | MHz | Config 1 | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  |
| Data PRBs allocated |  |  | Config 1 | 52 |  | 52 |  | 52 |  |
| BWP BW |  | MHz | Config 1 | 52 |  | 52 |  | N/A |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  | Config 1 | SR.1.1 FDD |  | - |  | - |  |
| measurement channel |  |  |  |  |  |  |  |  |  |
| RMSI CORESET Reference |  |  | Config 1 | CR.1.1 FDD |  | - |  | - |  |
| Channel |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.1.1 FDD |  | - |  | - |  |
| SMTC configuration defined |  |  | Config 1 | SMTC.2 |  | SMTC.2 |  | SMTC.5 |  |
| in A.3.11.1 and A.3.11.2 |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 15 |  | 15 |  | 15 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Noc |  | dBm/SCS | Config 1 | -98 |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1 | -94 |  | -94 |  | -Infinity | -91 |
| SSB_RP Note 3 |  | dBm/SCS | Config 1 | -94 |  | -94 |  | -Infinity | -91 |
| Ês /Noc |  | dB | Config 1 | 4 |  | 4 |  | -Infinity | 7 |
|  |  | dB | Config 1 | 4 |  | 4 |  | -Infinity | 7 |
| Io |  | dBm/9.36 MHz | Config 1 | -64.60 |  | -64.60 |  | -Infinity | -62.26 |
| Propagation Condition |  |  | Config 1 | AWGN |  | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Void NOTE 3: SS B_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void NOTE 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 6: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 7: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |

A.6.6.2.X.3	Test Requirements

In this test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2.

UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of Change 2>

<Start of Change 3>

A.7.6.2.Y	SA event triggered reporting tests for FR2 under non-DRX

A.7.6.2.Y.1	Test purpose and Environment

The purpose of this test is to verify that the UE which supports interFrequencyMeas-Nogap-r16 and fr1-FR2-CA-r19 indicated via threeCarrierMeasWithoutGap-r19 makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search without measurement gap requirements in clause 9.3.9.

The UE is only required to pass one of the three tests in A.4.6.2.x for FR1 EN-DC, A.6.6.2.x for FR1 CA, A.7.6.2.Y for FR1 and FR2 CA.

A.7.6.2.Y.2	Test parameters

In this test, NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 as SCell in FR2 on NR RF channel 2. NR cell 3 as neighbour cell in FR2 on NR RF channel 3 which is in the same band as NR Cell 2.  The SSB of Cell 3 is completely within UE’s active BWP BW. The RBs containing SSB from Cell 2 and Cell 3 should be different in frequency location within the cell bandwidth.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration

T1, the UE shall not have any timing information of NR Cell3.

UE needs to be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource

at every DRX cycle.

Table A.7.6.2.Y.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | FR1 cell: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode FR2 cell: 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.6.2.Y.2-2: General test parameters for inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1,2,3 |  |
| E-UTRA RF Channel Number |  | NA | NA | One E-UTRAN FDD carrier frequency is used. |
| Active cell |  | Config 1 | NR cell 1 (PCell), NR cell 2(SCell), NR cell 3(SCell) |  |
| Neighbour cell |  | Config 1 | NR cell 3 |  |
| SMTC-SSB parameters on NR RF Channels |  | Config 1 | SSB.1 FR1 SSB.1 FR2 | As specified in clause A.3.10.1 for FR1 and A.3.10.2 for FR2 |
| A3-Offset | dB | Config 1 | -6 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| A3-Threshold | dBm | Config 1 | -11 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3 s | Synchronous NR CA Synchronous EN-DC |
|  |  | Config 1 | 3s | Synchronous cells |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 3 for power class 1, or 1.5 for other power class. |  |

Table A.7.6.2.Y.2-3: Cell specific test parameters for inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter |  | Unit | Test configuration | Serving cell 1 |  | Serving cell 2 |  | Neighbour Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | N/A |  | Setup 3 as specified in clause A.3.15 |  | Setup 3 as specified in clause A.3.15 |  |
| Beam AssumptionNote 7 |  |  | Config 1 | N/A |  | Rough |  | Rough |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  |  | Config 1 | FDD |  | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 10: NPRB,c = 52 |  | 100: NPRB,c = 66 |  | 100: NPRB,c = 66 |  |
| Data PRBs allocated |  |  |  | Config 1 |  | 52 |  |  | 66 |  |  | 66 |  |  |
| BWP BW |  | MHz | Config 1 | 52 |  | 66 |  | 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  | Config 1 | SR.1.1 FDD |  | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET Reference |  |  | Config 1 | SR.3.1 TDD |  | CR.3.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.1.1 FDD |  | CCR.3.1 TDD |  | N/A |  |
| SMTC configuration defined |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 15 |  | 120 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Noc |  | dBm/SCS | Config 1 | -98 |  | -89 |  | -89 |  |
| Ês |  | dBm/SCS | Config 1 | -94 |  | -85 |  | -Infinity | -85 |
| SSB_RP Note 3 |  | dBm/SCS | Config 1 | -94 |  | -85 |  | -Infinity | -85 |
| BB Note 8 |  | dB | Config 1 | 4 |  | 4 |  | -Infinity | 4 |
| Io |  | dBm/95.04 MHz | Config 1 | N/A |  | -54.56 |  | -60.01 | -54.56 |
|  |  | dBm/9.36 MHz | Config 1 | -64.60 |  | N/A |  | N/A | N/A |
| Ês /Noc |  | dB | Config 1 | 4 |  | 4 |  | -Infinity | 4 |
| Propagation Condition |  |  | Config 1 |  |  | No external noise (Note 9) |  | No external noise (Note 9) |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Void NOTE 3: SS B_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void NOTE 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 6: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 7: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. NOTE 9: The downlink connection between the System Simulator and the UE is without Additive White Gaussian Noise, and has no fading or multipath effects as specified in TS 38.521-2 B.0 [40]. |  |  |  |  |  |  |  |  |  |

A.7.6.2.Y.3	Test Requirements

In this test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

2.4s for UE supporting power class 1, or

1.44s for UE supporting other power class.

UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of Change 3>