3GPP TSG-RAN WG4 Meeting #118				                                                                     R4-2600953

Gothenburg, Sweden, Feb. 09-13, 2026

Agenda Item:	8.8.4

Source: 	OPPO

Title: 	6G AI/ML use cases for RRM

Document for:	Approval

- Introduction

For the upcoming 6G, the SID [1] of 6GR was approved in RAN#108 meeting. The objectives related to AI air interface for 6GR are listed as follows.

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

And in last two meeting, the use cases for RRM were discussed. The agreements were captured in the WF [2][3] and the remaining issues were captured in the summary [4]. In this contribution, we would like to further provide our views on AI/ML use cases for RRM.

- Discussion

In last meeting, the AI/ML use cases for RRM were discussed and four use cases were agreed as the first priority in this meeting based on the following agreement. In this part, we provide our analysis based on the the exemplary template indicated in [2].

Agreement: 1. Postpone L1-beam level prediction for Tx beam 2. Further discussion on the following identified the sub use cases under AI for RRM use case in RAN4#118 as the first priority a. For L3-beam level prediction for Tx beam i. FR1/FR2-1 spatial domain (intra-cell) ii. FR1/FR2-1 spatial domain (inter-cell, non-collocated) iii. FR1/FR2-1 frequency domain (inter-cell, non-collocated) b. For L3-cell level prediction i. FR1/FR2-1 freq. domain (inter-cell, non-collocated) c. Dynamic Adaptation of Measurement Procedure (single domain prediction based, no RSRP prediction is involved) d. Spatial domain RX beam sweeping reduction (L1/L3)

2.1 L3 beam and cell level prediction

For L3-beam level prediction for Tx beam, there are three candidate sub-use cases which include spatial domain and frequency domain prediction for both FR1 and FR2-1. But since the assumption in FR1 is omnidirectional reception, we think we do not need to consider spatial domain prediction for FR1.

Proposal 1: RAN4 not to consider spatial domain prediction for L3 beam level measurement in FR1.

For FR2-1 spatial domain prediction which includes intra-cell and inter-cell prediction, the corresponding details are listed in the following Table 1 based on our understanding. The analysis is based on single cell approach and the cluster based approach can be further discussed when the methodology is clear.

Table 1: Analysis for sub-use case i and ii: intra-cell/inter-cell spatial domain prediction

| Use case a – sub use case i and ii: (intra-cell and inter-cell spatial domain prediction) |  | OPPO |
| --- | --- | --- |
| AI model input | Input in training | 1) L3 beam measurement based on set B of beams at time t1 for cell 1. 2) L3 beam measurement based on set B of beams at historic time instance(s) until t2 for cell 1. |
|  | Input in inference | 1) L3 beam measurement based on set B of beams at time t1 for cell 1. 2) L3 beam measurement based on set B of beams at historic time instance(s) until t2 for cell 1. |
| AI model output | Label in training (if applicable) | 1) L3 beam measurement based on set A of beams at time t1 for cell 1 or cell 2. 2) L3 beam measurement based on set A of beams at t2+t for cell 1 or cell 2. |
|  | Output in inference | 1) Predicted L3 beam measurement based on set A of beams at time t1 for cell 1 or cell 2. 2) Predicted L3 beam measurement based on set A of beams at t2+t for cell 1 or cell 2. |
| Assumption on training | Training type | Offline training |
|  | Label construction (if applicable) | L3 measurement data of full set of beams for both cells |
| model location for inference |  | UE-sided model or NW-sided model |
| Collaboration/interaction between UE and NW |  | Intra-cell: No collaboration; NW configures the reference signals as legacy. Inter-cell: No collaboration for NW-sided model; NW needs to transmit assistant information on cell deployment to UE for UE-sided model. |
| Evaluation methodology |  | System level simulation |
| Evaluation assumption |  | Reference to Rel-20 AI mobility |
| Evaluation KPI |  | Predicted RSRP accuracy |
| Evaluation benchmark |  | UE actual measurement |
| Preliminary evaluation results |  |  |
| High level potential specification impact |  | For UE-sided model: Measurement and prediction related core requirements; RSRP prediction accuracy requirements For NW-sided model: Main impact on RAN2 and no RAN4 impact |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Intra-cell: No big difference compared to 5G-A AI/ML use cases. Inter-cell: generalization issue, high complexity and additional assistant information |

Based on the above analysis, the use case of intra-cell spatial domain prediction is similar as the AI/ML based BM use case defined in Rel-19. And we do not see much new aspects compared to L1 beam prediction. The possible difference may include the impact of L3 filtering and the applicability to neighbor cell for L3 beam level measurement. So the methodology of AI/ML based BM can be used as starting point for intra-cell spatial domain prediction.

Observation 1: Compared to Rel-19 AI/ML based BM, no big difference is observed for intra-cell spatial domain prediction for L3 beam level measurement.

Proposal 2: Identify the difference between intra-cell spatial domain prediction for L3 beam level measurement and Rel-19 AI/ML based BM.

For non-collocated inter-cell spatial domain prediction, if the prediction is at the same time, we think it is not necessary because UE is supposed to measure all the detectable cells using single Rx beam at the same occasion in legacy. If the intention is only to reduce the measurement on subset of cells at this time, we think the currently expected gain is not enough considering the complexity introduced by AI/ML prediction.

If the prediction is for future time, it will be multiple domains prediction and the complexity is high since there are many uncertainties regarding channel condition, beam and detectable cells which may be unpredictable.

In addition, for UE-sided model, network deployment is needed for training to get the non-collocated characteristics which requires additional assistant information. And even if the information is provided to UE, the model can only be localized. When the UE moves to another place, the model is hard to apply, i.e., the generalization cannot be guaranteed. So for this use case, we think UE-sided model is not proper.

If considering NW-sided model, we understand network can only provide the expected measurement results for Tx beams, but UE still need measurements to decide the Rx beams. We do not see clear performance gain comparing to legacy measurement or single time domain prediction. And since no RAN4 impact is expected for NW-sided model, the use case should be left to RAN2 to discuss.

Observation 2: The benefits for non-collocated inter-cell spatial domain prediction are not clear while several feasibility issues are observed, e.g., generalization, high complexity.

Proposal 3: Deprioritize the non-collocated inter-cell spatial domain prediction for L3 beam level prediction for Tx beams.

For non-collocated frequency domain prediction for L3 beam level and cell level measurement, the analysis are provided in the following Table 2 based on our understanding.

Table 2: Analysis for sub-use case iii and use case b: frequency domain prediction

| Use case a – sub use case iii; and Use case b: (Frequency domain prediction) |  | OPPO |
| --- | --- | --- |
| AI model input | Input in training | L3 beam/cell level measurement at time t1 for frequency layer f1. |
|  | Input in inference | L3 beam/cell level measurement at time t1 for frequency layer f1. |
| AI model output | Label in training (if applicable) | L3 beam/cell level measurement at time t1 for frequency layer f2. |
|  | Output in inference | Predicted L3 beam/cell level measurement at time t1 for frequency layer f2. |
| Assumption on training | Training type | Offline training |
|  | Label construction (if applicable) | L3 beam/cell measurement data for both frequency layers |
| model location for inference |  | NW-sided model or UE-sided model |
| Collaboration/interaction between UE and NW |  | For NW-sided model: No collaboration, NW configures the reference signals and UE reports the measurement results as legacy. For UE-sided model: NW needs to transmit assistant information on cell deployment to UE. |
| Evaluation methodology |  | System level simulation |
| Evaluation assumption |  | TBA |
| Evaluation KPI |  | Predicted RSRP accuracy |
| Evaluation benchmark |  | UE actual measurement/expected RSRP at UE side |
| Preliminary evaluation results |  |  |
| High level potential specification impact |  | For UE-sided model: Measurement and prediction related core requirements; RSRP prediction accuracy requirements For NW-sided model: Depends on whether new metrics are needed |
| Feasibility issues, including complexity, and other aspects related to implementation |  | For UE-sided model: generalization issue, high complexity and additional assistant information For NW-sided model: TBA |
| Potential benefits |  | Measurement and gap Reduction |

For non-collocated frequency domain prediction, similar as non-collocated inter-cell spatial domain prediction, it is hard for UE side to do the prediction considering the high complexity due to the uncertainties regarding network deployment, channel condition, and detectable cells. And we see the following issues and difficulties for non-collocated frequency domain prediction, e.g.,

- The channel status changes for different frequencies are independent and with less relevance. It is hard to get the correlation between non-collocated frequencies.

- Some information of neighbor cells (e.g., relative location, transmission power) is not known to UE.

- The differences between site to site may be significant which makes the generalization hard to be guaranteed.

From our side, we think for a certain scenario e.g., the sites deployment is fixed, it is possible to find some solutions to enable the non-collocated frequency domain prediction e.g., through some assistant information. But such model can only be localized and the generalization issue is a big problem. Different from the co-located scenario, once the environment is changed, the relevance between two non-collocated frequency layers would have big difference and unpredictable, which makes the prediction performance hard to be guaranteed. So we understand NW-sided model is more proper for non-collocated frequency domain prediction.

Observation 3: There are several feasibility issues observed for non-collocated frequency domain prediction for UE sided model, e.g., generalization, high complexity for implementation and additional information.

Proposal 4: Not to consider UE-sided model for non-collocated frequency domain prediction for L3 beam and cell level measurement.

For NW-sided model, it is open to discuss, but most of the spec impacts are for RAN2 and the RAN4 impact depends on whether new metrics are defined. And for all the above measurement related use cases, it can be seen that they are deprioritized cases in Rel-20 AI mobility which is led by RAN2, and we think most likely RAN2 will also discuss and continue to lead these use cases in the upcoming meetings. So we understand the coordination with RAN2 are needed and it is better to wait for more RAN2 progress.

Observation 4: The measurement related use cases including L3 beam and cell level measurement prediction are leftovers from Rel-20 AI mobility which is led by RAN2 and most likely will also be discussed in RAN2.

Proposal 5: Wait for RAN2 progress for L3 beam level and cell level prediction to avoid overlapping and decide the leading group.

2.2 Dynamic adaptation of measurement procedure

For dynamic adaptation of measurement procedure, we see some example proposals in previous meeting, e.g., the measurement on some frequencies or cells can be relaxed based on AI/ML prediction, or the criteria for neighbor cell measurement triggering can be enhanced based on future prediction. The following analysis in table 3 are provided from our side.

Table 3: Analysis for use case c: dynamic adaptation of measurement procedure

| Use case c: (Dynamic adaptation of measurement procedure) |  | OPPO |
| --- | --- | --- |
| AI model input | Input in training | L3 beam/cell level measurement at historic time instance(s) until t2 for multiple frequency layers; Other possible information |
|  | Input in inference | L3 beam/cell level measurement at historic time instance(s) until t2 for multiple frequency layers; Other possible information |
| AI model output | Label in training (if applicable) | L3 beam/cell level measurement at t2+t for multiple frequency layers |
|  | Output in inference | Predicted L3 beam/cell level measurement at t2+t for multiple frequency layers; Predicted top-K frequency layers/cells; predicted time for cell change; probability of cell change at time t2+t; probability of cell change to frequency layer f |
| Assumption on training | Training type | Offline training |
|  | Label construction (if applicable) | L3 beam/cell measurement data for multiple frequency layers |
| model location for inference |  | UE-sided model |
| Collaboration/interaction between UE and NW |  | TBA |
| Evaluation methodology |  | System level simulation |
| Evaluation assumption |  | TBA |
| Evaluation KPI |  | Predicted RSRP accuracy; success rate of prediction |
| Evaluation benchmark |  | UE actual measurement; chosen frequencies/cells based on certain criteria; correct time/frequency for cell change |
| Preliminary evaluation results |  |  |
| High level potential specification impact |  | Corresponding core and accuracy requirements depending on the KPI |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Benchmark derivation and testability depending on selected KPI |
| Potential benefits |  | Measurement relaxation/reduction; Performance improvement for cell change |

For this use case, there may be multiple types of prediction output as proposed, including predicted L3 beam/cell level measurement, predicted top-K frequency layers/cells, predicted time for cell change and probability of cell change etc. Different types of prediction may be beneficial to measurement relaxation/reduction or performance improvement of cell change. It is important for groups to evaluate the performance gains for different prediction output and corresponding KPIs. When selecting the feasible methodology, UE implementation issues e.g., the model complexity and testability should be considered together with the performance gains.

In last meeting’s agreement, it is agreed to consider single domain prediction and no RSRP prediction is involved in dynamic adaptation of measurement procedure. But based on the above analysis, we understand RSRP prediction is important for most of the methodology and at least can be the intermediate or optional prediction output. So we suggest to revert the agreement on the involvement of RSRP prediction.

Proposal 6: Study and identify the potential model output and appropriate evaluation KPI for dynamic adaptation of measurement procedure.

Proposal 7: Do not preclude the RSRP prediction which can be at least intermediate or optional prediction results for dynamic adaptation of measurement procedure.

Proposal 8: When evaluating and selecting the feasible methodology (e.g., prediction output and evaluation KPI), performance gains together with the model complexity and testability should be considered.

2.3 Spatial domain RX beam sweeping reduction (L1/L3)

Among the above discussions, spatial domain prediction refers to Tx beam prediction which is included in 5G NR. Taking the following figure 5 as an example, the Tx beam prediction only focus on the Tx handling under same Rx beam (e.g., Case 1 or Case 2) which may reduce measurement efforts or network transmission overhead.

Figure 1: illustration of beam prediction

But from RAN4 perspective, for L1 and L3 measurement, Rx beam is also an important aspect since Rx beam sweeping is assumed for receiving different Tx beams which require additional measurement efforts and extend the measurement delay. So it is beneficial if some solutions can be found to predict the Rx beam or Tx-Rx beam pair (e.g., Case 3) to reduce beam sweeping and the measurement efforts on UE side. For this target, the use case of spatial domain Rx beam reduction is proposed, and the following details in table 4 are provided from our understanding.

Table 4: Analysis for use case d: spatial domain RX beam sweeping reduction (L1/L3)

| Use case d: (Spatial domain RX beam sweeping reduction (L1/L3)) |  | OPPO |
| --- | --- | --- |
| AI model input | Input in training | L1/L3 beam measurement with sub-set of Rx beams for frequency layer f1. The measurements include multiple cells and Tx beams |
|  | Input in inference | L1/L3 beam measurement with sub-set of Rx beams for frequency layer f1. |
| AI model output | Label in training (if applicable) | L1/L3 beam measurement with full set of Tx and Rx beams for frequency layer f1. |
|  | Output in inference | Predicted L1/L3 beam measurement with full set of Rx beams for frequency layer f1; predicted top-K Tx/Rx beams |
| Assumption on training | Training type | Offline training |
|  | Label construction (if applicable) | L1/L3 beam measurement data with full set of Tx and Rx beams |
| model location for inference |  | UE-sided model |
| Collaboration/interaction between UE and NW |  | No collaboration, NW configures the reference signals as legacy. |
| Evaluation methodology |  | System level simulation |
| Evaluation assumption |  | TBA |
| Evaluation KPI |  | RSRP prediction accuracy |
| Evaluation benchmark |  | UE actual measurement |
| Preliminary evaluation results |  |  |
| High level potential specification impact |  | UE capability on the reduced Rx beam sweeping; Measurement and prediction related core requirements; RSRP prediction accuracy requirements |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Complexity of implementation due to prediction cross beams, cells and time. |
| Potential benefits |  | Reduction of Rx beam sweeping factor |

Based on the above analysis, for this prediction approach, we think it cannot be a single domain prediction and at least the combination of time domain and spatial domain prediction can be considered for intra-frequency measurement. In addition, since Rx beam sweeping is UE implementation, and one Rx may receive multiple neighbor cells, the prediction results will include the measurements on multiple Tx beams and cells which may also lead high complexity of implementation.

Observation 5: The spatial domain Rx beam sweeping reduction includes the prediction cross multiple cells and multiple domains e.g., time and spatial domain.

On the other hand, we analyzed the model input and output for Rx beam can be the measurement results with certain Rx beams. But from prediction and reporting perspective, we think UE only need to report the predicted or selected Tx beams. It is up to UE implementation which Rx beam is used and no reporting is needed. The final impact on UE can be the capability of reduced Rx beam sweeping. From prediction methodology and performance perspective, we understand it is similar as Rel-19 AI/ML based BM and the evaluation method in 5G-A can be referred.

Proposal 9: Study the feasibility of spatial domain RX beam sweeping reduction (L1/L3) considering:

- appropriate evaluation KPI

- complexity due to prediction cross multiple cells and multiple domains

- difference with spatial domain prediction for Tx beams

- Conclusions

In this contribution, we discussed the 6G AI/ML use cases for RRM and got the following observations and proposals:

Observation 1: Compared to Rel-19 AI/ML based BM, no big difference is observed for intra-cell spatial domain prediction for L3 beam level measurement.

Observation 2: The benefits for non-collocated inter-cell spatial domain prediction are not clear while several feasibility issues are observed, e.g., generalization, high complexity.

Observation 3: There are several feasibility issues observed for non-collocated frequency domain prediction for UE sided model, e.g., generalization, high complexity for implementation and additional information.

Observation 4: The measurement related use cases including L3 beam and cell level measurement prediction are leftovers from Rel-20 AI mobility which is led by RAN2 and most likely will also be discussed in RAN2.

Observation 5: The spatial domain Rx beam sweeping reduction includes the prediction cross multiple cells and multiple domains e.g., time and spatial domain.

Proposal 1: RAN4 not to consider spatial domain prediction for L3 beam level measurement in FR1.

Proposal 2: Identify the difference between intra-cell spatial domain prediction for L3 beam level measurement and Rel-19 AI/ML based BM.

Proposal 3: Deprioritize the non-collocated inter-cell spatial domain prediction for L3 beam level prediction for Tx beams.

Proposal 4: Not to consider UE-sided model for non-collocated frequency domain prediction for L3 beam and cell level measurement.

Proposal 5: Wait for RAN2 progress for L3 beam level and cell level prediction to avoid overlapping and decide the leading group.

Proposal 6: Study and identify the potential model output and appropriate evaluation KPI for dynamic adaptation of measurement procedure.

Proposal 7: Do not preclude the RSRP prediction which can be at least intermediate or optional prediction results for dynamic adaptation of measurement procedure.

Proposal 8: When evaluating and selecting the feasible methodology (e.g., prediction output and evaluation KPI), performance gains together with the model complexity and testability should be considered.

Proposal 9: Study the feasibility of spatial domain RX beam sweeping reduction (L1/L3) considering:

- appropriate evaluation KPI

- complexity due to prediction cross multiple cells and multiple domains

- difference with spatial domain prediction for Tx beams

- References

RP-251881, New SID: Study on 6G Radio, RAN#108, June 2025

R4-2514637 WF on 6G AI, RAN4#116bis

R4-2522458 WF on [117][107] 6G AI, RAN4#117

R4-2522365 Topic Summary for [117][107] 6G AI, RAN4#117