# 3GPP TSG-RAN WG4 Meeting #118	R4-2601167

# Gothenburg, Sweden, Feb. 09 – 13, 2026

Agenda item:			8.8.1

Source:	Samsung

Title:	Discussion on AI/ML general aspects for 6GR

Document for:	Approval

# 1. Introduction

In RAN#108, RAN plenary approved a new RAN working group level SID on 6G Radio [RP-251881], which has been further revised into [1, RP-252912] in RAN#109. Specifically, AI/ML is the indispensable enabler for 6G RAN, for which at least AI/ML framework and new AI/ML use cases are highlighted as key aspects to be studied, with objectives provided as follows [1]:

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In RAN4#116bis, 6G AI/ML study scope and timeline has been approved [3], in which AI/ML framework (including RAN4 aspects for LCM procedures and other RAN4 specific issues) are identified as part of general aspects, and it is agreed that in Stage-1 (Q4’25-Q1’26), RAN4 will clarify the study scope for RAN4 AI/ML framework. In RAN4#117, further agreements have been achieved and captured in [5].

In this contribution, we would like to provide our further viewpoints on AI/ML general aspect.

# 2. Discussion

## 2.1 Post-deployment enhancement

In RAN4#117, the following WF is achieved for clarifying the differences between pre-deployment conformance testing and two post-deployment enhancement options (pre-activation functionality test and post-activation functionality testing based on performance monitoring) in terms of implementation timing, location, and agreed to further study these two post-deployment enhancement options in 6G research.

Issue 1-1: Post-deployment enhancement Agreement: - The following clarification is provided to align companies’ understanding on pre-deployment conformance and post-deployment enhancement options. Ways to guarantee AI/ML performance; When to perform; Where to perform; Comment. (1); Pre-deployment conformance test; Before cell-phone shipped into market; Testing lab for conformance testing; Same as existing conformance testing. (2); Post-deployment pre-activation functionality test (Option 1 in Rel-19 discussion); After cell-phone shipped into market, but before new AI/ML functionality activated; FFS in UE vendors’ lab or testing lab; Similar as product testing. (3); Post-deployment post-activation functionality testing [based on performance monitoring] (Similar to Option 2 in Rel-19 discussion); After new AI/ML functionality activated; FFS in-field practical network environment. - FFS the following options for post-deployment enhancement in 6G study: - Post-deployment pre-activation functionality test (Option 1 in Rel-19 discussion) - Post-deployment post-activation functionality testing based on performance monitoring (Option 2 in Rel-19 discussion)

Post-deployment pre-activation functionality test (Option 1 in Rel-19 discussion)

The Post-deployment pre-activation functionality test, referred to as Option 1 in Rel-19 discussions, is a targeted testing measure designed for AI/ML functionalities in cell-phones, conducted at the stage after the device has been shipped to the market but prior to the activation of any new AI/ML features. This test serves as a supplementary quality assurance layer that bridges the gap between pre-deployment conformance testing (completed before market shipment) and real-world usage, ensuring that the new AI/ML capabilities integrated into the device meet predefined requirements before end-users gain access to them.

The post-deployment pre-activation functionality test (Option 1 in Rel-19 discussion), if introduced, is used to ensure AI/ML functionality after product shipping but before activation, which is quite different from conformance test.

By verifying functionality at this post-shipment, pre-activation juncture, the test helps mitigate potential risks such as performance inconsistencies, compatibility issues, or user experience disruptions that might arise if untested AI/ML features were activated directly in the field, while also complementing pre-deployment conformance efforts to uphold overall product quality.

The post-deployment pre-activation functionality test (Option 1 in Rel-19 discussion) is important to mitigate potential risks if untested AI/ML features were activated directly in the field.

We expect that post-deployment pre-activation functionality test shall typically be carried out either in UE vendors’ dedicated labs or specialized testing facilities, leveraging controlled environments to simulate real-world scenarios without exposing end-users to unvalidated functionalities.

- If introduced, post-deployment pre-activation functionality test shall be allowed to be conducted in UE vendors’ lab or other testing lab.

Post-deployment post-activation functionality testing [based on performance monitoring] (Similar to Option 2 in Rel-19 discussion)

Post-deployment post-activation functionality testing [based on performance monitoring], similar to Option 2 in Rel-19 discussions, is a dynamic testing approach tailored for AI/ML functionalities in cell-phones, executed after the new AI/ML features have been activated for end-users and conducted in-field within practical network environments (with specific implementation details to be further defined, FFS). Unlike pre-activation testing conducted in controlled lab settings, this testing method leverages real-world usage scenarios and ongoing performance monitoring data to evaluate how AI/ML functionalities perform under actual operational conditions—including varying network signal strengths, user behavior patterns, and environmental factors that may not be fully replicable in controlled environments. By grounding the testing in live network environments, it enables the continuous validation of AI/ML functionality’s reliability, efficiency, and adaptability post-activation, as performance monitoring data provides insights into real-time behavior such as response times, accuracy, resource utilization, and compatibility with existing network infrastructure. This approach serves as a critical complement to pre-deployment and pre-activation testing efforts, allowing for the identification of performance gaps, edge cases, or degradation issues that may only emerge once the features are in active use, and supporting iterative optimization of AI/ML functionalities to align with real-world user needs and network conditions.

- Post-deployment post-activation functionality testing based on performance monitoring can be further discussed in 6G scope for the identified 6G use cases, as per-use case basis.

## 2.2 Model complexity

The meeting RAN4#117 agreed that RAN4 will further study AI model complexities by considering factors including the number of model parameters, computing complexity, size, inference latency, and generalization (with the study not intended to restrict model deployment) and will further define an example to differentiate "Simple" and "Complex" models based on specific metrics and corresponding threshold ranges.

Issue 1-2: Model complexity Agreement: - For model complexity, - RAN4 further study AI model complexities, by considering: number of model parameters, model computing complexity, model size, inference latency, and model generalization. - The complexity study is not intended to restrict model deployment. - FFS the following example to differentiate “Simple” and “Complex” models: "Simple model”; "Complex model". Num of model parameter; Low, e.g., <1M; High, e.g., 1M ~10M. Model computing complexity; Low, e.g., <1M FLOPs, or [1M FLOPs ~10M FLOPs]; High, e.g., [1M FLOPs ~10M FLOPs], or 10M FLOPs ~100M FLOPs. Model size; Small, e.g., <1MB; Large, e.g., 1MB ~10MB. [Inference latency]; Low; High. [Generalization]; Sufficient for simple tasks; High potential for complex tasks.

Defining model complexity boundaries for specific UE types, particularly eMBB handheld devices, is essential to ensure real-time inference feasibility. Given the resource constraints of handheld UEs—such as limited processing power, memory, and energy—models exceeding certain complexity thresholds (e.g., parameters >1M, FLOPs >10M, or size >1MB) may incur unsustainable inference latency, hindering real-time performance. The RAN4 study on AI model complexities, which evaluates parameters, computational load, size, latency, and generalization, provides a framework to establish these boundaries. Such boundaries could give follow-up AI/ML study and normative work a general baseline.

- For model complexity, RAN4 shall study the range of “complex model” complexity:

- Certain “complex model” range (e.g., >10M FLOPs and/or >10MB) shall be regarded as infeasible to be implemented as real-time inference (i.e., inference latency < 10ms) for 6G handheld UE type.

- Other complexity range for other UE types can be FFS with low priority.

## 2.3 AI/ML framework for RAN4-driven use cases

Based on Rel-18/19 relevant study and work items, the NR AI/ML framework for the air-interface is a compromise between practicality and forward compatibility. The main aspects of the NR AI/ML framework can be summarized with the following points in Table 1:

Table 1 AI/ML framework review of 5G-Adv AI/ML for air interface

|  | Rel-18/19 Discussion Points | Experience learnt for 6G |
| --- | --- | --- |
| AI/ML Framework and Life Cycle Management (LCM) | Data Collection (UE-sided): - Enabled through UE measurement - NW provide configuration and reference signal transmissions - Data collection framework was not complete in Rel-18/19. Data Collection (NW-sided): - Extension of the MDT framework. | Data collection work is mainly RAN2/SA related. |
|  | Applicability Report (UE-sided model) - UE reports its ML capabilities via capability reporting. - NW configurations, based on the reported capabilities, are used to enable inference operations. - UE may additionally report applicability/inapplicability of AI/ML functionalities in response to the NW’s indication. | Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Model management and model-ID-LCM (UE-sided model) - Model management is up to UE implementation. - NW controls the ML usage at the UE via functionality-based LCM. - Functionality-based LCM indirectly supports model selection, activation, deactivation, switching, and fallback Model management and model-ID-LCM (NW-sided model) is up to the NW implementation. | The necessity of model management depends on RAN4 discussion. Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Protection for NW-side implementation (UE-sided model) - NR framework leverages the concept of an Associated ID - Associated ID is to implicitly indicate NW-side additional conditions that may impact performance of a UE-side model. | Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Performance monitoring (UE-side model) - The monitoring outcome may assist NW decisions on configurations/functionalities switching including fallback to non-AI/ML approaches. - Two performance monitoring approaches: UE-assisted performance monitoring where performance monitoring metric is calculated at the UE and NW-side metric calculation considering ground-truth measurements report from UE as input. | Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Model training: - In the NR offline model training is assumed. Thus, there was no explicit specification support for on-device (online) model training and fine-tuning. | Need to reconsider by taking the solutions for RAN4 use case into account Model retuning and/or online training should also be considered. |
|  | Inter-vendor training collaboration (two-sided model): - Rel-19 identified three inter-vendor training collaboration options, namely: i) fully specified reference model ii) reference encoder parameters sharing-based approach iii.) encoder-related training dataset sharing-based approach. | No need to consider this if only one-sided model is considered for RAN4 use case. If two-sided model is used, Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Model Transfer: - In the NR model transfer across vendors, e.g., from NW to the UE, has not been supported. - It is assumed that the UE-part of the models are trained offline by UE-side vendors and delivered to the UEs in a transparent manner. | Work is mainly RAN2/SA related. Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Processing unit and timeline: - The 5G NR acknowledged AI/ML processing may be carried out by a separate hardware from non-AI/ML processes. - AI/ML processing unit (APU) is introduced to quantify the number of concurrently processed AI/ML-based CSI reports. - A separate CSI processing timeline is considered for AI/ML-based CSI reports. | Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Testing and Performance Requirements - Test cases, core and performance requirements, and generalization requirements are use-case specific - Requirements for LCM - Post-deployment testing (LCM based is prioritized). - Model level testing is not supported. | RAN4 centric work in 5G-A discussion. |

Based on Rel-18/19 experience, the LCM-relevant discussion went slowly, and it would be complicated and inefficiency to design LCM procedures for each studied use case. Therefore, to align with existing RAN1-led use cases (AI-BM, AI-CSI prediction and AI-positioning) and the new 6G use cases driven by different working groups, the unified LCM framework shall still be desired and targeted for.

By examining the Rel-18/19 outcome for LCM framework, RAN4 shall base its LCM study for AI/ML on the NR framework in TR 38.843, covering its terminology, UE/network data collection, applicability reporting, implicit-condition IDs, performance monitoring, APU timelines, and corresponding test/performance requirements.

- For RAN4 use case(s) to be identified in Rel-20, RAN4 should at least consider the following AI/ML framework aspects be applicable or not:

-	UE-side and NW-side data collection if needed

-	Applicability report

-	Associated ID to indicate additional conditions that may not be explicitly configured

-	Performance monitoring

-	Dedicated AI/ML processing unit (APU) and timeline

-	Testing and Performance Requirements

## 2.4 Principle for use cases selection and discussion

The following agreement on principle for AI/ML use case selection is achieved in RAN4#116bis.

Issue 3-1: Principle for AI/ML use cases selection Agreement: - Principles for AI/ML RAN4-driven use cases selection - RAN4 shall closely coordinate with other WG and strive not to duplicate the efforts - RAN4 should work with other WG to clarify the split between RAN4 driven AI/ML work and RAN1/2/3 driven work. - Coordinated discussions with RAN1/2/3 are required to avoid overlapping scope. - Typical RAN4-driven use case shall - ensure performance gain or gain from other aspects, by focusing on some RF/Demod/RRM topics - Prioritize the use case by taking into account: - use cases with gain over non-AI counterpart - gain may include performance gain or gain from other aspects, like overhead reduction. - trade-off between performance and complexity/overhead should also considered. - use cases with potential standard impact. - Note: the prioritization principle is the general principle and no intention to have implication on RAN4 6G AI work plan.

During RAN#110, discussion was held on down-selection of AI/ML use cases at 6G WG level, and the way forward (noted but not endorsed) was accordingly drafted by RAN1/2/3/4 chairs as below for information:

- The WG chairs will bring to the March RAN#111 plenary the AI/ML uses cases from their respective WGs - For each use case, ideally the WG chairs identify: - Use case and description. Identification of sub-cases as well. - Observations on benefits and/or gain (if available) /complexity/standardization effort required - Impacted working groups - work required by other WGs to complete the study - At RAN#111 Plenary to discuss the various use cases and attempt an initial prioritization

With the above joint suggestions from RAN WG chairs, it is suggested that RAN4 AI/ML group shall provide a list of RAN4 identified use cases under RAN4 discussion with priority, accordingly which can be used to be provided to RAN#111.

- RAN4 AI/ML group shall provide a list of RAN4-identified use cases under RAN4 discussion with priority, accordingly which can be used to be provided to RAN#111.

Furthermore, the following basic information template can be used to aligned companies’ view on certain use case.

- For each use case within RAN4-identified use case list, the following information template should be used to provide necessary information to RAN#111.

| RAN4-identified Use case |  | Name, and identification of sub-cases as well |
| --- | --- | --- |
| Supported companies |  | Company-A, B, … |
| Use case description | AI model input |  |
|  | AI model output |  |
|  | Label |  |
|  | Training type | e.g., offline training, online training, offline training with online refining |
|  | Model location for inference | e.g., NW-sided, UE-sided, two-sided model |
|  | Collaboration/interaction between UE and NW | e.g., network indication, UE capability reporting, etc. |
| Observations on benefits and/or gain |  |  |
| Observations on complexity |  |  |
| Standardization effort required |  | e.g., System-level simulation, etc. |
| Potential RAN4 spec impact |  |  |
| Impacted other working groups |  | If yes, elaborate work required by other WGs to complete the study |

# 3. Conclusion

In this contribution, we have provided our viewpoints on the general aspects, with the following observations and proposals achieved:

Post-deployment enhancement

- The post-deployment pre-activation functionality test (Option 1 in Rel-19 discussion), if introduced, is used to ensure AI/ML functionality after product shipping but before activation, which is quite different from conformance test.

- The post-deployment pre-activation functionality test (Option 1 in Rel-19 discussion) is important to mitigate potential risks if untested AI/ML features were activated directly in the field.

- If introduced, post-deployment pre-activation functionality test shall be allowed to be conducted in UE vendors’ lab or other testing lab.

- Post-deployment post-activation functionality testing based on performance monitoring can be further discussed in 6G scope for the identified 6G use cases, as per-use case basis.

Model complexity

- For model complexity, RAN4 shall study the range of “complex model” complexity:

- Certain “complex model” range (e.g., >10M FLOPs and/or >10MB) shall be regarded as infeasible to be implemented as real-time inference (i.e., inference latency < 10ms) for 6G handheld UE type.

- Other complexity range for other UE types can be FFS with low priority.

AI/ML framework for RAN4-driven use cases

- For RAN4 use case(s) to be identified in Rel-20, RAN4 should at least consider the following AI/ML framework aspects be applicable or not:

-	UE-side and NW-side data collection if needed

-	Applicability report

-	Associated ID to indicate additional conditions that may not be explicitly configured

-	Performance monitoring

-	Dedicated AI/ML processing unit (APU) and timeline

-	Testing and Performance Requirements

Principle for use cases selection and discussion

- RAN4 AI/ML group shall provide a list of RAN4-identified use cases under RAN4 discussion with priority, accordingly which can be used to be provided to RAN#111.

- For each use case within RAN4-identified use case list, the following information template should be used to provide necessary information to RAN#111.

| RAN4-identified Use case |  | Name, and identification of sub-cases as well |
| --- | --- | --- |
| Supported companies |  | Company-A, B, … |
| Use case description | AI model input |  |
|  | AI model output |  |
|  | Label |  |
|  | Training type | e.g., offline training, online training, offline training with online refining |
|  | Model location for inference | e.g., NW-sided, UE-sided, two-sided model |
|  | Collaboration/interaction between UE and NW | e.g., network indication, UE capability reporting, etc. |
| Observations on benefits and/or gain |  |  |
| Observations on complexity |  |  |
| Standardization effort required |  | e.g., System-level simulation, etc. |
| Potential RAN4 spec impact |  |  |
| Impacted other working groups |  | If yes, elaborate work required by other WGs to complete the study |

# 4. Reference

[1] RP-252912 (revision from RP-251881), “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone, RAN#109.

[2] R4-2514636,	“Topic Summary for [116bis][107] 6G AI”, Samsung, RAN4#116bis.

[3] R4-2514637,	“WF on 6G AI”, Samsung, RAN4#116bis.

[4] R4-2522365, “Topic Summary for [117][107] 6G AI”, Samsung, RAN4#117.

[5] R4-2522458, “WF on [117][107] 6G AI”, Samsung, RAN4#117.

[6] RP‑253864, “Way forward on AI/ML use cases RAN 110”, 3GPP RAN WG1 Chair, RAN WG2 Chair, RAN WG3 Chair, RAN WG4 Chair, RAN#110.