# 3GPP TSG-RAN WG4 Meeting #118                                 R4-2601298

# Gothenburg, Sweden, Feb. 09-13, 2026

Source:	ZTE Corporation, Sanechips

Title:	Study of RAN4 impacts for RRM measurement prediction

Agenda Item:	7.12.3

Document for:	Approval

- Introduction

In RAN plenary #109 meeting, a revised WID [1] for Rel-20 was approved to study the AI/ML for mobility in NR. The objectives for RAN4 of this WID are listed as follows.

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

In this document, we will provide some views on requirements impacts based on RRM measurement prediction from the RAN4’s perspective.

- Discussion

On RRM measurement prediction, we provide our views on RRM impact on RRM measurement prediction use-case in the follow [2].

Issue 2-1-1: What kind of core requirements to be defined Way forward: - Further discuss whether the follow requirements need to be defined: - Measurement delay/period requirements considering: - The impact on CSSF, gap sharing - The impact due to MRRT - Prediction reporting delay requirements - Further discuss the components, e.g., - Measurement delay/period requirements, - Inference delay - Uncertainty for reporting - Measurement and scheduling restrictions in OW/PW - OW/PW length - SCell related requirements - UE mandatory capability on the number of cells and frequency layers for actual measurements and inference-based measurements - Inference delay

For measurement delay/period requirements, we should discuss sperately for temproal domian prediction and frequency domian prediction. Compared with non-AI measurement delay requirement, some actual measurement can be skipping.

With the help of AI tools, it is not necessary to perform actual measurements at the time when UE measurements were previously required. For intra-frequency temporal domain prediction caseB, it can be observed that UE will skip some measurement occasions, which means that only subset of measurement occasions are measured so that SMTC cannot be directly used in the measurement delay requirements as legacy. For frequency-domain prediction, there is no need to measure the MO on another frequency layer, which means the use of GAP is eliminated. Meanwhile, for inter-frequency inter-cell (frequency domain), not all configured MOs need to be measurement, just need to consider the actually measurement frequency layers when we define measurement delay requirements. Thus, the CSSF assumptions need to be updated.

Observation 1: Compared with legacy measurement delay requirements, number of actual measured samples and frequency layers are reduced, which will impact measurement requirements.

Proposal 1: RAN4 should study whether/how to update measurement delay requirements for AI/ML mobility, at least including:

      - The impact on CSSF for frequency domain prediction

      - The impact due to MRRT for temproal domain caseB prediction

In addition, when we discuss the measurement delay, in addition MRRT, OW/PW length should also be considered for temporal domain. With the assistant of AI, actual measurements just be excuted in OW and the measurement in PW can be predicted without actual measurement. If only know MRRT, it’s hard to obtain the information that how many samples will be skipped. Therefore, OW length or PW length is needed when define delay requirement.

Proposal 2: In addition MRRT, OW/PW length should also be considered for temporal domain when we discuss the measurement delay.

For prediction reporting delay requirements, we can use the agreement made in Rel-19 AI BM as starting point. Similar as AI BM, prediction report delay requirement should be defined for AI mobility, including aperiodic, semi-persistent and periodic report. Meanwhile, measurement delay+inference delay+Uncertainty for reporting should be discussed.

Agreement in RAN4#116 Issue 2-2: Prediction report delay Agreement: For aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay -The reporting delay includes the inference delay For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+time for the first available reporting occasion

Figure 1: illustration of prediction reporting delay [3]

Observation 2: for Rel-19 AI BM, prediction report delay requirements have been defined for aperiodic, semi-persistent and periodic report. Meanwhile, measurement delay, inference delay and uncertainty for reporting have been considered.

- Proposal 3: Similar as AI BM, prediction reporting delay requirements should be defined for AI mobility. And the agreement in Rel-19 AI BM can be used as starting point, including measurement delay, inference delay and uncertainty for reporting.

Issue 2-2-2: Prediction accuracy requirements for measurement prediction Way forward: - Further discuss the following issues based on the proposals in [1]: - Issue 1: Whether the prediction accuracy has the same or different accuracy requirements as non-AI/ML RRM measurement. - Issue 2: Whether to define separate prediction accuracy for different sub-use case/scenario. - Issue 3: Whether the following parameters need to be the side condition of the accuracy requirements. - OW/PW length - MRRT - Skipping pattern - UE speed - SINR - L1 filtering approach and sample number - Channel correlation coefficient

- For Prediction accuracy requirements, whether the prediction accuracy has the same or different accuracy requirements as non-AI/ML RRM measurement. We can discuss this issue based on simulation results. From our view, if possible, the same accuracy requirements should be defined for measurement prediction as non-AI/ML RRM measurement. With the help of AI models, we hope to maintain the performance of legacy while reducing the number of measurement samples. For this purpose, the definition of prediction accuracy should be consistent with the actual measured accuracy.

- Proposal 4: Prefer to define the same accuracy requirements for measurement prediction as non-AI/ML RRM measurement

Based on the RAN2’s agreement, we should define the prediction accuray of the cell-level measurement prediction, and RSRP difference to the actual measurement should be defined. Meanwhile, measurement reduction rate is also considered as one KPI in RAN2 and it can be depotitized compared to measurement prediction accuracy.

RAN2#126 => Definition of measurement reduction for intra-frequency scenario is defined as:(20/20) Measurement reduction rate in temporal domain (MRRT) assuming same length of measurement time instances: MRRT= skipped measurement time instances / total measurement time instances Measurement reduction rate in spatial domain (MRRS): MRRS = skipped beams to be measured/ total beams to be measured RAN2#130 The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered izln WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction)

RAN2 has defined measurement reduction for intra-frequency scenario, including MRRT and MRRS. Measurement reduction rate in temporal domain (MRRT) is the ratio between skipped measurement time instances and total measurement time instances, and measurement reduction rate in spatial domain (MRRS) is tha ratio between skipped beams to be measured and total beams to be measured. In our understanding, these two indicators can be used to evaluate the benefits brought by AI models in mobilioty.

According to the WID, spatial domain is not considered in Rel-20, and RAN4 only need to study the impacts of MRRT. As mentioned by other company before, only know the information of MRRT is useless because different skipping patterns of the same MRRT have different impacts on the prediction accuracy. In SI phase, even RAN2 didn’t consider measurement error in simulation, the trend of some configurations on prediction accuracy can be as a reference for RAN4. During RAN2’s discussion, they found that the prediction accuracy is affected by OW length, MRRT, and UE speed for intra-frequency temporal domain case B. Specifically, higher accuracy can be achieved with a longer OW, lower MRRT, and lower UE speed.

Observation 3: Regarding temporal domain measurement prediction B, even with the same MRRT, skipping pattern (e.g. [400ms,400ms] or [600ms,600ms]) can also impact the prediction accuracy.

Proposal 5: Regarding temporal domain measurement prediction B, study the impact of measurement skipping pattern on prediction accuracy for the same MRRT.

In R19, RAN4 has an agreement that consider the impacts on prediction accuracy due to OW, PW, MRRT and UE speed and so on. When we discuss the impact on RRM requirements due to the length of OW and/or PW, several factors should be considered, such as L3 filtering method, prediction by L1/L3-RSRP, different patterns for prediction window and observation window, and so on. For L3 filtering method, sliding and non-sliding is different for the definiation of requirements. If sliding method is used, the prediction result can be obtained by just one additional sample, however, more than one new samples are needed if non-sliding method is used. Similar situations can also occur in different sub-use cases. For prediction based on L1-RSRP (sub-use case1 and 3), the required observation window length and prediction window length are different from those predicted by L3-RSRP(sub-use case2). In addition, there are differences in the lengths of OW and PW for the two different patterens proposed for RAN2 in temporal domain measurement prediction case B.

Intra-frequency temporal domain case B-To be discussed in Phase 2: In case B, measurement results in prediction window are predicted by historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option) after measurement result(s) in previous prediction window is/are skipped. Example 1 of intra-frequency temporal domain case B Example 2 of intra-frequency temporal domain case B

For intra-frequency temporal domain case B, the prediction accuracy is affected by OW length, MRRT, and UE speed. Specifically, higher accuracy can be achieved with a longer OW, lower MRRT, and lower UE speed. Therefore, how to study the mpacts on prediction accuracy due to OW, PW, MRRT and UE speed should be considered due to there are numerous possible scenarios/combinations. From our side, there are many configuration and the combination of various configurations can create a large number of possible scenarios. RAN4 needs to discuss whether/how to make down selection on these scenarios to analyse the impacts on prediction accuracy.

Observation 4: For intra-frequency temporal domain case B, the prediction accuracy is affected by OW length, MRRT, and UE speed. Specifically, higher accuracy can be achieved with a longer OW, lower MRRT, and lower UE speed.

Proposal 6: For intra-frequency temporal domain case B, RAN4 needs to discuss whether/how to make down selection on numerous scenarios to analyse the impacts on prediction accuracy.

As to the LCM, RAN4 should LCM-related core requirements to support the LCM functionality management , if any. In traditional discussions, Life Cycle Management (LCM) typically encompasses several key stages during the measurement process as follow:

- Measurement Configuration: The network (gNB/eNB) sends measurement configuration signaling to the UE, specifying how measurements should be performed.

- Measurement Activation: The network issues an activation command, or certain trigger conditions are met, prompting the UE to begin executing the measurement task.

- Measurement Execution: The UE collects measurement data periodically or on demand as instructed.

- Measurement Reporting: According to the configured criteria, such as periodic intervals, event triggers, or threshold crossings, the UE reports the measurement results back to the network.

- Measurement Deactivation/Release: The network sends a deactivation command or releases the measurement context, which ends the measurement process

Acccording to the WID, RAN2 will discuss and specify signalling and protocol aspects to enable LCM functionality management for UE sided model and NW sided model for RRM measurement prediction. Meanwhile, the similar issus on LCM framework is also discussed in RAN4 under Rel-19 AI/ML for NR air interface WI. Therefore, we think it is premature to discuss the LCM in RAN4 AI Mob, and perfer to wait more progress made by RAN2 or AI/ML for NR air interface WI.

Proposal 7: RAN2 will discuss and specify signalling and protocol aspects to enable LCM functionality management in Rel-20, it is premature to discuss the LCM-related core requirements in RAN4 AI Mobility.

- Conclusion

In this contribution, we have the following observations and proposals for the AI/ML monility:

Observation 1: Compared with legacy measurement delay requirements, number of actual measured samples and frequency layers are reduced, which will impact measurement requirements.

Proposal 1: RAN4 should study whether/how to update measurement delay requirements for AI/ML mobility, at least including:

      - The impact on CSSF for frequency domain prediction

      - The impact due to MRRT for temproal domain caseB prediction

Observation 2: for Rel-19 AI BM, prediction report delay requirements have been defined for aperiodic, semi-persistent and periodic report. Meanwhile, measurement delay, inference delay and uncertainty for reporting have been considered.

Proposal 3: Similar as AI BM, prediction reporting delay requirements should be defined for AI mobility. And the agreement in Rel-19 AI BM can be used as starting point, including measurement delay, inference delay and uncertainty for reporting.

- Proposal 4: Prefer to define the same accuracy requirements for measurement prediction as non-AI/ML RRM measurement.

Observation 3: Regarding temporal domain measurement prediction B, even with the same MRRT, skipping pattern (e.g. [400ms,400ms] or [600ms,600ms]) can also impact the prediction accuracy.

Proposal 5: Regarding temporal domain measurement prediction B, study the impact of measurement skipping pattern on prediction accuracy for the same MRRT.

Proposal 6: For intra-frequency temporal domain case B, RAN4 needs to discuss whether/how to make down selection on numerous scenarios to analyse the impacts on prediction accuracy.

Proposal 7: RAN2 will discuss and specify signalling and protocol aspects to enable LCM functionality management in Rel-20, it is premature to discuss the LCM-related core requirements in RAN4 AI Mobility.

- Reference

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, OPPO, Interdigital

- R4-2522668, WF on NR_AIML_Mob_RRM, OPPO

- R4-2520846, Topic summary for [117][221] NR_AIML_Mob_RRM, Moderator (OPPO)