3GPP TSG-RAN WG4 Meeting # 118                                    R4-2601302

Gothenburg, Sweden, Feb. 09-13, 2026

Agenda item:	7.13.3

Source: 	ZTE Corporation, Saneships

Title: 	Discussion on SCell activation at SpCell LTM cell switch

Document for:	Approval

Introduction

In the RAN #109 meeting, the WID on several enhancements for NR mobility was approval. There are two objectives need to discuss in Rel-20, one is LTM SCell activation improvements for further reducing interruption delays, and the other is Dynamic L1 measurement and reporting configuration change. This meeting is the first time for this topic, and we need to provide the initial views on LTM SCell activation improvements. For Dynamic L1 measurement and reporting configuration change, it will be triggered by RAN2’s LS according to schedule [1].

The objectives are as the following - LTM SCell activation improvements for further reducing interruption delays - Specify the necessary configuration and UE procedures for NW triggering of LTM SCell activation as part of the SpCell LTM cell switch [RAN2, RAN3]. - Evaluate and specify as necessary the needed RAN4 requirements for this [RAN4]. NOTE 1: Reuse the candidate cell LTM measurement and reporting procedures where possible for the SCell activation improvements NOTE 2: Early CSI acquisition procedure for SCell is not in the scope. NOTE 3: This objective is not applicable to non-co-located CA and is not applicable to C-LTM NOTE 4: This objective is applicable to MCG and SCG, and is applicable to both intra-CU and inter-CU cases. NOTE 5: The objective is only applicable to the case of SCell activation within the same cell group from which the LTM cell switch command MAC CE is received. - Dynamic L1 measurement and reporting configuration change - Specify the necessary configuration and procedures for MAC CE based configuration selection among prior provided RRC configurations of UE L1 measurement and reporting LTM configuration [RAN2] NOTE 6: This sub-objective is applicable to C-LTM as well. NOTE 7: This objective is limited to the configuration related to L1 measurement and reporting configuration and strives to avoid RAN1 impact (including reporting) NOTE 8: RAN1/RAN3/RAN4 involvement is based on LS if necessary

In this contribution, we would like to provide our initial views on LTM SCell activation as part of the SpCell LTM cell switch.

Discussion

According the work item, other work groups will specify the necessary configuration and UE procedures for NW triggering of LTM SCell activation as part of the SpCell LTM cell switch, and RAN4 should evaluate and specify as necessary the needed RRM requirements for this.

From our understanding, the procedure for NW triggering of LTM SCell activation as part of the SpCell LTM cell switch is more likely the procedure for direct SCell Activation at Handover in clause 8.3.5, which is for handover with one SCell for which the parameter sCellState is set to activated, and the difference between them is that one is based on L3 handover, and the other is based on L1 cell switch. Therefore, we can use the requirements for direct SCell activation at handover in clause 8.3.5 as baseline and further discuss which component can be executed earlier or can equal to zero under side conditions.

Proposal 1: RAN4 should define delay requirement for SCell activation at SpCell LTM cell switch.

Proposal 2: For SCell activation at SpCell LTM cell switch, the requirements for direct SCell activation at handover in clause 8.3.5 as baseline and further discuss which component can be executed earlier or can equal to zero under side conditions.

First, we should consider the known cell condition for LTM SCell activation. In legacy, SCell activation is based on L3 measurement and we can observed from the known SCell condition as blew:

In existing known SCell condition, there is a sub-condition that the UE has sent a valid measurement report for the SCell being activated in FR1 or the UE has sent a valid L3-RSRP measurement report with SSB index in FR2.

Clause 8.3.5: The SCell in FR1 is known provided the following conditions are met for the SCell: - During the last 5 seconds before the reception of the direct SCell configuration command: - the UE has sent a valid measurement report for the SCell being directly activated, and - the SSB measured remains detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, - the SSB measured during the period equal to [5] seconds also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise, the SCell is unknown. The SCell in FR2 is known provided it meets the corresponding conditions as defined in clause 8.3.2. Otherwise, the SCell is unknown. clause 8.3.2: For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index, and - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes. Otherwise, the first SCell in FR2 band is unknown.

For FR1, known SCell means the UE has sent a valid measurement report for the SCell being directly activated during the last 5 seconds before the reception of the direct SCell configuration command. A valid measurement report here can include L1 measurement report and L3 measurement report. So, we have no strong view whether to update the wording to make it more clear or not in FR1. However, for FR2, not only valid L3-RSRP measurement report with SSB index, but also valid L1-RSRP measurement report should be considered due to LTM. Therefore, the known SCell condition should be updated to consider L1-RSRP measurement report.

Proposal 3: The known SCell condition should be updated to consider both L1 measurement report and L3 measurement report.

For SCell activation requirements, Tactivation_time is different for known and unknown SCell in FR1 and FR2. We have organized the activation delays for FR1 and FR2, known cells, and unknown cells in tables below. Taking FR2-1 as an example, if the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band, we can observe from the table that unknown SCell needs more time to excute cell search, measurement and report.

Table 1-1. Tactivation_time for known and unknown SCell in FR2

(no active serving cell on that FR2 band)

| Scenario# |  | Tactivation_time |
| --- | --- | --- |
| Known SCell | semi-persistent CSI-RS is used for CSI reporting | 3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) |
|  | periodic CSI-RS is used for CSI reporting | max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ) |
| Unknown SCell | semi-persistent CSI-RS is used for CSI reporting | 6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) |
|  | periodic CSI-RS is used for CSI reporting | 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)} |

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, Tactivation_time is defined as below.

Table 1-2. Tactivation_time for known and unknown SCell in FR2

(at least one active serving cell on that FR2 band)

| Scenario# | Tactivation_time |
| --- | --- |
| - The UE is provided with SMTC for the target SCell, and - The SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in clause 3.6.3, and - The parameter ssb-PositionsInBurst is same for the serving cell(s) and the Scell, and - SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell. | TFirstSSB+ 5ms |
| if the UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell | Tactivation_time is 3 ms |

For FR1, if valid EMR be considered, activation delay for unknown SCell with valid EMR reporting would be reduced as well.

Table 2. Tactivation_time for known and unknown SCell in FR1

| Scenario# |  |  | Tactivation_time |
| --- | --- | --- | --- |
| Known SCell | measurement period<2400ms |  | - TFirstSSB+ 5ms |
|  | measurement period not smaller than 2400ms |  | - TFirstSSB_MAX + Trs + 5ms |
| Unknown SCell | - with indicated ‘ssb-PositionInBurst’ | intra-band contiguous case | - TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms, |
|  |  | intra-band non-contiguous case and inter-band case | - TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms |
|  | without indicated ‘ssb-PositionInBurst’ | intra-band contiguous case | - Tactivation_time is 3 ms |
|  |  | intra-band non-contiguous case and inter-band case | - 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. |

Fig1. Illustration of unknown SCell activation delay in FR2

Observation 1: Tactivation_time for unknown SCell is longer than known SCell due to measurement and report.

In Rel-18 and Rel-19 mobility, we haven’t considered the unknown cell when defined the cell switch delay requirement. For LTM cell switch based on L1 measurement or L3 measurement, the cell should be known when cell switch command is received. Otherwise, UE may still need to perform cell search, which cause quite long delay/interruption during the cell switch, and the potential benefit introduced by LTM is wasted. Considering L3 measurement based HO/PSCell_change already support the unknown case, as the optimization to legacy L3 HO/PSCell_change, it is reasonable not to consider the unknown case for LTM cell switch at least in RAN4. In Rel-20, when we discuss how to reduce SCell activation delay，we should confirm whether to consider unknown case or not. From our view, it’s reasonable to reuse the agreement for PCell/PSCell switch and only consider known cell case.

Clause 6.3.1.1 for LTM PCell cell switch The requirements in this section apply when - the target cell is known, and - the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, or - the target cell is an FR1 cell, and the UE has reported L3-RSRP measurement result with SSB index associated to the target TCI state within 5 seconds before receiving the LTM cell switch command, and SNR of the SSB associated to the target TCI state ≥ -3dB, or - the target cell is an FR1 cell, and the UE is configured with event-triggered L1 measurement reporting and report on leave, and the UE has sent an event-triggered L1 measurement report with RS associated to the target TCI state and has not sent report on leave for the same RS before receiving the cell switch command. - PL-RS is QCLed with RS of the TCI state indicated by LTM cell switch command, and - For CFRA cell switch, RS of the TCI state indicated by LTM cell switch command is QCLed with the SSB indicated for CFRA. Clause 8.20.1 for LTM PSCell Cell Switch: The requirements in this section apply when - the target cell is known, and - the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, or - the target cell is an FR1 cell, and the UE has reported L3-RSRP measurement result with SSB index associated to the target TCI state within 5 seconds before receiving the LTM TCI state activation command, and SNR of the SSB associated to the target TCI state ≥ -3dB, or - the target cell is an FR1 cell, and the UE is configured with event-triggered L1 measurement reporting and report on leave, and the UE has sent an event-triggered L1 measurement report with RS associated to the target TCI state and has not sent report on leave for the same RS before receiving the cell switch command. - PL-RS is QCLed with RS of the TCI state indicated by LTM cell switch command, and - For CFRA cell switch, RS of the TCI state indicated by LTM cell switch command is QCLed with the SSB indicated for CFRA.

Proposal 4: Similar as previous releases, prefer not to consider unknown SCell case.

Except reducing delay through excuting L1 measurement earlier, early DL sync also can be applied. In Rel-18, if UE supports eraly TCI state activation, T_SSB and TRS_processing can be set zero under some side conditions, which make the cell switch delay and interruption shorten. The same principle can be reused for SCell activation. We can observed that UE need to do TCI activation after L1 measurement report during the legacy procedure, if TCI activation can be done earlier and timing can be maintained, the component of TCI activation during SCell activation can removed, which make the SCell activation delay shorter. Therefore, we prefer to consider early DL sync for LTM SCell activation.

Proposal 5: To reduce interruption delay, prefer to consider early DL sync for LTM SCell activation.

Conclusions

In this paper, we have some discussion on SCell activation at SpCell LTM cell switch. And we have the following initial proposals:

Proposal 1: RAN4 should define delay requirement for SCell activation at SpCell LTM cell switch.

Proposal 2: For SCell activation at SpCell LTM cell switch, the requirements for direct SCell activation at handover in clause 8.3.5 as baseline and further discuss which component can be executed earlier or can equal to zero under side conditions.

Proposal 3: The known SCell condition should be updated to consider both L1 measurement report and L3 measurement report.

Observation 1: Tactivation_time for unknown SCell is longer than known SCell due to measurement and report.

Proposal 4: Similar as previous releases, prefer not to consider unknown SCell case.

Proposal 5: To reduce interruption delay, prefer to consider early DL sync for LTM SCell activation.

References

- RP-252113, Revised Work Item: NR mobility enhancements Phase 5, Apple Inc