TSG-RAN Working Group 4 (Radio) meeting #118	  R4-2601381

Gothenburg, Sweden, 9 – 13 February 2026

Source:	Ericsson

Title:	System parameters – waveform: on UE Tx assumptions and the reply LS to RAN1

Agenda item:	8.2.1

Document for:	Approval

# Background

In this contribution we show measurement results with and without DPD relevant for RAN4 studies and consider the PA model(s) to be used for waveform evaluations in RAN1.

For the PA model the WG [1] lists the following

- Staged development of the PA model used for waveform evaluation.

  - For RAN1: Provide a model for timely waveform comparison.

    - Targeted for RAN4#118 meeting

  - For RAN4: Continue internal development of PA models, if needed, with more realistic considerations for RF requirements evaluation.

- Prioritize PA model(s) for 7 GHz, PC2/PC3 for waveform evaluation.

  - Consider models like the Generalized Memory Polynomial (GMP)

  - Other options not precluded

- Develop PA models covering different frequency ranges, power classes if single PA model is not accurate enough for all evaluation scenarios.

- Agree on calibration conditions and applicable requirements for the PA model (e.g., achieved ACLR for a reference waveform at a specific MPR) to ensure fair comparisons.

Besides the measurement results, we propose an alternative LS to RAN1 with recommendations on Tx modelling in case RAN4 cannot agree on a specific PA model. L1 improvements for 6 GR should be demonstrated for many types of implementations.

# Transmitter characteristics

## Tx assumptions for the study item

That the assumed IQ-imbalance rejection and LO leakage requirements of the modulation and frequency up-conversion of NR should be updated to reflect state-of-the art design is a given, an update of the assumed CIM3 and CIM5 performance might need further discussion.

In relation to PA modelling, a cascaded DPD and PA architecture could be a starting point for a Tx model. The DPD can of course be used for different purposes and is also dependent on the model assumed for the pre-distorter and the (adaptive) algorithms. Allegedly, many state-of-the-art UE designs use DPD for HOM. Obviously, a specific PA model and thus a specific UE design cannot be assumed for specifying requirements during the work-item phase, but more details e.g. on memory polynomials could be assumed for the 6G SI.

Observation 1: for RAN4, a specific PA model cannot be assumed for specifying requirements during the work-item phase, but more details on e.g. memory polynomials could be assumed for the 6G SI.

Assumptions on APT could also be considered for capturing behaviour at different input levels (supply voltages) when relevant for the study or later during specification of MPR. This to make the specified power back-off, if any, more relevant for operations in the field (in whichever direction 6GR MPR allowance would take as compared to NR).

## Mid-band measurements with and without DPD

In what follows we show measurements on a cascaded proprietary DPD with a n77 PA integrated with a band filter and antenna switches. The measurement set-up is as follows:

- uplink signal: CP-OFDM, SCS = 30kHz, 14 OFDM symbols, two slots

- modulation order: 64 QAM

- bandwidth: 100 MHz

- APT window length: one OFDM symbol

- Carrier frequency: 3.7 GHz

Figure 1 shows the measured AM-AM characteristics: the purple points without DPD, the orange without DPD but with APT, the green and the blue with different PD algorithms and memory polynomial (MP). The output power is measured after the n77 band filter and antenna switch (hence including the insertion loss).

Figure 1: AM-AM characteristics without DPD (orange with APT) and with DPD (e.g. green and blue).

Figure 2 shows the corresponding AM-PM with the same legends. The phase error much larger without DPD. The measured EVM without DPD is 4.9% with a peak input power of about 5 dBm, the EVM is reduced to 1.3% per APT window with DPD.

Figure 2: AM-PM characteristics without DPD (orange with APT) and with DPD (e.g. green and blue).

The impact of PA load mismatch is not considered in the results.

The resulting ACLR is shown in Figure 3 measured for peak input power up to 5 dBm; the purple curve is without DPD and APT. Using DPD the ACLR exceeds 40 dBc for 64QAM CP-OFDM according to the measurements. This aspect could possibly be reflected in future “calibration conditions” for RAN4 studies and specification of requirements.

The results also demonstrate clearly the memory effects for wider bandwidths. These are less pronounced, but obviously not absent, for smaller bandwidths. The observations on the effect of APT are less conclusive from these initial measurements.

Figure3: ACLR without DPD (purple, orange with APT) and with DPD (e.g. green and blue).

The AM-PM results for 20 MHz channel bandwidth are shown in Figure 4 for comparison. The phase error and EVM lower: 3.3% without DPD with a peak input power of about 5 dBm, the EVM is reduced to about 0.6% per APT window with DPD.

Figure 4: AM-PM characteristics without DPD (orange with APT) and with DPD (e.g. green and blue) for a 20 MHz channel bandwidth.

The results above display a large variation of performance depending on the algorithm and pre-requisites used for the single PA measured, the results also depend on the model.

We observe from the measurement that inclusion of DPD in the Tx assumptions significantly alters the transmitter characteristics and would have a profound effect on e.g. studies of DPoD relevant for consideration of expected performance in the field:

Proposal 1: Tx assumptions with a cascaded DPD and PA is relevant for state-of-the-art design and should be considered in studies of e.g. DPoD performance and any corresponding UE requirements with increased EVM.

## Measurements above 7 GHz (9-10 GHz)

Next we consider results in the 7-15 GHz range obtained using a GaN PA, a different technology to that measured in band n77. We consider the GaN technology in a multi-stage PA to get sufficient power capability and bandwidth in the cmWave band range and yet with good linearity.

We also consider low-complexity DPD architectures to with reduced computational complexity and power consumption with a view to increase efficiency for HOM. The algorithm is based on iterative learning control avoiding calculation of DPD in each step and matrix inversion.

The measurements were carried out for

- uplink signal: CP-OFDM, SCS = 30kHz, 14 OFDM symbols, two slots

- modulation order: 64 QAM

- bandwidth: 100 MHz

- Carrier frequency: 9-10 GHz

with a peak envelope input power is 17 dBm for the results shown.

The AM-AM characteristics is shown in Figure 5 and the AM-PM results in Figure 6.

Figure 5: AM-AM characteristics with and without DPD (orange) and with DPD for 100 MHz channel bandwidth.

Figure 6: AM-PM characteristics without DPD (orange) and with DPD for a 100 MHz channel bandwidth.

The measured EVM is 7.9% without DPD and is reduced to 4.9% with DPD. We remark that these results are in no way optimal but shows the improvement with DPD.

The ACLR is shown in Figure 7. We note that the ACLR is > 30 dBc below the carrier and > 25 dBc above, the asymmetry due to memory effects. The in-channel performance is also affected.

Figure 7: ACLR without DPD (orange) and with DPD.

We observe that

Observation 2: improvements of the EVM for 64QAM with DPD are also observed for 7 GHz (different PA technology to that used for the mid-band results). GaN technology provides good power capability for the 7 GHz range.

## RAN1 request of PA models

RAN4#117 discussed PA models to be provided to RAN1 in response to [2] at length without conclusion. In case this prevails, RAN4 could instead provide guidance and recommendation on PA and Tx modelling given the request for a timely answer before RAN1 waveform studies are completed. We trust that RAN1 is capable of carrying out studies with recommendation on appropriate Tx assumptions.

The results above illustrate the difficulties of recommending a specific model with given parameters, different models were used for modelling the measured PA (one sample) and the results vary. However, EVM improvements of DPD could be demonstrated even if different models were used.

NR narrow-band models based on AM-AM and AM-PM characteristics could be used within their range of validity but are clearly not representative of 7 GHz performance with bandwidths up to 200 MHz. L1 improvements for the 6GR UL should be demonstrated for many UE implementations (read Tx assumptions in this case) and not depend on the use of a specific model/assumption.

In the draft Reply LS below we first provide the following recommendations and guidance

- for 7 GHz, characteristics for commercial PAs in the range 4.4-4.9 GHz can be assumed (as noted in discussions at RAN4#117)

- for bandwidths > [20] MHz, (generalised) memory polynomials should be used; narrow-band (NR) models are not precluded but should be used in their range of validity

- there are also other parameters that affect in-channel Tx performance other than the PA: for studies of EVM with e.g. constellation shaping and improvements of UL maximum output power for higher-order modulation (HOM), an IQ image rejection in the range 30-34 dB and LO suppression > 30 dBc could be assumed

- DPD in the UE can also be considered for EVM studies for HOM with any shaping

with the understanding that RAN1 will not only consider the waveform but also other aspects such as UL coverage enhancements for 6GR in general. In the absence of an agreed PA model, we also ask that companies list the prerequisites:

- company inputs should be presented along with details on the Tx assumptions used e.g. if memory effects were considered, the type of PA model, frequency range and whether enhancements like DPD was assumed

this to assess the prerequisites for the results of a study. For “fair” comparison between results nevertheless, we propose a calibration point

- the legacy ACLR (out-of-band-emission) requirement of [30] dBc should be met in bands below 7 GHz, [26] dBc above 7 GHz at the maximum output power, this even if only in-channel performance is studied by RAN1.

Providing recommendations with a “calibration point” in terms of an OOBE requirement is one way to provide a timely response. RAN4 normally specifies Tx requirements without any quotation of model used, whereas for RAN1 we recommend they list all prerequisites such as type of model used and the frequency range.

Proposal 2: in the event RAN4 cannot agree upon a PA model for the RAN1 LS reply, this can be limited to a set of guidelines and recommendations on Tx modelling for the RAN1 waveform studies.

# Proposal

We first observe that

Observation 1: for RAN4, a specific PA model cannot be assumed for specifying requirements during the work-item phase, but more details on e.g. memory polynomials could be assumed for the 6G SI.

For RAN4 we propose

Proposal 1: Tx assumptions with a cascaded DPD and PA is relevant for state-of-the-art design and should be considered in studies of e.g. DPoD performance and any corresponding UE requirements with increased EVM.

and observe

Observation 2: improvements of the EVM for 64QAM with DPD are also observed for 7 GHz (different PA technology to that used for the mid-band results). GaN technology provides good power capability for the 7 GHz range.

For the RAN1 reply we propose that

Proposal 2: in the event RAN4 cannot agree upon a PA model for the RAN1 LS reply, this can be limited to a set of guidelines and recommendations on Tx modelling for the RAN1 waveform studies.

# References

- R4-2522450, “WF on [117][101] 6G system parameter”, Huawei

- R1-2508069, “LS on PA models in 6GR waveform evaluations”, Huawei

3GPP TSG-RAN WG4 Meeting #118	R4-26XXXXX

Gothenburg, Sweden, 9 – 13 February 2026

# Title:	[DRAFT] Reply LS on PA models in 6GR waveform evaluations

# Response to:	LS on PA models in 6GR waveform evaluations (R1-2508069/R4-2520006)

# Release:	Rel-20

# Work Item:	FS_6G_Radio

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

#### Name: Thomas Montzka

#### E-mail Address: Thomas.Montzka@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: None

1. Overall Description:

RAN4 would like to thank RAN1 for the LS in R1-2508069 on PA models for 6GR waveform evaluations.

It is the understanding of RAN4 than RAN1 for its studies will not only consider the waveform but also other aspects such as UL coverage enhancements for 6GR in general.

Rather than providing explicit PA model(s) with the understanding that L1 improvements for 6GR should work

for many implementations, RAN4 would like to provide the following recommendations and guidance:

- for 7 GHz, characteristics for commercial PAs in the range 4.4-4.9 GHz can be assumed

- for bandwidths > [20] MHz, (generalised) memory polynomials should be used; narrow-band (NR) models are not precluded but should be used in their range of validity

- there are also other parameters that affect in-channel Tx performance other than the PA: for studies of EVM with e.g. constellation shaping and improvements of UL maximum output power for higher-order modulation (HOM), an IQ image rejection in the range 30-34 dB and LO suppression > 30 dBc could be assumed

- DPD in the UE can also be considered for EVM studies for HOM with any shaping

In the absence of one agreed PA model for RAN1 waveform evaluations, company inputs should be presented along with details on the Tx assumptions used e.g. if memory effects were considered, type of PA model, frequency range and whether enhancements like DPD was assumed, this to assess the prerequisites for the results of a study.

In order to compare results, the legacy ACLR (out-of-band-emission) requirement of [30] dBc should be met in bands below the 7-15 GHz range, [26] dBc at 7 GHz, at the stated maximum output power, this even if only in-channel performance is studied by RAN1.

RAN4 will also study aspects of UL coverage improvements and EVM requirements for HOM as part of the 6G study item.

2. Actions:

To RAN5 group.

ACTION: RAN1 should take the above into account for its waveform studies.

3. Date of Next RAN WG4 Meetings:

3GPPRAN4#118bis		13 – 17 April 2026		St Julians, Malta

3GPPRAN4#119		18 – 22 May 2026		TBD, China