3GPP TSG RAN WG4 Meeting #118	R4-2601473

Gothenburg, Sweden, Feb. 09-13, 2026

Agenda item:			6.11.2

Source:	vivo

Title:	Discussion on RRM performance requirements for beam management

Document for:	Discussion

- Introduction

The performance part of this WI was preliminarily discussed in the last meeting. Based on the progress of WF from that meeting [1], we will provide our analysis on the following aspects:

- Accuracy requirements

- RRM test cases

- Discussion

The discussion on the performance part in the last meeting conclude the following issues:

Issue 2-9: Test cases/scenarios Tests only for case 1 Test scenarios: SSB to CSI-RS CSI-RS to CSI-RS Test case scenarios: - TCI state switch with predicted known conditions (CSI-RS to CSI-RS is applicable for UE supports FG59-1) - FFS: SSB to CSI-RS - Test L1-RSRP prediction reporting delay. - Test prediction accuracy based on measurement, and prediction covers the below options with respect to UE capability: - L1-RSRP prediction - Beam ID prediction FFS on any other tests FFS how many tests, testing time and coverage to also be considered

- 2.1 Prediction Accuracy requirements

Issue 2-6: Value of x Agreement: - Regarding the metric for beam ID only prediction, where top-1 predicted beam(s) are reported, RAN4 requirements are specified based on the Y% of probability that the ground truth RSRP of the predicted beam is larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – X dB - X and Y are TBD based on the simulations Regarding the metric for RSRP+beam ID prediction, only RSRP accuracy requirement will be defined.

Regarding the accuracy requirements, a thorough analysis has already been conducted in the core part section.

As for BM-Case 1, the supported reporting content is as follows, mainly including:

- CRI or SSBRI of Top 1 beam

- Absolute predicted RSRP for Top-1 beam

- Differential predicted RSRP with reference to the largest predicted RSRP

Agreement For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case1, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams CRI or SSBRI #1. CRI or SSBRI #2. …. CRI or SSBRI #K. RSRP #1. differential RSRP #2. …. differential RSRP #K. - CRI or SSBRI #k is mapped to RSRP #k, where k = 1,2,…,K - RSRP #1 is absolute predicted RSRP - Differential RSRP #2~#K are differential predicted RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1

Regarding beam prediction accuracy, since RAN4 has not previously define similar requirements, the basic framework and conditions for this accuracy requirement need further discussion within RAN4. To some extent, this also depends on the ongoing discussions regarding testability. As for deriving the specific accuracy values, based on our understanding, they should be determined through the simulations currently being conducted.

Regarding L1-RSRP prediction accuracy, it includes both Absolute Predicted RSRP and Differential Predicted RSRP.

For Absolute Predicted RSRP, which applies to predicting the Top 1 beam, the existing Absolute RSRP accuracy requirements for FR2 can serve as a reference framework for defining this requirement.

For Differential Predicted RSRP, similar to the existing Relative RSRP accuracy requirements for FR2, when the UE reports more than one beam, this requirement should also be checked. The specific accuracy values corresponding to these two requirements, as well as the beam prediction accuracy, should be derived from ongoing simulations. Specifically, the predicted RSRP of all beams that are required to be reported should met this requirement.

Proposal 1: For Beam ID prediction accuracy, RAN4 needs to further discuss the basic framework and conditions. The specific metric values should be determined through the simulations currently being conducted.

Proposal 2: For L1-RSRP prediction accuracy, which includes Absolute Predicted RSRP and Differential Predicted RSRP requirements, they apply respectively to the predicted Top 1 beam and the remaining beams that need to be reported (if reporting more than one beam is required). The existing Absolute RSRP accuracy requirements for FR2 and Relative RSRP accuracy requirements can serve as a reference framework for defining this requirement.

When defining specific performance requirements, it is necessary to consider how many different sets of requirements should be defined to accommodate different scenarios. Therefore, we first need to identify the factors influencing prediction performance. We have listed several potential factors:

- The number of set B beams,

- The number of set A beams

- Beam pattern

For the number of Set B/Set A beams, current simulations consider the combination where Set B contains 1/4 of Set A beams (e.g., Set B = 8 beams, Set A = 32 beams). In addition to this scenario, RAN4 may also need to evaluate cases where Set B contains 1/8 of Set A beams (e.g., Set B = 8 beams, Set A = 64 beams) to assess the extent of prediction performance degradation compared to the 8/32 scenario.

For beam pattern, this factor has been discussed in previous simulation meetings. Different beam patterns do affect simulation performance. Based on prior discussions, the beam pattern adopted in current simulation assumptions ensures that Set B beams are distributed across both elevation and azimuth, which should represent a typical real-world scenario. Thus, further validation of additional patterns may not be necessary.

Proposal 3: When defining specific performance requirements, on how many different sets of metric requirements should be defined to accommodate various scenarios, RAN4 to first identify the factors that influence prediction performance, potentially including:

- The number of set B beams, the number of set A beams

  - candidate value:

    - With measurements of Set B of beams that of 1/4 of Set A of beams (e.g., 8 for set B, 32 for set A)

    - With measurements of Set B of beams that of 1/8 of Set A of beams (e.g., 8 for set B, 64 for set A)

- Beam pattern

  - Consider the beam pattern defined in Table 6 of the existing simulation assumptions

- 2.2 RRM test cases

Based on the agreement listed as above, two test cases will be defined in RAN4, including

- TCI state switch with predicted known conditions (CSI-RS to CSI-RS is applicable for UE supports FG59-1)

  - FFS: SSB to CSI-RS

- Test L1-RSRP prediction reporting delay.

For TCI state switching, both CSI-RS to CSI-RS and SSB to CSI-RS should be covered. This should include both MAC-CE based active TCI state switching and RRC based active TCI state switching. The legacy corresponding test cases on Active TCI state switching delay can serve as a reference framework for defining this requirement.

For L1-RSRP prediction reporting delay, the following specific test cases need to be defined. Currently, the legacy corresponding test cases on L1-RSRP measurement for beam reporting can serve as a reference framework for defining this requirement.

- SSB based L1-RSRP measurement when DRX is not used

- SSB based L1-RSRP measurement when DRX is used

- CSI-RS based L1-RSRP measurement when DRX is not used

- CSI-RS based L1-RSRP measurement when DRX is used

Proposal 4: For test cases on TCI state switching with predicted known conditions, both CSI-RS to CSI-RS and SSB to CSI-RS should be covered. This should include both MAC-CE based active TCI state switching and RRC based active TCI state switching. The legacy corresponding test cases on Active TCI state switching delay can serve as a reference framework for defining this requirement.

Proposal 5: For L1-RSRP prediction reporting delay, the following specific test cases need to be defined. Currently, the legacy corresponding test cases on L1-RSRP measurement for beam reporting can serve as a reference framework for defining this requirement.

- SSB based L1-RSRP measurement when DRX is not used

- SSB based L1-RSRP measurement when DRX is used

- CSI-RS based L1-RSRP measurement when DRX is not used

- CSI-RS based L1-RSRP measurement when DRX is used

- Summary

Proposal 1: For Beam ID prediction accuracy, RAN4 needs to further discuss the basic framework and conditions. The specific metric values should be determined through the simulations currently being conducted.

Proposal 2: For L1-RSRP prediction accuracy, which includes Absolute Predicted RSRP and Differential Predicted RSRP requirements, they apply respectively to the predicted Top 1 beam and the remaining beams that need to be reported (if reporting more than one beam is required). The existing Absolute RSRP accuracy requirements for FR2 and Relative RSRP accuracy requirements can serve as a reference framework for defining this requirement.

Proposal 3: When defining specific performance requirements, on how many different sets of metric requirements should be defined to accommodate various scenarios, RAN4 to first identify the factors that influence prediction performance, potentially including:

- The number of set B beams, the number of set A beams

  - candidate value:

    - With measurements of Set B of beams that of 1/4 of Set A of beams (e.g., 8 for set B, 32 for set A)

    - With measurements of Set B of beams that of 1/8 of Set A of beams (e.g., 8 for set B, 64 for set A)

- Beam pattern

  - Consider the beam pattern defined in Table 6 of the existing simulation assumptions

Proposal 4: For test cases on TCI state switching with predicted known conditions, both CSI-RS to CSI-RS and SSB to CSI-RS should be covered. This should include both MAC-CE based active TCI state switching and RRC based active TCI state switching. The legacy corresponding test cases on Active TCI state switching delay can serve as a reference framework for defining this requirement.

Proposal 5: For L1-RSRP prediction reporting delay, the following specific test cases need to be defined. Currently, the legacy corresponding test cases on L1-RSRP measurement for beam reporting can serve as a reference framework for defining this requirement.

- SSB based L1-RSRP measurement when DRX is not used

- SSB based L1-RSRP measurement when DRX is used

- CSI-RS based L1-RSRP measurement when DRX is not used

- CSI-RS based L1-RSRP measurement when DRX is used

- References

- R4-2522430, WF on [117][112] R19 AI for air interface, Qualcomm

- R4-2511797, WF on updated simulation assumption for AI based beam management, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT