3GPP TSG-RAN WG4 Meeting #118	R4-2601774

Gothenburg, SE, 9th – 13th February 2026

Source:	Nokia

Title:	On RRM performance requirements for CJTC reporting

Agenda item:	6.12.2.2

Document for:	Discussion

Introduction

In this paper we discuss remaining topics for CJTC reporting.

As part of this discussion, we present proposals on

- how to capture the CJTC accuracy

- revision of accuracy targets which are currently not achievable

- clarification on the reasons for invalid reports

Discussion

Requirements for CJT compensation reporting

In the last RAN4 meeting, there was a discussion about how to capture accuracy requirements for CJTC as part of the CR discussion. This discussion is not captured in the WF, but we bring our analysis here. The main point for discussion on the CJTC accuracy requirements was whether to capture the requirements in number of quantization steps or in Hz, us (%CP).

Table 1 Simulation results for 900 MHz and 3500 MHz centre frequencies.

|  |  | FC = 900 MHz |  | FC=3500 MHz |  |
| --- | --- | --- | --- | --- | --- |
| SNR (dB) | SCS | maxabsP5P95 quantization steps | maxabsP5P95 [HZ] | maxabsP5P95 quantization steps | maxabsP5P95 [HZ] |
| -3 | 15 | 113 | 79.8 | 29 | 79.6 |
| 0 |  | 64 | 45.2 | 16 | 43.9 |
| 3 |  | 42 | 29.6 | 11 | 30.2 |
| -3 | 30 | 127 | 89.6 | 55 | 151.0 |
| 0 |  | 120 | 84.7 | 31 | 85.1 |
| 3 |  | 78 | 55.1 | 20 | 54.9 |

In our view, RAN4 defines requirements in a general way, what should not be linked to very specific configurations. Having that in mind, we discuss focusing on FO error. The CJTC report is quantized based on the centre frequency, and therefore, an error of 1 quantization step means something completely different depending on the centre frequency. Table 1 shows our simulation results for achieved UE accuracy based on 900 MHz and 3500 MHz. When comparing the error in quantization steps for 3 dB SNR and 15 kHz SCS, the quantization error is drastically different. For 900 MHz the achieved error is 42 quantization steps for 900 MHz whereas it is 11 steps for 3.5 GHz. When looking at the error values in Hz, the difference is only from 29.6 Hz to 30.2 Hz.

If RAN4 would define the FO error is in levels, it would be much stricter requirement for lower frequency bands, because the quantization levels are in ppm. Therefore, the way that it is most simple to define generic requirements is in Hz.

The CJTC report FO error in steps is highly dependent on the centre frequency.

The expected reporting error of FO is similar for 3.5 GHz and 900 MHz carrier.

Additionally, from the throughput analysis shown in Figure 1, the achieved throughput for a given FO error in Hz is very similar for 900 MHz and 3.5 GHz centre frequencies.

Define accuracy for FO in Hz

Figure 1 Throughput analysis based on the FO error

When TO accuracy is concerned, the options for defining the requirements are 3

- Option 1: requirements in us

- Option 2: requirements in percentage of the CP length

- Option 3: requirements in quantization levels.

As for the FO requirements, defining the requirements for TO in quantization levels have some serious drawbacks that we would like to highlight. The error in quantization levels will be depending on the configuration of the CJTC reporting, and will therefore be hard to map to different configurations if Option 3 above is chosen. As for the requirements as percentage of the CP length or in micro-seconds, there is big technical concern between the two options. We think derining in us is slightly clearer.

Define accuracy for TO in us

In RAN4#118, the following is decided for CJTC reporting with square brackets for few of the FO requirements and the DO requirements on the second set [1]:

# <Topic 1: Performance requirements for CJT reporting> Issue 1-1: Accuracy requirements for delay and frequency offset reporting for CJT calibration Agreement: - For FO, define two set of accuracy requirements with UE capability based on 3dB SNR, for 0.2ppm and 256 levels - For 15kHz SCS, [33] and [3] - For 30kHz SCS, [68] and [4] - For DO, define two set of accuracy requirements with UE capability based on 3dB SNR, for CP and 256 levels - For 15kHz SCS, 14 and [3]; - For 30kHz SCS, 18 and [4] - Note: Achieving better accuracy requires advanced UE implementation and hence UE capability to be introduced. - The Note will not be captured in the CR.

The combined simulation results from different companies is shown in the Appendix of this paper.

When observing the CJTC requirements for the set 2 accuracy, the requirements with 3 or 4 quantization levels are extreme in comparison to the achieved results from different companies, i.e. this accuracy level was not achieved in any of the simulation results shown by other companies. Therefore, we propose that those are revised to reflect achievable targets.

Figure 1 presents link level simulation results showing the achievable throughput for different values of frequency offset error. When observing the results for 15 kHz SCS, 50 Hz error (19 steps for 3.5 GHz FC) provides a throughput result which is close to ideal CJT. When observing the results for 30 kHz SCS, it can be observed that 100 Hz (37 steps for 3.5 GHz FC) error is enough for providing nearly ideal CJT gain.

Therefore, considering our results, we would like to revise the set 2 accuracy levels to 50 Hz and 100 Hz for 15 kHz and 30 kHz SCS, which can be achieved at least by 2 simulation results provided in RAN4 116bis (see Appendix).

Revise set 2 accuracy requirements for CJTC-F to

  - 50 Hz (19 steps) for 15 kHz SCS

  - 100 Hz (37 steps) for 30 kHz SCS

Figure 2 presents link level simulation results showing the achievable throughput for different values of frequency offset error. When observing this figure for 15 kHz SCS, an error of 13 steps, or 0.2345 us, is good enough to provide good throughput gain in comparison to no compensation. Likewise, for 30 kHz SCS, an erro26 steps or 235 ns is also good enough to provide good throughput gain due to CJT.

Additionally, the minimum UE capability for quantization steps is 32 steps for CJTC-D. From the discussion for the Set 2 UE capability, it was not discussed whether this UE capability would be applicable only for UEs supporting a certain quantization level. Therefore, we propose that the minimum accuracy for set 2 should be achievable also for UEes supporting 32 quantization levels for CJTC-D.

Considering the discussion above, we propose to review the set 2 requirements to 9 levels to 30 kHz SCS (83 ns) and 8 levels for 15 kHz SCS (150 ns). Those values are achievable by at least 2 companies based on the combine results of the appendix, and are also possible to be reached for UEs supporting 32 quantization levels.

Revise set 2 accuracy for CJTC-D to 150 ns (8 quantization levels) for 15 kHz SCS and 83 ns (9 quantization levels) for 30 kHz SCS.

Figure 2 Throughput analysis based on the TO error

Requirements for CJT compensation reporting

There is an important issue that could be discussed as part of the tests for CJT calibration, and this is the impact of invalid measurements in the test. The invalid conditions are defined as a possible reporting state for CFO and PO, however the RAN 1 agreements did not include a definition of the invalid state. UE is not configured in anyway to detect such invalid states, therefore the decision making associated with invalid state reporting is left to implementation of the device. The latter may be a problem during the test because if different UE devices under test, report invalid measurements in different amounts and by unknown reasons, the number of valid measurements out of the total of measurements is affected by that, and that could bias the test statistics.

We think that the test should be able to measure not only the accuracy of a valid measurement, but also it should be able to introduce features in the test that make it possible to include both invalid and valid measurements in the test in defined proportions, and verify the capability of the UE to determine whether a measurement is to be reported as valid or invalid. For that, it should also be needed to include in the test, for instance, a test mode feature in which the testing equipment may configure the UE with conditions that define the validity or invalidity of the measurements. Otherwise, UE may inform the threshold(s) used to determine the validity of a measurement. By knowing or configuring the conditions, the test can be configured to test the UE with a determined sequence of valid and/or invalid measurements.

The endorsed CR from RAN4 #117 includes Invalid states in the mapping tables, however there is no definition of what invalid means.

Define the conditions where the UE may report invalid measurements.

Discuss about the impact of invalid measurements in the CJTC reporting test and possible ways to verify the conditions for invalid reports for CJT calibration.

Conclusion

The following Observations and Proposals were made:

Observation 1: The CJTC report FO error in steps is highly dependent on the centre frequency.

Observation 2: The expected reporting error of FO is similar for 3.5 GHz and 900 MHz carrier.

Proposal 1: Define accuracy for FO in Hz

Proposal 2: Define accuracy for TO in us

Proposal 3: Revise set 2 accuracy requirements for CJTC-F to

- 50 Hz (19 steps) for 15 kHz SCS

- 100 Hz (37 steps) for 30 kHz SCS

Proposal 4: Revise set 2 accuracy for CJTC-D to 150 ns (8 quantization levels) for 15 kHz SCS and 83 ns (9 quantization levels) for 30 kHz SCS.

Proposal 5: Define the conditions where the UE may report invalid measurements.

Proposal 6: Discuss about the impact of invalid measurements in the CJTC reporting test and possible ways to verify the conditions for invalid reports for CJT calibration.

- References

- R4-2522638, WF on NR_MIMO_Ph5_RRM_Part2, MediaTek

Appendix – combined simulation results from RAN4 116bis

Table 2 – Combined of simulation results from RAN4 116bis for FO reporting

|  |  | FO estimation error in AFO = 0.2 ppm and MFO = 256 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | SNR | Frequency offset between 2 TRPs | MTK | Ericsson | Huawei | Nokia |
| 15 kHz | 3 | 350 Hz | 26 | 7 | 33 | 11 |
| 30 kHz | 3 | 350 Hz | 52 | 12 | 68 | 17 |

Table 3– Combined of simulation results from RAN4 116bis for TO reporting

|  |  | TO estimation error in AD = CP and MD = 256 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | SNR | Time offset between 2 TRPs | MTK | Ericsson | Huawei | Nokia |
| 15 kHz | 3 | 0.5 CP | 10 | 3 | 12 | 5 |
| 30 kHz | 3 | 0.5 CP | 14 | 5 | 8 | 8 |