3GPP TSG-RAN WG4 Meeting #118 						R4-2601874

Göteborg, Sweden,  Feb 9 - 13, 2026

Source:		Ericsson; Rohde & Schwarz

Title:			A-IoT BS conformance remaining issue

Agenda item:		6.14.2

Document for:		Approval

# Introduction

In this paper, we present our views on remaining issue for Rel-19 A-IoT conformance testing.

# Discussion

In WF[1], FFS test model for the modulation quality test.

Issue 1-3: Test model for modulation quality

- Agreement:

  - FFS test model for the modulation quality test, e.g M and # of PRB in FRC table

For the test model, the A-FR1_B1-1/2/3/4 are basically differentiated with number of RB and M.  The simulation shows the synthesized waveform with the less RB and high M can have more prominent “distortion” and may be subject to further impairment by RF chain and worth to be tested.  We suggest that for modulation test, use the low RB number and high M value combination to test, which is A-FR1-B1-1 (M=6 for postamble) or A-FR1-B1-2 (M=12 for postamble) and only the postamble is tested for modulation signal quality. As the channel bandwidth is declared by manufacturer, at least the 200kHz (RB) can be supported, therefore, it will be safe to use A-FR1-B1-1.  Note in clause 6.2.4,the 4 bits [1 1 1 1 ] is defined for postamble and with Manchester coding, it will be always low to high transition so the bit pattern of [0 1 0 1 0 1 0 1].

The higher M and lower RB size introduce more “distortion” on the synthesized signal

Modulation signal test on the post-amble only and use A-FR1-B1-1 to test.

Figure 3: Modulation quality comparison with different RB nubmer and M

In the modulation quality requirement, the E-field strength (V/m) is specified as requirement. E-field strength has been used as a requirement in OTA and the measurement method is OTA test measured in a far field anechoic chamber. In 38.141-2, the RMS filter-strength can be converted to EIRP at certain distance r and quoted as below:

“Adjust the signal generators to the type of interfering signals, levels and the frequency offsets as specified for general test requirements in table 7.6.5.1.1-1. The distance between the test object and test antenna injecting the interferer signal is adjusted when necessary to ensure specified interferer signal level to be received.”

Table 7.6.5.1.1-1: OTA out-of-band blocking performance requirement

| Wanted signal mean power (dBm) | Interfering signal RMS field-strength (V/m) | Type of interfering signal |
| --- | --- | --- |
| EISminSENS + 6 dB (Note 1) | 0.36 V/m | CW carrier |
| NOTE 1: EISminSENS depends on the channel bandwidth as specified in TS 38.104 [2], clause 10.2.1. NOTE 2: The RMS field-strength level in V/m is related to the interferer EIRP level at a distance described as , where EIRP is in W and r is in m; for example, 0.36 V/m is equivalent to 36 dBm at fixed distance of 30 m. |  |  |

In 38.141-2, the equation between EIRP to RMS field-strength is given to facilitate the test method for OTA test.

The mathematical relation between the E-field and the received power is explained well in [2]. The power flux density received at effective antenna area is used as an intermediate parameter to link the received power and the E-field strength. E-field requirement may be relevant to the A-IoT device 1 design but as the conducted test is agreed last meeting instead of OTA,  it is a question that how to relate the E-field  to the conducted measurement. E-field strength measured in volts per meter describing the voltage induced on a receiving antenna at a specific point in space, polarization and frequency. The relation between measured power at antenna and the E-field is described in [5] and it is found out that the intended measurements can be calculated in a relatively manner so the distance, antenna gain etc can be cancelled out in the equations, this makes it possible to measure it in conduct manner.

It is feasible to measure the E-field using the conducted manner.

According to the equations in [2] and conversion between measurements and different performance qualities[5], the test procedure can be constructed below:

Testing procedure:

- Transmit the test model [A-IoT-TM-x.1] for modulation quality test.

- Locate pulse train within post-amble part of the transmitted signal and select one pulse corresponding to chip 0. measure either preceding pulse (chip 1) or selected pulse (chip 0) according to description below:

PAn : the RMS average power measured over 1/2 duration of the high level of preceding pulse;

PBn : the RMS average power measured over 1/2 duration of the low level of preceding pulse;

PAn_h : the highest instant power measured of the high level of preceding pulse over 1/2 duration of preceding pulse;

PAn_l : the  lowest instant power measured of the high level of preceding pulse over 1/2 duration of preceding pulse;

PBn_h : the highest instant power measured of the low level over 1/2 duration of selected pulse.

PBn_l : the lowest instant power measured of the low level over 1/2 duration of selected pulse.

- Mark the point in time as t1 on the selected pulse where the power starts to fall from high level of preceding pulse to low level of selected pulse at a power level below:

P90%_fall=90%∙PAn-PBn2

- Mark the point in time t2 on the selected pulse where the power starts to fall from high level of preceding pulse to low level of selected pulse at another power level below:

P10%_fall=10%∙PAn-PBn2

- Mark the point in time as t3 on the selected pulse where the power starts to rise from low level of selected pulse to high level of following pulse at the power level P10%_rise below:

P10%_rise=P10%_fall

- Mark the point in time t4 on the selected pulse where the power starts to rise from low level of selected pulse to high level of following pulse at the power level P90%_rise  below:

P90%_rise=P90%_fall

- Compare the measurements to the ripple requirement using formular below:

Ripplehigh%= maxPAn_h-PAnPAn-PBn; PAn_l-PAnPAn-PBn∙100%

Ripplelow%=maxPBn_h-PBnPAn-PBn; PBn_l-PBnPAn-PBn∙100%

- Compare the measurements to the modulation requirement using formular below:

Modulation depth=PAn-PBnPAn

- Compare the measurements to the rising/fall time using formular below:

Tf,10-90 = t2- t1

Tr,10-90 = t4- t3

The test procedure described above is wordy and it can be considered to be placed in annex if needed.

Discuss and agree the test procedure above to be used in test case in modulation signal test.

Figure X-1:Timing mask for OOK chip 0 pulse

# Conclusions

In this contribution, we present our overview on the A-IoT conformance testing with below observation and proposals:

Observation 1 The higher M and lower RB size introduce more “distortion” on the synthesized signal

Observation 2 In 38.141-2, the equation between EIRP to RMS field-strength is given to facilitate the test method for OTA test.

Observation 3 It is feasible to measure the E-field using the conducted manner.

Proposal-1: Modulation signal test on the post-amble only and use A-FR1-B1-1 to test.

Proposal-2: Discuss and agree the test procedure above to be used in test case in modulation signal test.

# References

- R4-2522956, Way Forward for [117][314] A-IoT_BS_CW_conformance, Huawei

- https://scdn.rohde-schwarz.com/ur/pws/dl_downloads/dl_application/application_notes/1ma85/1MA85_7e_Estimator.pdf

- EPC Compliant Class-1 Generation-2 UHF RFID Devices Conformance Requirements

- ISO/IEC FCD 10373-6; Identification cards — Test methods — Part 6: Proximity cards

- R4-2522003, A-IoT BS conformance remaining issue, Ericsson