3GPP TSG-RAN WG4 Meeting #118	R4-2601921

Gothenburg, SE, 9th – 13th February 2026

Source:	Nokia

Title:	Discussion on the applicability of test cases when downselection of SMTCs is triggered at UE side

Agenda item:	6.19.2.2

Document for:	Discussion

Introduction

In [1], RAN plenary has approved a new WI for NTN, which will progress the effort of specification for NTN systems in 3GPP technologies in Rel-19 (NR_NTN_PH3). The WID contain a handful of objectives and among them, there is one topic on Downlink Coverage Enhancements. One of the main features discussed to be implemented as part of this feature is an extended SSB periodicity for the main (SA) cell in NTN, to match the periodic “active” and “inactive” periods of the cell. The switching between “active” and “inactive” is created in NTN to handle the limited power availability in the satellite.

In previous RAN4 meeting, agreements were made respective to the scope of the test cases, including selection of test cases with multiple periodicities. There was an open issue left in the WF from previous meeting regarding whether to implement a test caser relative to the downselection of SMTCs when more than 4 SMTCs is configured [1].

Discussion

In the WF from previous meeting, [1], it RAN4 discussed whether to introduce test cases for the downselection of the SMTCs, when more than 4 SMTCs are configured:

Sub-topic 1-2 Other parts for NR NTN Issue 1-2-1: Test scope of SMTC enhancement for the UE capable of coverage enhancement < Way forward >: - For more than 4 SMTCs per frequency in RRC Idle/Inactive mode, RAN4 shall wait for conclusion on SMTC selection related issue discussed in core maintenance, and then to discuss the following options: - Option 1: RAN4 not to define the related test cases. - Option 2: RAN4 to define the TC for Cell reselection to FR1 intra-frequency NR case with more than 4 SMTCs in RRC Idle mode.

A Test Case can only be implemented if there is a clear “test outcome” expected from the UE, such that the test can verify whether the UE is performing the actions as intended. In this particular case, to test whether the UE is implementing the downselection in appropriate manner, it is important that the test can unequivocally identify which SMTCs are to be selected by the UE, such  that there is such determinism on the test outcome. However this is not possible in this case. The main reason is that RAN2, when developing this feature, has indicated that the selection of SMTCs to be measured is left for UE implementation, and the “refLocMapList” and “refLocList” are references that “may” be used (see highlight below from RAN2 text). A UE might decide to implement different methods for the selection of SMTCs, such as RSRP based measurements or others. Based on that, we consider the feature not prone to be tested and we prefer option 1.

The downselection of SMTCs to be used does not need to follow the reference location list, UE can do it using other means, for example, RSRP based measurements.

smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in smtc4list and smtc5list, it is up to the UE to select which SMTCs to consider, for which the UE may use location information in refLocMapList and refLocList in SIB19, if present. The network only includes SMTC offsets in smtc5list for (quasi-)Earth fixed cells. The total number of configurable SMTC offsets across smtc4list and smtc5list does not exceed 6. The total number of different SMTC periodicities across smtc and smtc5list does not exceed 2. If an entry in smtc5list is present but the pci-List and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list and the periodicity value in smtc5list. If an entry in smtc5list is present but its fields are absent, the UE applies the values from the entry at the same position in smtc4list.

One of the reasons for RAN2 agreement is to not add the obligation of the NW to implement the refLocList (specification says “if present”) and to not add requirements to the UE to acquire continuously up-to-date reference location lists.

Do not define new test cases to verify the downselection of SMTCs when 4 or more SMTCs are implemented.

Conclusion

The paper has discussed whether to include a new test case with multiple SMTCs, when the UE is forced to downselect the number of SMTCs. Based on the technical discussion we have made the following observation and the following proposal with our technical recommendation:

Observation 1: The downselection of SMTCs to be used does not need to follow the reference location list, UE can do it using other means, for example, RSRP based measurements.

Proposal 1: Do not define new test cases to verify the downselection of SMTCs when 4 or more SMTCs are implemented.

- References

- R4-2522938, Topic summary for [117][327] Rel-19 RRM perf_Part3_NTN, Moderator (CATT), WG4 Meeting # 117, Dallas, US, 17th – 21st Nov, 2025

- TS 38.331, NR; Radio Resource Control (RRC); Protocol specification, V19.1.0, 3GPP