3GPP TSG RAN WG4 Meeting #118	R4-2602001

Gothenburg , 9th – 13st February 2026

Source:	Rohde & Schwarz (R&S)

Title:	Measurement Uncertainty for KU Band SAN Conformance

Agenda item:	5.7.4

Document for:	Discussion

# Introduction

While at RAN4#117 (Dallas), it was decided that we should pursue MU studies for KU Band SAN Conformance. R&S would like to propose the usage of existing tables from 37.941 and reuse them for KU (12-18 GHz).

# Discussion

In order to determine the scope of testing, first various apertures and variants of SAN were identified and referenced in the table below[1]. GEO SAN aperture was outside the limits for standard test procedures. LEO SAN aperture is well within testability norms.

Observation 1: Given the size of GEO KU SAN ~4m, its testability with commercial chambers is out of reach

When testing LEO SAN at KU frequencies, a DFF Chamber will need to be very large. Fraunhofer distance which is the boundary for far field measurements is 2D2λ, and at 12-18 GHz, this distance is about 60-90 meters). Hence only an IDFF (Indirect Far Field) chamber like a CATR(Compact Antenna Test Range) is feasible. For testability and MU calculations, we should ONLY consider CATR chambers for KU Bands (similar to FR2 frequencies).

| Aperture size D | Frequency | Fraunhofer Distance |
| --- | --- | --- |
| 0.9 m | 12 GHz | 60 m |
| 0.9 m | 18 GHz | 90 m |

Observation 2: Consider ONLY CATR Chambers for KU LEO SAN (similar assumptions to FR2)

On the Chamber (CATR) side, we expect room to bring down the MU. We are requesting feedback from the Industry experts on this. CATR centric uncertainties are listed as A*_* & B*_* in the MU excel table drafts attached with this discussion paper. On the Test Equipment side, the MU allowance is like FR2.

Observation 3: MU reduction study needed on the Chamber centric variables; Industry feedback requested

# Summary

Observation 1: Given the size of GEO KU SAN ~4m, its testability with commercial chambers is out of reach

Observation 2: Consider ONLY CATR Chambers for KU LEO SAN (similar assumptions to FR2)

Observation 3: MU reduction study needed on the Chamber centric variables; Industry feedback requested

# References

[1] 	38.863, NTN RF & Co-ex aspects

[2]	37.941, Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements

[3]	R4-2522216, 3GPP RAN4 #117 Topic summary for [117][308] Rel-19_BS_SAN_conf_Part2_NTN