3GPP TSG-RAN WG4 Meeting #118	R4-2602012

# Gothenburg, Sweden, 09 – 13 February 2026

Agenda item:		7.16.3

Source: 		Qualcomm Incorporated

Title: 		Feasibility considerations for higher power class for NB-IOT NTN

Document for:		Approval

## 1	Introduction

Work item for Non-Terrestrial Networks (NTN) for Internet of Things (IoT) phase 4 tasks RAN4 to study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37 dBm) for NB-IoT NTN. In this contribution initial considerations on the feasibility are provided.

## 2	Discussion

Coexistence considerations

In Release 19 RAN4 evaluated the feasibility for power classes up to PC1, i.e. up to 31 dBm, and specified support for NB-IoT NTN PC2 and PC1 for both handheld and non-handheld devices. As part of this work also co-existence study was done, results being captured in TR 38.863. For IoT NTN PC1 result resulted in ACLR of 24.7 dB with isolation distance and 31.4 dB without isolation distance.

These results are based on the worst case scenario (scenario 4 in TR 38.863) where NTN UL interferes TN UL, a scenario which could take place e.g. between n256 UL and n1 UL which are adjacent.

ACLR requirement for NB-IoT NTN is 37 dB, specified as UTRA ACLR limit following measurement bandwidths of UTRA system. Just looking at the ACLR numbers, there is margin in place to increase output power from PC1 level, even further so if the requirement is set considering the isolation distance as done in previous releases.

Given that RAN4 is allocated only with 0.25 TUs per meeting, extensive co-existence simulations may be challenging to perform. It could be preferable to aim to take advantage of the existing simulation results for PC1 and the margins in place compared to the specified ACLR limit to draw conclusions on required ACLR and whether existing ACLR limit could be re-used e.g. up to 37 dBm.

Observation 1: Co-existence evaluations for PC1 indicate possibility to increase output power above PC1 level.

Proposal 1: Aim to draw co-existence conclusion by scaling the PC1 evaluation results

Proposal 2: Re-use 37 dB ACLR requirements for power class(es) up to 37 dBm.

Implementation considerations

Handheld satellite phones which have very high output power exist. For example, there are phones for Iridium system which closely align with the WI target power [1]. From the technical specifications it can be seen that this handheld device can provice 4 hours of talk time, while the EU type examination certification shows 37.7 dBm output power.

Figure 1: Snippets of materials available in [1]

The produce regulatory and safety card which is linked to the same web page confirms that this is conducted power provided to the antenna connector. The device weight is not different from a smartphone with a large display.

Observation 2: Handheld battery powered satellite phones with much higher conducted output power than PC1 are available today.

Given the existence of this kind of products in the market, it is definitely feasible to create handheld formfactor devices with output power up to 37 dBm. It is recognized that for smartphones there may be more restrictions than for a dedicated satellite phone, though, and design for the Iridium system benefits from limited UL duty cycle, similar to NB-IoT TDD. For non-handheld form factors there are fewer implementation restrictions to consider. Below we provide examples of methods that are available in standards to make implementation easier, and which can be considered, if necessary.

- Extending lower power tolerance

- UL duty cycle based power reduction

- Considering MPR0 waveform only for narrow allocation in the middle of the band

- Allowing MPR for all tone allocations

- Considering nominal output power which is less than 37 dBm

  - 37 could still be allowed by looser upper tolerance

Observation 3: Final requirements could be defined in a manner which facilitate easier implementation, if necessary.

Finally, when it comes to component availability and performance, it is perfectly expected that not all components are available in mass market and supporting up to 37 dBm operation today, especially for smartphone form factor. There has been no mass market for them as even standard does not allow such operation for 3GPP systems yet. Therefore, solving the chicken-egg problem can start by enabling the higher power in standards, after which component development is more likely to accelerate.

Observation 4: Component performance and availability is likely to improve after standards enable using said higher power. Therefore, it can be expected that all components may not commercially available to support 37 dBm today.

Proposal 3: No device form factors should be ruled out at least in the initial phase of the feasibility study.

Single Tx PC1 has been specified also for NR in terrestrial operation. PAs capable of PC1 operation, with potential further enhancements, as well as enhanced narrowband PAs could be considered as part of the evaluations on feasible requirements.

Proposal 4: Consider e.g. NR PC1 PAs and enhanced narrowband PAs as part of the evaluation

## 3	Conclusion

In this contribution up to 37 dBm power class for NB-IoT NTN was discussed. Following observations and proposals were made:

Observation 1: Co-existence evaluations for PC1 indicate possibility to increase output power above PC1 level.

Proposal 1: Aim to draw co-existence conclusion by scaling the PC1 evaluation results

Proposal 2: Re-use 37 dB ACLR requirements for power class(es) up to 37 dBm.

Observation 2: Handheld battery powered satellite phones with much higher conducted output power than PC1 are available today.

Observation 3: Final requirements could be defined in a manner which facilitate easier feasibility, if necessary.

Observation 4: Component performance and availability is likely to improve after standards enable using said higher power. Therefore, it can be expected that all components may not commercially available to support 37 dBm today.

Proposal 3: No device form factors should be ruled out at least in the initial phase of the feasibility study.

Proposal 4: Consider e.g. NR PC1 PAs and enhanced narrowband PAs as part of the evaluation

## 4	References

[1]  Iridium 9555 Satellite phone, 9555 Satellite Phone