# 3GPP TSG-RAN WG4 Meeting #118	R4-2602020

# Gothenburg, Sweden, February 09 - 13, 2026

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.181 | CR |  | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 36.181: SAN conformance requirements for BOG feature (LTE-based 5G broadcast operation over geosynchronous satellite) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_band_5G_bcast_GSO-Perf |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | While final CR to TS 36.181 was assigned to another company, in this draft CR we share our views (as Rapporteur), on implementation of the BOG feature (LTE-based 5G broadcast operation over geosynchronous satellite) in TS 36.181. This draft CR is expected to be Merged into the final CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implementation of the BOG feature (LTE-based 5G broadcast operation over geosynchronous satellite) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation of the BOG feature (LTE-based 5G broadcast operation over geosynchronous satellite) would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 3, 4.1.2.2, 4.9.2, 6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

# 1	Scope

The present document specifies the Radio Frequency (RF) test methods and conformance requirements for Satellite Access Node (SAN) type 1-H and SAN type 1-O, supporting:

- standalone NB-IoT operation, or

- E-UTRA, or

- NB-IoT operation in NTN NR in-band, or

- LTE-based 5G broadcast operation over geosynchronous (BOG) satellite.

These have been derived from and are consistent with the conducted requirements for SAN type 1-H, and radiated requirement for SAN type 1-H and SAN type 1-O in SAN specification defined in TS 36.108 [2].

==============Next change==============

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.108: "Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node radio transmission and reception"

[3]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[4]	ITU-R Recommendation M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[5]	3GPP TR 37.941: "Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements"

[6]	3GPP TS 38.181: "NR; Satellite Access Node (SAN) conformance testing"

[7]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation"

[8]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures"

[9]	3GPP 38.141-2: "NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing"

[10]	3GPP TS 36.141: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing"

[11]	ITU-R Recommendation SM.328: "Spectra and bandwidth of emissions".

[12]	3GPP TS 36.211: Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”

[13]	3GPP TR 38.811: "Study on New Radio (NR) to support non-terrestrial networks".

[14]	3GPP TS 38 108: "Space Access Node (SAN) radio transmission and reception".

[15]	ITU-R Recommendation SM.1541-6: "Unwanted emissions in the out-of-band domain".

[16]	[12]	WRC-19 Resolution 761, Coexistence of International Mobile Telecommunications and the broadcasting-satellite service (sound) in the frequency band 1452-1492 MHz in Regions 1 and 3, WRC-19, https://www.itu.int/dms_pub/itu-r/oth/0C/0A/R0C0A00000F00162PDFE.pdf

==============Next change==============

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5G Broadcast over Geosynchronous Satellite: LTE-based 5G Broadcast utilizing a geosynchronous satellite.

SAN RF Bandwidth: RF bandwidth in which a SAN transmits and/or receives single or multiple carrier(s) within a supported operating band.

==============Next change==============

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AA	Antenna Array

AAS	Active Antenna System

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AoA	Angle of Arrival

AWGN	Additive White Gaussian Noise

BOG         	LTE-based 5G Broadcast over Geosynchronous Satellite

BW	Bandwidth

CA	Carrier Aggregation

CP-OFDM	Cyclic Prefix-OFDM

CW	Continuous Wave

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

EARFCN	E-UTRA Absolute Radio Frequency Channel Number

EIRP	Equivalent Isotropic Radiated Power

EIS	Equivalent Isotropic Sensitivity

EVM	Error Vector Magnitude

E-UTRA	Evolved UTRA

FR	Frequency Range

FRC	Fixed Reference Channel

GEO	Geostationary Earth Orbiting

ICS	In-Channel Selectivity

LEO	Low Earth Orbiting

MCS	Modulation and Coding Scheme

N-TM	NB-IoT Test Model

NB-IoT	Narrowband – Internet of Things

NTN	Non-Terrestrial Network

OOB	Out-of-band

OOBE	Out-of-band Emissions

OSDD	OTA Sensitivity Directions Declaration

OTA	Over-The-Air

PRB	Physical Resource Block

QAM	Quadrature Amplitude Modulation

RB	Resource Block

RDN	Radio Distribution Network

RE	Resource Element

REFSENS	Reference Sensitivity

RF	Radio Frequency

RIB	Radiated Interface Boundary

RMS	Root Mean Square (value)

RoAoA	Range of Angles of Arrival

RX	Receiver

SAN 	Satellite Access Node

SCS	Sub-Carrier Spacing

SDO	Standalone Downlink Only

SPRF 	Satellite Payload RF

TAB	Transceiver Array Boundary

TRP	Total Radiated Power

TX	Transmitter

==============Next change==============

#### 4.1.2.2	Measurement of transmitter

The maximum conducted Test System uncertainty for conducted transmitter tests minimum requirements is given in table 4.1.2.2-1. And the maximum OTA Test System uncertainty for OTA transmitter tests minimum requirements is given in table 4.1.2.2-2.

Table 4.1.2.2-1: Maximum Test System uncertainty for conducted transmitter tests

| clause | Maximum Test System Uncertainty |
| --- | --- |
| 6.2 SAN output power | ±0.7 dB for E-UTRA, f ≤ 3 GHz ±1 dB for standalone NB-IoT |
| 6.3 Output power dynamics | ±0.4 dB |
| 6.5.2 Frequency error | ±12 Hz |
| 6.5.3 EVM | ±1% |
| 6.5.5 DL RS power | ±0.8 dB, f ≤ 3 GHz |
| 6.6.2 Occupied bandwidth | 1.4 MHz channel BW: 30 kHz 10 MHz channel BW: |
| 6.6.3 Adjacent Channel Leakage power Ratio (ACLR) | ±0.8 dB |
| 6.6.4 Operating band unwanted emissions | ±1.5 dB, f ≤ 3 GHz |
| 6.6.5.5 TX spurious emissions: General requirements | 9 kHz < f ≤ 4 GHz: ±2 dB 4 GHz < f ≤ 15 GHz: ±4 dB |
| 6.6.5.6 TX spurious emissions: Protection of SAN receiver | ±3 dB |
| NOTE 1: Test system uncertainty values are applicable for normal condition unless otherwise stated. NOTE 2: Measurement testing system uncertainty requirements for SAN output power, frequency error, EVM and DL RS requirement are identical for NB-IoT standalone mode and NB-IoT operation in NTN NR in-band. |  |

==============Next change==============

### 4.9.2	Test models

#### 4.9.2.1	General

The following clauses will describe the SAN test models needed for SAN type 1-H and SAN type 1-O.

Feature-specific test models are classified in the following clauses:

- Test models for E-UTRA are captured in clause 4.9.2.2:

- Test models for NB-IoT are captured in clause 4.9.2.3:

- Test models for BOG are captured in clause 4.9.2.4.

#### 4.9.2.2	Test models for E-UTRA

The set-up of physical channels for transmitter tests shall be according to one of the E-UTRA test models (E-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all E-UTRA test models:

-	The test models are defined for a single antenna port (using p = 0); 1 code word (q = 0), 1 layer, precoding is not used; unless specified otherwise

-	Duration is 10 subframes (10 ms)

-	Normal CP

-	Virtual resource blocks of localized type, no intra-subframe hopping for PDSCH

-	UE-specific reference signals are not used

Power settings of physical channels are defined by physical channel EPRE relative to the EPRE of the RS. The relative accuracy of the physical channel EPRE as referred to the EPRE of the RS shall have a tolerance of ±0.5 dB.

##### 4.9.2.2.1	E-UTRA Test Model 1.1 (E-SAN-TM1.1)

The test model E-TM1.1 in TS 36.141 [10] apply for E-SAN-TM1.1 and carrier bandwidth is only limited to 1.4 MHz.

This model shall be used for tests on:

-	SAN output power

-	Unwanted emissions

-	Occupied bandwidth

-	ACLR

-	Operating band unwanted emissions

-	Transmitter spurious emissions

##### 4.9.2.2.2	E-UTRA Test Model 1.2 (E-SAN-TM1.2)

The test model in E-TM1.2 in TS 36.141 [10] apply for E-SAN-TM1.2 and carrier bandwidth is only limited to 1.4 MHz.

This model shall be used for tests on:

-	Unwanted emissions

-	ACLR

-	Operating band unwanted emissions

##### 4.9.2.2.3	E-UTRA Test Model 2 (E-SAN-TM2)

The test model in E-SAN-TM2 in TS 36.141 [10] apply for E-SAN-TM2 and carrier bandwidth is only limited to 1.4 MHz.

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power),

-	EVM of single 64QAM PRB allocation (at min power)

-	Frequency error (at min power)

##### 4.9.2.2.4	E-UTRA Test Model 3.1 (E-SAN-TM3.1)

The test model in E-TM3.1 in TS 36.141 [10] apply for E-SAN-TM3.1 and carrier bandwidth is only limited to 1.4 MHz.

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper OFDM symbol power limit at max power with all 64QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error

-	EVM for 64QAM modulation (at max power)

##### 4.9.2.2.5	E-UTRA Test Model 3.2 (E-SAN-TM3.2)

The test model in E-TM3.2 in TS 36.141 [10] apply for E-SAN-TM3.2 and carrier bandwidth is only limited to 1.4 MHz.

This model shall be used for tests on:

-	Transmitted signal quality

-	Frequency error

-	EVM for 16QAM modulation

##### 4.9.2.2.6	E-UTRA Test Model 3.3 (E-SAN-TM3.3)

The test model in E-TM3.3 in TS 36.141 [10] apply for E-SAN-TM3.3 and carrier bandwidth is only limited to 1.4 MHz.

This model shall be used for tests on:

-	Transmitted signal quality

-	Frequency error

-	EVM for QPSK modulation

#### 4.9.2.3	Test Model for NB-IoT

The set-up of physical channels for transmitter tests shall be according to the NB-IoT Test Model (N-TM) below.

The following general parameters are used:

-	The test models are defined for a single antenna port (using p = 1000);

-	Duration is 10 subframes (10 ms)

-	Normal CP

The following physical channel parameters are used:

-	The ratio of synchronisation signal EPRE and NRS EPRE is 0 dB

-	NPDCCH format 1

The test model for NB-IoT are identical for NB-IoT standalone mode and NB-IoT operation in NTN NR in-band.

#### 4.9.2.3A	Test Model for BOG

==============Next change==============

## 6.3	Output power dynamics

### 6.3.1	General

The requirements in clause 6.3 apply during the transmitter ON period. Transmit signal quality requirements (as specified in clause 6.5) shall be maintained for the output power dynamics requirements of this clause.

NOTE:	The requirements in clause 6.3 do not apply for BOG, as the DL transmission bandwidth is fixed.

==============Next change==============

### 6.5.3	Modulation quality

#### 6.5.3.1	Definition and applicability

Modulation quality is defined by the difference between the measured carrier signal and an ideal signal. Modulation quality can e.g. be expressed as Error Vector Magnitude (EVM). The Error Vector Magnitude is a measure of the difference between the ideal symbols and the measured symbols after the equalization. This difference is called the error vector.

For SAN type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

#### 6.5.3.2	Minimum Requirement

The minimum requirement is in TS 36.108 [2], clause 6.5.2.

#### 6.5.3.3	Test purpose

The test purpose is to verify that modulation quality is within the limit specified by the minimum requirement.

#### 6.5.3.4	Method of test

##### 6.5.3.4.1	Initial conditions

Test environment: Normal; see annex B.

RF channels to be tested for single carrier: B, M and T; see clause 4.9.1.

RF bandwidth positions to be tested for multi-carrier: BRFBW, MRFBW and TRFBW in single-band operation, see clause 4.9.1.

##### 6.5.3.4.2	Procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.1.1 for SAN type 1-H. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	For a TAB connector declared to be capable of single carrier operation only (D.39), set the TAB connector under test to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test models:

-	E-SAN-TM3.1 if 64QAM is supported by SAN without power back off, or

-	E-SAN-TM3.1 at manufacturer's declared rated output power if 64QAM is supported by SAN with power back off, and E-SAN-TM3.2 at maximum power, or

- 	E-SAN-TM3.2 if highest modulation order supported by SAN is 16QAM, or

-	E-SAN-TM3.3 if highest modulation order supported by SAN is QPSK.

For TAB connector declared to be capable of multi-carrier operation, set the TAB connector under test to transmit according to the applicable test configuration and corresponding power setting specified in clauses 4.7 and 4.8 using the corresponding test models on all carriers configured:

-	E-SAN-TM3.1 if 64QAM is supported by SAN without power back off, or

-	E-SAN-TM3.1 at manufacturer's declared rated output power if 64QAM is supported by SAN with power back off, and E-SAN-TM3.2 at maximum power, or

-	E-SAN-TM3.2 if highest modulation order supported by SAN is 16QAM, or

-	E-SAN-TM3.3 if highest modulation order supported by SAN is QPSK.

For E-SAN-TM3.1, power back-off shall be applied if it is declared.

2)	Measure the EVM and frequency error as defined in annex H.

#### 6.5.3.5	Test requirements

The EVM of each E-UTRA carrier for different modulation schemes on PDSCH shall be less than the limits in table 6.5.3.5-1.

Table 6.5.3.5-1 EVM requirements for E-UTRA carrier

| Modulation scheme for PDSCH or sPDSCH | Required EVM (%) |
| --- | --- |
| QPSK | 18.5 |
| 16QAM | 13.5 |
| 64QAM (NOTE) | 9 |
| NOTE: EVM requirement for 64QAM is optional. |  |

The EVM of each NB-IoT carrier on NB-PDSCH shall be less than the limits in table 6.5.2.5-2:

Table 6.5.3.5-2 EVM requirements for NB-IoT carrier

| Modulation scheme for NB-PDSCH | Required EVM (%)(%) |
| --- | --- |
| QPSK | 18.5 |
| 16QAM | 13.5 |

The EVM of each BOG carrier for different modulation schemes on PMCH shall be less than the limits in table 6.5.3.5-3.

Table 6.5.3.5-3: EVM requirements for BOG

| Modulation scheme for PMCH | Required EVM (%) |
| --- | --- |
| QPSK | 18.5 |
| 16QAM | 13.5 |

The EVM requirement shall be applicable within a time period around the centre of the CP therefore the EVM requirement is tested against the maximum of the RMS average of 10 subframes at the two window W extremities.

Table 6.5.2.5-3 and table 6.5.2.5-4 specify EVM window length (W) for normal CP, the cyclic prefix length  is 160 for symbols 0 and 144 for symbols 1-6.

Table 6.5.2.5-3 EVM window length for normal CP for E-UTRA

| Channel Bandwidth MHz | FFT size | Cyclic prefix length for symbols 0 in FFT samples | Cyclic prefix length for symbols 1-6 in FFT samples | EVM window length W | Ratio of W to total CP for symbols 1-6 (NOTE) (%) |
| --- | --- | --- | --- | --- | --- |
| 1.4 | 128 | 10 | 9 | 5 | 55.6 |
| NOTE: These percentages are informative and apply to symbols 1 through 6. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |  |

Table 6.5.2.5-4 EVM window length for normal CP for NB-IoT

| FFT size | Cyclic prefix length for symbols 0 in FFT samples | Cyclic prefix length for symbols 1-6 in FFT samples | EVM window length W | Ratio of W to total CP for symbols 1-6 (NOTE) (%) |
| --- | --- | --- | --- | --- |
| 128 | 10 | 9 | 3 | 33.3 |
| NOTE: These percentages are informative and apply to symbols 1 through 6. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |

Table B.5.1-3: EVM window length for extended CP for 15 kHz subcarrier spacing

| Channel bandwidth (MHz) | FFT size | Cyclic prefix in FFT samples | EVM window length W | Ratio of W to total CP (NOTE) (%) |
| --- | --- | --- | --- | --- |
| 10 | 1024 | 256 | 250 | 97.7 |
| Note: These percentages are informative. |  |  |  |  |

==============Next change==============

### 6.5.5	DL RS power

#### 6.5.5.1	Definition and applicability

For E-UTRA, DL RS power is the resource element power of Downlink Reference Symbol.

The absolute DL RS power is indicated on the DL-SCH. The absolute accuracy is defined as the maximum deviation between the DL RS power indicated on the DL-SCH and the DL RS power of each E-UTRA carrier at the SAN TAB connector.

For NB-IoT, DL NRS power is the resource element power of the Downlink Narrow-band Reference Signal.

The absolute DL NRS power is indicated on the DL-SCH. The absolute accuracy is defined as the maximum deviation between the DL NRS power indicated on the DL-SCH and the DL NRS power of each NB-IoT carrier at the SAN TAB connector.

For BOG, this requirement is not applicable.

==============Next change==============

##### 6.6.2.4.2	Procedure

1	Measure the spectrum emission of the transmitted signal using at least the number of measurement points, and across a span, as listed in table 6.6.2.4.2-1. The selected resolution bandwidth (RBW) filter of the analyser shall be 30 kHz or less.

Table 6.6.2.4.2-1: Span and number of measurement points for OBW measurements

|  | SAN channel bandwidth BWChannel (MHz) |  |  |
| --- | --- | --- | --- |
|  | 0.2 (NOTE 1) | 1.4 (NOTE 2) | 10 (NOTE 3) |
| Span (MHz) | 0.4 | 10 | 20 |
| Minimum number of measurement points | 400 | 1429 | 400 |
| NOTE 1: 0.2 MHz channel bandwidth applies to NB-IoT only. NOTE 2: 1.4 MHz channel bandwidth applies to E-UTRA only. NOTE 3: 10 MHz channel bandwidth applies to BOG only. |  |  |  |

NOTE:	The detection mode of the spectrum analyzer will not have any effect on the result if the statistical properties of the out-of-OBW power are the same as those of the inside-OBW power. Both are expected to have the Rayleigh distribution of the amplitude of Gaussian noise. In any case where the statistics are not the same, though, the detection mode must be power responding. The analyser may be set to respond to the average of the power (root-mean-square of the voltage) across the measurement cell.

2)	Compute the total of the power, P0, (in power units, not decibel units) of all the measurement cells in the measurement span. Compute P1, the power outside the occupied bandwidth on each side. P1 is half of the total power outside the bandwidth. P1 is half of (100 % - (occupied percentage)) of P0. For the occupied percentage of 99 %, P1 is 0.005 times P0.

3)	Determine the lowest frequency, f1, for which the sum of all power in the measurement cells from the beginning of the span to f1 exceeds P1.

4)	Determine the highest frequency, f2, for which the sum of all power in the measurement cells from f2 to the end of the span exceeds P1.

5)	Compute the occupied bandwidth as f2 - f1.

#### 6.6.2.5	Test requirements

For E-UTRA, the occupied bandwidth for each carrier shall be less than the channel bandwidth as defined in TS 36.108 [2], clause 5.3A.

For NB-IoT standalone operation, the occupied bandwidth for carrier shall be less than the channel bandwidth as defined in TS 36.108 [2], clause 5.3B.

For BOG operation, the occupied bandwidth for carrier shall be less than the channel bandwidth as defined in TS 36.108 [2], clause 5.3C.

==============Next change==============

#### 6.6.3.5	Test requirements

The ACLR is defined with a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centred on the assigned channel frequency and a filter centred on the adjacent channel frequency according to the tables below.

The ACLR shall be higher than the value specified in table 6.6.3.5-1 and table 6.6.3.5-2.

Table 6.6.3.5-1: SAN ACLR limit for GEO class

| SAN channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | SAN adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| 1.4, 10 | BWChannel | E-UTRA of same BW | Square (BWConfig) (NOTE 1) | 13.2 |
|  | 2 x BWChannel | E-UTRA of same BW | Square (BWConfig) (NOTE 1) | 13.2 |
| NOTE 1: BWChannel and BWConfig are the SAN channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: 1.4 MHz channel applies for E-UTRA; 10 MHz channel applies for BOG. |  |  |  |  |

==============End of change==============