3GPP TSG RAN WG4 Meeting #118    		                          R4-2602124

Gothenburg, Sweden, Feb. 09-13, 2026

Agenda item:		7.6.4.2

Source:			Samsung

Title:			Discussion on HD-FDD for Ku band with FR1 numerology

Document for:		Discussion and decision

Introduction

In the RAN4#117, we have already discussed the RRM requirements for HD-FDD for Ku band with FR1 numerology and latest agreements are included in [1]. There are still some FFS issues in [1]. In this contribution, we continue to discuss the FFS parts of RRM requirements for HD-FDD VSAT for Ku band with FR1 numerology.

- Discussion

In the RAN4#117 meeting, the potential issues for further discussion are captured in WF [1] as below:

#### Issue 2-1-1: RRM requirements scope for HD-FDD VSAT with FR1 numerology Agreement - RAN4 to confirm that VSATs in Ku bands with FR1 numerology does not take inter-satellite measurements into this WI scope. - Discuss whether to capture requirement applicability rule regarding the VSAT types, - Option 1: clarify the HD-FDD VSAT requirements applicability in clause 3 in CR - Option 2: no applicability rule for VSAT type needed.

In WID, the objective of the WI is to introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna and Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type. When the relevant requirements are defined, it is necessary to clearly describe the applicable VSAT types for these requirements. If the applicability of HD-FDD VSAT requirements is not clarified in clause 3 of the CR, then each requirement shall specify the applicable HD-FDD VSAT types, leading to some redundant wording. Therefore, we prefer Option 1, which clarifies the applicability of HD-FDD VSAT requirements in clause 3 of the CR.

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type

Proposal 1: RAN4 to clarify the HD-FDD VSAT requirements applicability in clause 3 in CR.

#### Issue 2-1-2: Handover and statellite switch Agreement - For HD-FDD VSAT, reuse the existing handover, conditional handover, satellite switch requirements defined for NTN VSAT - FFS whether to capture the following applicability conditions in CR - For RACH-based HO, - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU - For Time/location-based CHO, - One SSB is available during T∆ - One SSB is available during TIU - For satellite switching requirements, - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU

RAN4 have already discussed the same issue in RedCap with NTN in R19, and RAN1 has defined that SSB is always prioritized over UL. Considering that HD-FDD VSAT with FR1-NTN does not introduce any special characteristics for handover, we support option 1, for HD-FDD VSAT, reuse the existing handover, conditional handover, satellite switch requirements defined for NTN VSAT

Proposal 2: RAN4 to reuse the existing handover, conditional handover, satellite switch requirements defined for NTN VSAT.

#### Issue 2-1-5: Other RRM requirement impact due to HD-FDD Agreement - For HD-FDD VSAT, reuse the legacy applicability conditions for HD-FDD for the following existing VSAT requirements - Maximum interruption in paging reception - Random access requirements - RLM and L1-RSRP requirements, including CSI-RS based measurement and L1 indication. - FFS whether to add condition as at least one SSB is available.

We don’t see the requirement of adding the condition that at least one SSB is available. We think reusing the legacy applicability conditions for HD-FDD for the Maximum interruption in paging reception, Random access requirement and RLM and L1-RSRP requirements are sufficient.

Proposal 3: RAN4 to reuse the legacy applicability conditions for HD-FDD for the following existing VSAT requirements
o	Maximum interruption in paging reception
o	Random access requirement
o	RLM and L1-RSRP requirements, including CSI-RS based measurement and L1 indication.

Conclusion

This contribution discusses the impact of HD-FDD for FR1-NTN core part operation on RAN4 RRM. The following are the proposed points in this contribution.

Proposal 1: RAN4 to clarify the HD-FDD VSAT requirements applicability in clause 3 in CR.

Proposal 2: RAN4 to reuse the existing handover, conditional handover, satellite switch requirements defined for NTN VSAT.

Proposal 3: Proposal 3: RAN4 to reuse the legacy applicability conditions for HD-FDD for the following existing VSAT requirements
o	Maximum interruption in paging reception
o	Random access requirement
o	RLM and L1-RSRP requirements, including CSI-RS based measurement and L1 indication.

Reference

[1] R4-2523030, Way Forward for [117][330] NR_IoT_NTN_Ph2_Part2 RRM part