3GPP TSG-RAN WG4 Meeting #118										                    R4-2602131

Gothenburg, Sweden, 9 – 13 February 2026

Title:	MPR reduction via ACLR relaxation for Band n104

Agenda item:	10.2.2

Source:	Skyworks Solutions, inc.

Document for:	Approval

# 1.	Introduction

In this contribution we publish some Power Amplifier (PA) measurements to share our views on how much MPR reduction may be applied to band n104 thanks to ACLR relaxation.

# 2.	Discussion

We summarize below the agreements reached at meeting #117 [1]. In this contribution, we some PA measurement results to evaluate MPR reduction for n104. Due to lack of time, we are unable to present our results for CA_n104C at this meeting but would be glad, if time permits, to share these measurements at meeting #118-bis.

<Way forward>

RAN4 introduces a band specific ACLR of 26 dB to n104 with a dedicated MPR for following cases.

- PC3 for n104 single carrier

- PC3 for CA_n104C, where only contiguous RBs allocations are considered. For non-contiguous RBs allocations, the existing MPR applies

- 1Tx and 2Tx are considered

<Topic 2: Evaluation conditions>

<Way forward>: Following conditions will be used for MPR evaluation for n104 and for CA_n104C contiguous RB allocations with new ACLR of 26 dB

- PA calibration point: PC3 -30dBc ACLR at MPR1 with the DFT-s-OFDM QPSK 20MHz Lcrb=100(RBstart=0) waveform

- RF impairments: -28dBc I/Q, CIM3 : -60dBc, CIM5 : -70dBc

- Definition of inner/outer/edge RB allocations is the same as is today

- If required, modification of edge RB definition for n104 is not out of scope.

<Topic 3: How to introduce the ACLR>

<Way forward >: A new n104 specific ACLR of 26 dB for PC3is introduced for single CC and intra-band contiguous CA without NS signaling.

## MPR reduction evaluation for Band n104

## Scope of the study

The objective of this study is to evaluate if the power class (PC3) MPR could be reduced specifically for band n104 thanks to ACLR relaxation from the 30dBc requirement to 26dBc. We noticed that at RAN4 meeting #117, some companies proposed MPR reduction for many RB allocations, including for example Edge RB allocations. In our view, it is important that RAN4 agrees that if any MPR reduction is to be agreed upon, such reduction may only apply to RB allocations for which the current PC3 MPR is limited by ACLR, i.e. the scope of the PC3 MPR reduction should focus only on eligible RB allocations and waveforms. For example, DFT-s-OFDM 256QAM waveforms should de-facto be out-of-scope since MPR is dominated for all RB allocations by EVM or by the in-band emissions (IBE) requirements.

Proposal 1: The scope of the PC3 MPR reduction via ACLR relaxation for Band n104 should be restricted to eligible RB allocations, i.e. to allocations and waveforms for which the MPR is “gated” or limited by the ACLR requirement of -30dBc.

The preliminary measurements we presented at the last RAN4 meeting were reproduced from ealier studies on QPSK power boosting and indicated that relaxing the PC3 ACLR to 26dBc may create further opportunities for less than zero MPR, i.e. for power boosting. After careful evaluation of the measurements presented in this contribution and considering that 5G-Advanced requirements are coming to an end, we consider that any power boosting should be postponed to 6GR studies or Work Items. In addition, 6GR may create much more promising opportunities for MPR reduction and power boosting than a sole ACLR reduction. For example, early discussions on 6GR indicate that RAN4 may study the benefits of improving the PA linearity by calibrating the PA at MPR0 rather than MPR1. Another opportunity for power boosting improvement may arise from improved RF transceiver impairments, for example by improving the current 5G NR assumptions of -28dBc image rejection and local oscillator leakage. It may also be possible that for 6G, and in particular for band 104, that RAN4 revisits the 1st segment of the spectrum emission mask which currently sets stringent emission requirements to protect a close proximity narrowband carrier. In our view, such SEM requirement is obsolete for band n104 and any new 6G bands in the range ~7GHz to 8GHz.

Observation 1: Power boosting should not be part of the band n104 MPR reduction since 1) if power boosting was to be considered, then other gating factors than just ACLR should be accounted for, 2) power boosting was already specified for 5G NR, there is little time left to perform such studies in Rel-20 for 5G, 3) it is more appropriate to study power boosting in the scope of 6GR Rel-20 studies where we anticipate great opportunities for power boost. For example, new MPR0 PA calibration, improved RF transceiver Image rejection/LO leakage assumptions, ACLR relaxation, and may be even revisited SEM requirements.

Proposal 2: MPR reduction via ACLR relaxation for PC3 should not target power boosting, i.e. it should target MPR ≥ 0dB relative to the PC3 PPowerClass of 23dBm (nominal PC3 maximum output power).

## Measurement results

Our measurements cover the following limited range of RB allocations in a 20MHz SCS 30kHz carrier:

- LCRB candidates: 1, ,4, 16, 25, 32, 40, 50

- RBstart candidates: ranging from 0 to center of the channel

- Waveforms:

  - DFT-s-OFDM: Pi/2 BPSK, QPSK, 16QAM, 64QAM

  - CP-OFDM QPSK,

for which we evaluate the PA output back-off (OBO) to gate ACLR at 30dBc, 26dBc, EVM, IBE and SEM.

In this section we discuss only the DFTs OFDM QPSK results. All other measurements results can be found the Annex.

## PC3 DFT-s-OFDM QPSK Results

Table 2 summarizes the measurement results for DFT-s-OFDM QPSK waveforms in which we highlight:

- in blue: the tested outer RB allocations

- in yellow, column “ACLR 30dBc”: Eligible RB allocations (cf. proposal 1) to MPR reduction due to gating by ACLR30dBc

- in red text (both ACLR30dBc / ACLR26dBc columns): negative OBO meaning that for these allocations, the PA is able to deliver less than 0dB MPR, i.e. it is able to support power boosting.

The results obtained for gating against ACLR30/ACLR26 are captured in separate columns, and for each waveform the gating requirement is reported. For example, the egde RB allocation 1(RBstart=0) is gated by SEM and hence this allocation is not eligible to MPR reduction.

In the last two columns, we present the reduction of OBO between gating at ACLR 30 vs gating at ACLR 26 and in the last column we show the absolute OBO reduction that can be achieved when gating the measurements with ACLR 26dBc.

Table 2: PC3 PA OBO measurement results for ACLR30 and ACLR 26

We observe that:

- For gating at 30dBc ACLR:

  - The results are similar to those previously studied in the scope of the QPSK power boosting. For many RB allocations, the PA is able to achieve power boosting, i.e. showing how much margin is provided by the current MPR requirements.

  - Eligible RB allocations, ie. allocations gated by ACLR are all OUTER RB allocations.

  - However, not all OUTER allocations are eligible to MPR reduction. For example, we find that LCRB=4(RBstart=0) is gated by SEM.

- For gating at 26dBc ACLR:

  - For nearly all of the tested waveforms, the PA is able to achieve less than 0dB MPR,

  - Looking at the relative OBO results, not all eligible allocations may benefit from OBO reduction, e.g. LCRB=16(RBstart=2),

  - Looking at the absolutive OBO results, we observe that the PA is able to achieve at least a 1dB OBO reduction all the eligible OUTER whose LCRB >16RB.

Similar trends can be observed from the collected data for DFT-s-OFDM Pi2 BPSK and 16QAM and CP-OFDM QPSK – see the Annex for details. For DFT-s-OFDM 64QAM, OBO reduction seems feasible for very few RB allocations.

Observation 2: From the PC3 DFT-s-OFDM QPSK results, we observe:

- RB allocations eligible to MPR reduction (cf. proposal 1) are all OUTER allocations

- However, not all OUTER allocations are eligible to MPR reduction.

- The MPR of all OUTER RB allocations whose transmission bandwidth “LCRB” is greater than NRB/2 may be reduced to 0dB for PC3.

- Based on the results captured in the Annex MPR reduction may also be achieved for all OUTER RB allocations whose LCRB” is greater than NRB/2, and for those allocations:

  - DFT-s-OFDM Pi/2 BPSK: 	MPR 0dB can be proposed

  - DFT-s-OFDM 16QAM: 	MPR 0.5dB can be proposed

  - CP-OFDM QPSK: 			MPR 2.0B can be proposed

  - Results for CP-OFDM 16QAM are not available at the time of writing.

## PC3 MPR proposal for Band n104

We make the following proposal based on observation 2. We tentatively propose 1dB MPR reduction for CP-OFDM, hoping to be able to confirm in the course of this meeting.

Proposal 3: Consider the following PC3 MPR requirements for Band n104 for ACLR relaxation to 26dBc.

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w/ Rel-15 DMRS | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52,3 | ≤ 0.52 06 | 02,4 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52,3 | 02 | 02,4 |
|  | QPSK | ≤ 1 | ≤ 1 06 | 05 |
|  | 16 QAM | ≤ 2 | ≤ 2 ≤ 0.56 | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 | ≤ 3 ≤ 26 | ≤ 1.5 |
|  | 16 QAM | ≤ 3 | ≤ 3 [≤ 2]6 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for conditions where note 1 does not apply. NOTE 3: For 3 MHz channel bandwidth the Pi/2 BPSK edge allocation MPR is 1 dB NOTE 4: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is set to 1, the reference power is increased by ΔPPowerBoost NOTE 5: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is set to 1, the reference power is increased by ΔPPowerBoost NOTE 6: Applicable for LCRB >= NRB/2. |  |  |  |  |

# Conclusion

In this contribution we present PC3 PA measurements to evaluate MPR reduction that may be achieved by relaxing the ACLR requirements from 30dBc to 26dBc.

Proposal 1: The scope of the PC3 MPR reduction via ACLR relaxation for Band n104 should be restricted to eligible RB allocations, i.e. to allocations and waveforms for which the MPR is “gated” or limited by the ACLR requirement of -30dBc.

Observation 1: Power boosting should not be part of the band n104 MPR reduction since 1) if power boosting was to be considered, then other gating factors than just ACLR should be accounted for, 2) power boosting was already specified for 5G NR, there is little time left to perform such studies in Rel-20 for 5G, 3) it is more appropriate to study power boosting in the scope of 6GR Rel-20 studies where we anticipate great opportunities for power boost. For example, new MPR0 PA calibration, improved RF transceiver Image rejection/LO leakage assumptions, ACLR relaxation, and may be even revisited SEM requirements.

Proposal 2: MPR reduction via ACLR relaxation for PC3 should not target power boosting, i.e. it should target MPR ≥ 0dB relative to the PC3 PPowerClass of 23dBm (nominal PC3 maximum output power).

Observation 2: From the PC3 DFT-s-OFDM QPSK results, we observe:

- RB allocations eligible to MPR reduction (cf. proposal 1) are all OUTER allocations

- However, not all OUTER allocations are eligible to MPR reduction.

- The MPR of all OUTER RB allocations whose transmission bandwidth “LCRB” is greater than NRB/2 may be reduced to 0dB for PC3.

- Based on the results captured in the Annex MPR reduction may also be achieved for all OUTER RB allocations whose LCRB” is greater than NRB/2, and for those allocations:

  - DFT-s-OFDM Pi/2 BPSK: 	MPR 0dB can be proposed

  - DFT-s-OFDM 16QAM: 	MPR 0.5dB can be proposed

  - CP-OFDM QPSK: 			MPR 2.0B can be proposed

  - Results for CP-OFDM 16QAM are not available at the time of writing.

Proposal 3: Consider the following PC3 MPR requirements for Band n104 for ACLR relaxation to 26dBc.

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w/ Rel-15 DMRS | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52,3 | ≤ 0.52 06 | 02,4 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52,3 | 02 | 02,4 |
|  | QPSK | ≤ 1 | ≤ 1 06 | 05 |
|  | 16 QAM | ≤ 2 | ≤ 2 ≤ 0.56 | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 | ≤ 3 ≤ 26 | ≤ 1.5 |
|  | 16 QAM | ≤ 3 | ≤ 3 [≤ 2]6 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for conditions where note 1 does not apply. NOTE 3: For 3 MHz channel bandwidth the Pi/2 BPSK edge allocation MPR is 1 dB NOTE 4: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is set to 1, the reference power is increased by ΔPPowerBoost NOTE 5: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is set to 1, the reference power is increased by ΔPPowerBoost NOTE 6: Applicable for LCRB >= NRB/2. |  |  |  |  |

# 4.  References

- R4-2522382 WF on n104 ACLR and MPR evaluation, 3GPP TSG-RAN WG4 Meeting #117, Huawei, HiSilicon.

# Annex - PC3 DFT-s-OFDM Pi/2 BPSK Results

# Annex - PC3 DFT-s-OFDM 16QAM Results

# Annex - PC3 DFT-s-OFDM 64QAM Results

# Annex - PC3 CP-OFDM QPSK Results