3GPP TSG-RAN WG4 Meeting #118	R4-2602136

Göteborg, Sweeden, 9th – 13th of February, 2026

Title: 	Discussion on use cases and NTN configurations - coexistence

Agenda Item:	8.4.3 - NTN SAN RF and NTN coexistence

Release:	Rel-20

Source: 	THALES

Document for:	Discussion

# Discussion

During RAN4#117 the following studies have been considered:

Agreement: The following coexistence studies are considered for the 6G SI with the motivation and needs of such study to be discussed and clarified: - NTN-NTN coexistence - S-band - NTN-TN coexistence: New satellite orbit MEO for S-/Ku-/Ka-band - NTN and TN coverage overlap – adjacent channel – ~2 GHz Study performance impacts based on existing requirements. - NTN-TN coexistence: New satellite orbit vLEO for S-band - NTN-TN coexistence: New 6G C-band considering LEO600 and GEO - NTN-TN coexistence: New 6G Q/V-band considering LEO600 and GEO The priority order for #2 to #6 will be decided in RAN4#118 meeting.

Proposal 1: RAN4 should make an effort and try not descope from priorities already proposed for NTN.

As a matter of fact, in RAN1 the following key issues are currently being discussed:

Proposal 1: - NTN-related technical considerations should be addressed early in the 6G study, including in waveform design, frame structure, channel coding, AI/ML, and evaluation assumptions. - The following aspects should be considered in the design of 6GR radio interface: - Challenges inherent to NTN radio links such as like high Doppler shifts, large and variable round-trip times (RTT), and low signal-to-noise ratios (SNRs). - GNSS independent Physical layer operation - Compatibility with diverse satellite orbits, such as Very LEO, LEO, MEO, and GEO. Proposal 2: For harmonized 6GR design for TN and NTN, RAN1 studies to identify the technical aspects affected by NTN characteristics, including at least: - Time and frequency synchronization - FFS: whether to support pre-compensation of RTT and Doppler at UE and/or Network side. - Physical layer should be designed to work independently of GNSS - Frame structure - Coverage enhancements - PAPR reduction for NTN downlink transmission - Ultra-low BLER avoiding HARQ in NTN - Beam hopping and longer SSB periodicity - Duplexing mode: Support of FDD, HD-FDD and TDD duplexing modes - NTN Propagation impairments - Positioning, Navigation and Timing - 6G NTN coexistence with IoT-NTN or NR-NTN in same beam

Proposal 2: RAN4 should extend the NTN work to consider the topics currently discussed in RAN1

- Set 1 of Key Features for 6G NTN, including at least:

  - NTN-related technical considerations should be addressed early in the 6G study, including in waveform design, frame structure, channel coding, AI/ML, and evaluation assumptions.

  - The following aspects should be considered in the design of 6GR radio interface:

    - Challenges inherent to NTN radio links such as like high Doppler shifts, large and variable round-trip times (RTT), and low signal-to-noise ratios (SNRs).

    - GNSS independent Physical layer operation

    - Compatibility with diverse satellite orbits, such as Very LEO, LEO, MEO, and GEO.

- Set 2 of Key Features for 6G NTN (for harmonized 6GR design for TN and NTN, technical aspects affected by NTN characteristics), including at least:

  - Time and frequency synchronization

    - FFS: whether to support pre-compensation of RTT and Doppler at UE and/or Network side.

    - Physical layer should be designed to work independently of GNSS

  - Frame structure

  - Coverage enhancements

  - PAPR reduction for NTN downlink transmission

  - Ultra-low BLER avoiding HARQ in NTN

  - Beam hopping and longer SSB periodicity

  - Duplexing mode: Support of FDD, HD-FDD and TDD duplexing modes

  - NTN Propagation impairments

  - Positioning, Navigation and Timing

  - 6G NTN coexistence with IoT-NTN or NR-NTN in same beam

Proposal 3: RAN4 to agree on the satellite (SAN) parameters provided at R4-2522244 - SAN Characteristics for 6GR (Thales, ESA, Viasat, Eutelsat Group, Airbus, SES, Hispasat, RAN4#117).

# Annex 1 – PA models – see R4-2522242 - NTN PA model discussion for 6GR air interface (Thales, RAN4#117)

# Annex 2 – SAN parameters – see R4-2522244 - SAN Characteristics for 6GR (Thales, ESA, Viasat, Eutelsat Group, Airbus, SES, Hispasat, RAN4#117)

RAN1 #122bis made the following agreements related to NTN evaluation:

Agreement

The attached templates for NTN in R1-2507956 are endorsed in principle.

Agreement

At least the following carrier frequencies could be considered (from RAN1 perspective) for 6GR NTN evaluations:

- L-band (i.e., 1.5GHz)

- S-band (i.e. 2 GHz)

- Ku-band (FFS detailed frequency range)

- Ka-band (i.e. 30 GHz for UL, 20GHz for DL)

# 3.1 Frequency bands to be considered

Proposal 1: RAN4 to consider at least the following lists of bands for 6G NTN:

- L-band (i.e., 1.5GHz)

- S-band (i.e., 2 GHz)

- C-band (i.e., 4 GHz)

- Ku-band (FFS detailed frequency range)

- Ka-band (i.e., 30 GHz for UL, 20 GHz for DL)

- Q/V-band (i.e., 37 GHz for UL, 47 GHz for DL)

# 3.2 Satellite Payload Parameters

In the following, we provide new satellite access node characteristics for the evaluation of NTN based 6GR access. The aim is to update the satellite parameters listed in Table 6.1.1.1-1 from TR 38.821 (Release 16) with the current state-of-the-art space technologies for 6G network deployments.

The next tables are reporting SAN parameters (like EIRP, G/T, antenna size, and beam diameter) for L-, S-, C-, Ku-, Ka-, and Q/V bands.

NOTE-1: The reported beam diameters refer to the Nadir pointing of the satellite.

NOTE-2: For all frequency ranges, the number of simultaneous active beams may be as low as 1.5% with respect to the total visible beams in the field of view.

Table 1: Satellite parameters in L- and S- bands.

|  |  | Around 1.5 GHz |  | Around 2 GHz |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | L-band LEO 1200km | L-band LEO 600km | S-band LEO 1200km | S-band LEO 600km | S-band VLEO 300km |
| DL | Equivalent satellite antenna aperture | 2.5 m | 2.5 m | 2.5 m | 2.5 m | 1.5 m |
|  | Satellite EIRP density | 38 dBW/MHz | 38 dBW/MHz | 40 dBW/MHz | 40 dBW/MHz | 24 dBW/MHz |
|  | Max Tx Antenna Gain | 30 dBi | 30 dBi | 32 dBi | 32 dBi | 27 dBi |
|  | 3dB beamwidth | 4.50 deg | 4.50 deg | 3.6 deg | 3.6 deg | 6.0 deg |
|  | Beam diameter | 95 km | 50 km | 75 km | 40 km | 30 km |
| UL | Equivalent satellite antenna aperture | 2.5 m | 2.5 m | 2.5 m | 2.5 m | 1.5 m |
|  | Max Rx Antenna Gain | 30 dBi | 30 dBi | 32 dBi | 32 dBi | 27 dBi |
|  | G/T | 3 dB/K | 3 dB/K | 5 dB/K | 5 dB/K | 0.5 dB/K |

Table 2: Satellite parameters in C-band [5]

|  |  | Around 4 GHz |  |
| --- | --- | --- | --- |
|  |  | C-band GEO | C-band LEO 600km |
| DL | Equivalent satellite antenna aperture | 12 m | 2 m |
|  | Satellite EIRP density | 59 dBW/MHz | 34 dBW/MHz |
|  | Max Tx Antenna Gain | 50.5 dBi | 35 dBi |
|  | 3dB beamwidth | 0.42 deg | 2.50 deg |
|  | Beam diameter | 260 km | 30 km |
| UL | Equivalent satellite antenna aperture | 12 m | 2 m |
|  | Max Rx Antenna Gain | 50.5 dBi | 35 dBi |
|  | G/T | 23 dB/K | 7.5 dB/K |

Table 3: Satellite parameters in Ku-band

|  |  | Around 11/14GHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Ku-band MEO 8000km | Ku-band LEO 1200km | Ku-band LEO 600km |
| DL | Equivalent satellite antenna aperture | 1.50 m | 0.45 m | 0.40 m |
|  | Satellite EIRP density | 28 dBW/MHz | 13 dBW/MHz | 7.0 dBW/MHz |
|  | Max Tx Antenna Gain | 42.5 dBi | 32 dBi | 31 dBi |
|  | 3dB beamwidth | 1.00 deg | 3.33 deg | 3.75 deg |
|  | Beam diameter | 140 km | 75 km | 40 km |
| UL | Equivalent satellite antenna aperture | 1.10 m | 0.33 m | 0.30 m |
|  | Max Rx Antenna Gain | 42.5 dBi | 32 dBi | 31 dBi |
|  | G/T | 15.5 dB/K | 5.0 dB/K | 4.0 dB/K |

Table 4: Satellite parameters in Ka-band

|  |  | Around 20/30GHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ka-band GEO | Ka-band MEO 8000km | Ka-band LEO1200km | Ka-band LEO 600km | Ka-band LEO 300km |
| DL | Equivalent satellite antenna aperture | 6.0 m | 1.0 m | 0.35 m | 0.30 m | 0.20 m |
|  | Satellite EIRP density | 42 dBW/MHz | 32 dBW/MHz | 17 dBW/MHz | 11 dBW/MHz | 5.0 dBW/MHz |
|  | Max Tx Antenna Gain | 60 dBi | 43 dBi | 34 dBi | 32.5 dBi | 29 dBi |
|  | 3dB beamwidth | 0.16 deg | 0.95 deg | 2.75 deg | 3.20 deg | 4.75 deg |
|  | Beam diameter | 100 km | 130 km | 60 km | 40 km | 25 km |
| UL | Equivalent satellite antenna aperture | 4.00 m | 0.67 m | 0.23 m | 0.20 m | 0.13 m |
|  | Max Rx Antenna Gain | 60 dBi | 43 dBi | 34 dBi | 32.5 dBi | 29 dBi |
|  | G/T | 31 dB/K | 16 dB/K | 6.7 dB/K | 5.5 dB/K | 2 dB/K |

Table 5: Satellite parameters in Q/V-band [5]

|  |  | Around 37/47GHz |  |
| --- | --- | --- | --- |
|  |  | Q/V-band GEO | Q/V-band LEO 600km |
| DL | Equivalent satellite antenna aperture | 2.7 m | 0.27 m |
|  | Satellite EIRP density | 45 dBW/MHz | 15 dBW/MHz |
|  | Max Tx Antenna Gain | 60.4 dBi | 40.4 dBi |
|  | 3dB beamwidth | 0.18 deg | 1.88 deg |
|  | Beam diameter | 120 km | 20 km |
| UL | Equivalent satellite antenna aperture | 2.1 m | 0.21 m |
|  | Max Rx Antenna Gain | 60.4 dBi | 40.4 dBi |
|  | G/T | 31.8 dB/K | 11.8 dB/K |

Proposal 2: RAN4 to consider for NTN-based 6GR access the SAN characteristics reported in the R1-2509055 from Table 1 to Table 5.

## Satellite Antenna Model

In 5G analysis and studies, the reference satellite antenna model was reflector-based. In 6G, we would like to add the satellite antenna model based on phased-array antennas for NGSO. Mostly of the basic considerations and formulas were already introduced in [4].

Indeed, given the maximum Tx/Rx antenna gain and the equivalent aperture from the previous tables, it is straightforward to compute the necessary single element gain and the total number of elements. The proposed methodology for deriving these parameters of the phased-array antennas is shortly reported below:

- in FR1, the array spacing is exactly dx=dy=0.5λ, which ensures the absence of grating lobes and still a reasonable number of elements in the whole array;

- in FR2, the primary goal is to keep the number of elements not greater than 2000 and at the same time to guarantee all grating lobes off the visible Earth.

Table 6: Satellite phased-array parameters in FR1

|  | L-band | S-band | S-band | C-band |
| --- | --- | --- | --- | --- |
| Equivalent satellite antenna aperture | 2.5 m | 2.5 m | 1.5 m | 2.0 m |
| Array spacing | 0.5l | 0.5l | 0.5l | 0.5l |
| Total array elements | 676 | 1024 | 400 | 2116 |
| Single element gain | 1.14 dBi | 1.28 dBi | 0.92 dBi | 1.74 dBi |

Table 7: Satellite phased-array parameters in Ku-band

|  | Ku-band | Ku-band | Ku-band |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture | 1.5 m | 0.45 m | 0.40 m |
| Array spacing | 1.35l | 0.66l | 0.6l |
| Total array elements | 1936 | 676 | 676 |
| Single element gain | 9.64 dBi | 3.75 dBi | 2.73 dBi |

Table 8: Satellite phased-array parameters in Ka-band

|  | Ka-band | Ka-band | Ka-band |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture | 1.0 m | 0.35 m | 0.30 m |
| Array spacing | 1.35l | 0.66l | 0.6l |
| Total array elements | 2116 | 1024 | 900 |
| Single element gain | 9.72 dBi | 3.75 dBi | 2.98 dBi |

Table 9: Satellite phased-array parameters in Q/V-band [5]

|  | Q/V-band |
| --- | --- |
| Equivalent satellite antenna aperture | 0.27 m |
| Array spacing | 0.5l |
| Total array elements | 3481 |
| Single element gain | 5 dBi |

Proposal 3: RAN4 to consider new phased-array parameters for NTN antenna model reported in R1-2509055 from Table 6 to Table 9.

# References

- RP-251881, New SID: Study on 6G Radio, RAN#108

- RP-252912, Revised SID: Study on 6G Radio, RAN#109

- Chairman’s Notes, 3GPP RAN1 #122bis, Prague, CZ, August 2025

- R1-2403403, “On the SAN phased-array antenna characteristics”, ESA, April 2024

- Carla Amatetti, Juan Bucheli, Dorin Panaitopol, Sorya Tong, Xavier Leturc et al., “6G NTN in C and Q/V-Bands: Link Budget Analysis and Waveforms Performance”, IEEE Globecom 2024.