3GPP TSG-RAN4 Meeting #117	R4-2522358

Dallas, USA, 17 November – 21 November 2025

Title: 	WF on 6G BS RF and co-existence

Agenda item:	8.1

Source:	Feature Lead (Ericsson)

Document for:	Approval

# Agreements and Way Forward

## BS RF requirements

### Issue 1-1-1: 5G RF requirements applicable to 6G

Agreement:

The following existing 5G BS RF requirements are applicable to 6G BS RF at least for the 5G re-farming bands.

- Frequency error

- Occupied bandwidth

- Spurious general requirement

- Receiver spurious requirement

### Issue 1-1-2: 5G RF requirements to be re-evaluated without any new study for 6G

Agreement:

The following 5G BS RF requirements would need to be re-evaluated based on TR 38.817-02, e.g. the existing formula is reused, once the spectrum utilization and channel bandwidths list will be decided for 6G. No further study would be needed for those requirements.

- Sensitivity (OTA)

- Reference sensitivity

- Receiver intermodulation

And at least for the 5G re-farming bands:

- Dynamic Range

- In channel selectivity

### Issues 1-1-3 to 1-1-12: BS RF requirements to be studied

Agreement:

The following BS RF requirements will be studied during the 6G SI:

- 1st priority:

  - EVM, detailed scope to be further discussed:

    - For new 6G modulation

    - Study + measurement methodologies

    - More pragmatic EVM requirement framework (e.g., MIMO layers dependent framework, other considerations are not precluded)

  - In-band and Out of band blocking

    - Reconsidering approach taken for UTRA

    - For bands in upper FR1 and/or for new 6G bands only.

  - Protection of the BS receiver of own or different BS

  - OBUE for wider channel BW

  - Time alignment error

- 2nd priority:

  - BS output power

  - RE power control dynamic range

  - Total power dynamic range

  - Transmit ON/OFF, only for new 6G bands

  - ACLR absolute limit clarification

The 1st priority means the technical discussion should start from next meeting, focus will be on these 1st priority topics.

The 2nd priority means tdoc could be submitted from next meeting but the discussion will only happen if there is time left during the meetings.

## Requirements for cmWave bands

### Issue 1-2-1: Conducted requirements and FR2 like methodology

Agreement:

For bands in the 5-8 GHz:

- Conducted and OTA requirements will be studied.

- FFS if similar approach as FR2 could be used (e.g. single declared sensitivity).

### Issue 1-2-2: RF requirements to be studied for bands above 7 GHz

Way Forward:

The following list of requirements for bands above 7 GHz would be considered:

- If RAN4 agrees to redo coexistence study for ~7GHz:

    - ACLR/ACS

    - BS Rx requirements based on coexistence results, including ICS.

    - Dynamic range

- For further discussion:

    - RE power control dynamic range

    - Transmit On/Off power requirements

## Coexistence studies

### Issue 2-1-1: Coexistence studies

Way Forward:

Scenario: 7 GHz, Urban Macro, PC2 UE (FFS if PC3 and/or PC1.5 or PC1 will be considered)

- If RAN4 agrees to study the coexistence, ~7GHz including n104 should be prioritized.

- Regarding less than 1GHz, the discussion in this thread can be put on hold until the decision in coming RAN#110.

Companies proposing to redo coexistence studies should:

  - Identify the key assumptions differences (comparing to the assumptions and works done in TR 38.921 for 6.425-7.125GHz and TR 38.922 for 7.125 – 8.4 GHz).

  - Identify the potential impacts on the conclusions of previous coexistence studies (e.g. how much ACLR/ACS difference is expected).

  - Identify the next steps (update existing requirements? New requirements/new type? Regulation impacts?).

Other scenarios (e.g. Urban micro, Indoor hotspot, Dense urban, …) are not precluded but companies should then justify why the proposed scenario(s) would be more stringent comparing to Urban Macro, impacting the coexistence study results.

## NTN aspects

### Issue 3-1-3: NTN-NTN RF coexistence scenarios

Agreement:

1st priority:

  - Time Alignment Error

  - Frequency error based on non-ideal feeder link, …

  - Modulation quality based on real world experience from SAN development, SAN PA models + EVM improvement

  - Total power dynamic range, based on evolved SAN architecture, PA model, waveform

2nd priority (will be discussed only if time left in next meetings):

  - RE power control dynamic range, based on waveform and PA model

  - Additional spurious coexistence (NTN/NTN coexistence)

  - Receiver sensitivity level based on new channel models

  - Dynamic range based on new channel models

  - In band blocking for corner SAN – SAN cases

  - Out of band blocking for corner SAN – SAN cases

## Issue 3-2-1: SAN RF requirements to be studied

Agreement:

<to be added>

# Annex A – Current scope of the 6G study on BS RF and coexistence (status after RAN4#117)

## BS type 1-H enhancement  (RAN4#116bis)

BS Hybrid beamforming type of architecture shall be studied.

## BS RF requirements to be studied (RAN4#117)

The following BS RF requirements will be studied during the 6G SI:

- 1st priority:

  - EVM, detailed scope to be further discussed:

    - For new 6G modulation

    - Study + measurement methodologies

    - More pragmatic EVM requirement framework (e.g., MIMO layers dependent framework, other considerations are not precluded)

  - In-band and Out of band blocking

    - Reconsidering approach taken for UTRA

    - For bands in upper FR1 and/or for new 6G bands only.

  - ACLR absolute limit clarification

  - OBUE for wider channel BW

  - Time alignment error

- 2nd priority:

  - BS output power

  - RE power control dynamic range

  - Total power dynamic range

  - Transmit On/Off, only for new 6G bands

  - Protection of the BS receiver of own or different BS

## SAN RF requirements to be studied (RAN4#117)

1st priority:

  - Time Alignment Error

  - Frequency error based on non-ideal feeder link, …

  - Modulation quality based on real world experience from SAN development, SAN PA models + EVM improvement

  - Total power dynamic range, based on evolved SAN architecture, PA model, waveform

2nd priority (will be discussed only if time left in next meetings):

  - RE power control dynamic range, based on waveform and PA model

  - Additional spurious coexistence (NTN/NTN coexistence)

  - Receiver sensitivity level based on new channel models

  - Dynamic range based on new channel models

  - In band blocking for corner SAN – SAN cases

  - Out of band blocking for corner SAN – SAN cases

## NTN Coexistence (RAN4#117)