3GPP TSG-RAN WG4 Meeting #117                                                              R4-2522366
Dallas, USA, 17th – 21st November, 2025

Agenda item:			8.1

Source:	Samsung

Title:	WF on [117][107] 6G AI

Document for:	Approval

# Topic #1: RAN4 AI/ML framework and other general aspects

#### Issue 1-1: Post-deployment enhancement

Tentative Agreement:

- The following clarification is provided to align companies’ understanding on pre-deployment conformance and post-deployment enhancement options.

|  | Ways to guarantee AI/ML performance | When to perform | Where to perform | Comment |
| --- | --- | --- | --- | --- |
| (1) | Pre-deployment conformance test | Before cell-phone shipped into market | Testing lab for conformance testing | Same as existing conformance testing |
| (2) | Post-deployment pre-activation functionality test (Option 1 in Rel-19 discussion) | After cell-phone shipped into market, but before new AI/ML functionality activated | FFS in UE vendors’ lab or testing lab | Similar as product testing |
| (3) | Post-deployment post-activation functionality testing based on performance monitoring (Option 2 in Rel-19 discussion) | After new AI/ML functionality activated | In-field practical network environment | No yet introduced in Rel-19. |

- FFS the following options for post-deployment enhancement in 6G study:

  - Post-deployment pre-activation functionality test (Option 1 in Rel-19 discussion)

  - Post-deployment post-activation functionality testing based on performance monitoring (Option 2 in Rel-19 discussion)

#### Issue 1-2: Model standardization and deployment

Tentative Agreement:

- For model complexity,

  - RAN4 further study AI model complexities, by considering: number of model parameters, model computing complexity, model size, data requirement, inference latency, and model generalization.

  - FFS the following example to differentiate “Simple” and “Complex” models:

|  | "Simple model” | "Complex model" |
| --- | --- | --- |
| Num of model parameter | Low, e.g., <1M | High, e.g., 1M ~10M |
| Model computing complexity | Low, e.g., <1M FLOPs, or [1M FLOPs ~10M FLOPs] | High, e.g., [1M FLOPs ~10M FLOPs], or 10M FLOPs ~100M FLOPs |
| Model size | Small, e.g., <1MB | Large, e.g., 1MB ~10MB |
| [Inference latency] | Low | High |
| [Training data requirement] | Low | High |
| [Generalization] | Sufficient for simple tasks | High potential for complex tasks |

### Issue 1-5: Interoperability for one-sided and two-sided models

Tentative Agreement:

- Interoperability for one-sided model,

  - RAN4 to study interoperability issues arising from the deployment of UE sided AI/ML models for agreed functionalities in 6G.

- Interoperability for two-sided model,

  - No more discussion in 6G study, while the to-be-specified mechanism for CSI compression in Rel-20 5G-A WI can be used to address interoperability issues in AI/ML based 6G features that require two-side model deployment.

# Topic #2: AI/ML use cases for RF issues (Per-use case discussion)

## 2.1 Use Case #1: AI-nonlinearity compensation

#### Issue 2-1: AI-nonlinearity compensation (sub-)use case selection/prioritization

Agreement:

- RAN4 will study the following 6G AI use cases

- Case 1.1: AI-based DPoD+no DPD

- Case 1.2: AI-based DPoD+non-AI DPD

- Case 2: no DPoD+AI based DPD

- benchmark: 1) non-AI DPD only 2)non-AI DPoD only

  - The interested companies can choose to compare with no DPD/DPoD and non-AI DPD(UE)+non-AI DPoD(BS)

- Training: TBD

- Evaluation metric: TBD

#### Issue 2-3: AI-DPoD in gNB – key information for use case description

For information:

|  |  | Ericsson | OPPO | Samsung | ZTE |
| --- | --- | --- | --- | --- | --- |
| AI model input | Input in training | The model was trained using labeled IQ samples obtained after the IFFT in DFT-s-OFDM | Received signal after channel estimation and equalization | The estimated RS signal time domain sample with 1st, 3rd terms | The estimated data signal time domain sample with 1st, 3rd, 5rd terms |
|  | Input in inference | The estimated IQ samples after IFFT in DFT-s-OFDM | Received signal after channel estimation and equalization | The estimated data signal time domain sample with 1st, 3rd terms | The estimated data signal time domain sample with 1st, 3rd, 5rd terms |
| AI model output | Label in training (if applicable) | Known transmitted coded bits available at the receiver side | Received signal before demodulation [or before channel decoding] | original/known RS signal | original/known data signal |
|  | Output in inference | Soft bits | Received signal before demodulation [or before channel decoding] | data signal with non-linear distortion compensated | data signal with non-linear distortion compensated |
| Assumption on training | Training type | Offline Training | offline training [FFS online training, tuning] | Online training/finetune | offline training |
|  | Label construction (if applicable) | Based on transmitted bits, which can be obtained at the receiver by relying on - bits decoded after successful cyclic redundancy check (CRCs), - transmission of bit sequences known to both transmitter and receiver. | May need data collection for offline training [FFS online training, tuning] | DMRS with 1st, 3rd terms | Data signal time domain with 1st, 3rd, 5rd terms |
| Model location for inference |  | NW-sided model | NW-sided model | NW-sided model | NW-sided model |
| Collaboration/interaction between UE and NW |  | The NW may request the UE to - adjust PA back-off associated with relaxed EVM requirements, - generate and provide data to support training and refinement of AI/ML models at the NW side, - provide measurement reports and receive updated configuration from the NW. | For inference, no collaboration/interaction For training, collaboration/interaction may be needed for date collection | UE capability and network indication could be required | N/A |
| Evaluation methodology |  | Link level simulation | Link level simulation | Link level simulation | Link level simulation |
| Evaluation assumption |  | See R4-2521210 | See R4-2521243 | See R4-2520373 | See R4-2521537 |
| Evaluation KPI |  | BLER, throughput | BLER gain @ EVM(3.5%, 8%) | BLER, MPR, EVM, throughput | BLER, MPR, EVM, throughput |
| Evaluation benchmark |  | Legacy receiver without DPoD with PA distortion Legacy receiver without DPoD without PA (upper bound performance) Receiver with AI-DPoD | Conventional receiver (without non-linearity compensation) | Legacy receiver without DPoD Receiver with non-AI based DPoD, GMP-NC | Legacy receiver without DPoD |
| Preliminary evaluation results |  | Performance benefit: 100% link-level throughput gain at mid/high SNR levels at BLER = 10-1 | NW DPoD achieve a performance gain of approximately 1 dB at a BLER of 10% with 3.5% EVM. When EVM increases to 8%, the proposed AI-DPoD schemes still achieve 10% BLER at an SNR of 22.2 dB, whereas the benchmark is unable to achieve 10% BLER even at higher SNR levels. | Performance benefit: AI-NC provides 1-2dB improvement at 10% BLER when using 256QAM, 1024QAM and 4096QAM for the given EVM values. | Tx EVM can be relaxed to 4.04% |
| High level potential specification impact |  | - Adjusted in-band distortion (e.g. EVM) and impact to other requirements (e.g. MPR, A-MPR), - UE capability exchange, - Data collection using know bit sequences generated at the UE side by the mean of a shared seed between the UE and the BS, under different hardware operating conditions. | EVM/[MPR] relaxing in RAN4 requirement | RAN4 requirements, e.g, EVM, MPR reporting Signalling/procedure related to LCM for NW-sided model | UE capability report on supported MPR relaxing RAN4 measurement requirement on MPR delta |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Complexity: 100 MFLOPs | Model: AI DPoD + AI Demod FLOPs: 86.4M + 11.1M Trainable parameters: 44K | Low complexity neural network for online tuning | Complexity: FCN is used: Number of input neurons is 30 Number of output neurons is 2 Number of hidden layers is 4 Number of neurons in each hidden layer is 64 FLOPs: 0.59G, Params: 0.01M Performance benefit: AI-NC provides 2.5dB MPR improvement when using 256QAM for the given SNR value(SNR=16). |

#### Issue 2-4: AI-DPoD in gNB – evaluation related proposals

Agreement:

- For evaluation procedure Step-2 (Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation)

  - Analyze on the feasibility for both online and offline training

    - FFS different data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region: e.g., simulation-based datasets (with realistic Tx front-end and PA models) and measurement-based datasets (from multiple UEs).

  - For a given Tx EVM value derived based on the non-linearity models, evaluate the performance with following assumptions on selected RAN4 test cases to find the target SNR to meet RAN4 requirement or test metric, e.g., 70% of maximum throughput or 10% BLER

    - Case 1.1: AI-based DPoD + non-linearity TX model(s) (no DPD)

    - Case 1.2: AI-based DPoD + non-linearity TX model(s) (non-AI DPD)

    - Case 1.3: non-AI-based DPoD + non-linearity TX model(s) (no DPD)

    - Case 1.4: non-AI-based DPoD + non-linearity TX model(s) (non-AI DPD)

    - Benchmark

      - Benchmark 1): No DPoD + non-linearity TX model (non-AI DPD)

      - Additional benchmark: No DPoD + non-linearity TX model (no DPD)

    - FFS which RAN4 use case should be used.

    - FFS the possibility to align non-AI and AI-based DPoD

    - FFS how to proceed the work before non-linearity model is not agreed in other sessions.

      - FFS how to proceed if there are multiple non-linearity models from 6G system parameter and UE RF sessions.

    - FFS TxEVM values

    - Detailed RF transmission scheme and configuration:

      - Use 5G NR existing supported scheme and configuration as starting point, and will be finally determined based on the conclusion from 6G system parameter and UE RF sessions

#### Issue 2-5: AI-DPD in UE – key information for use case description

For information:

|  |  | vivo |  |  | Nokia |
| --- | --- | --- | --- | --- | --- |
|  |  | Direction 1 | Direction 2 | Direction 3 |  |
| AI model input | Input in training | Time domain samples after PA distortion and PA config e.g. Vcc etc. |  | Time domain samples before PA distortion and PA config e.g. Vcc etc. | Time domain samples before pre-distortion |
|  | Input in inference | Time domain samples before PA distortion and PA config e.g. Vcc etc. | Time domain samples after PA distortion and PA config e.g. Vcc etc. | Time domain samples before PA distortion and PA config e.g. Vcc etc. | Time domain samples after pre-distortion |
| AI model output | Label in training (if applicable) | Time domain samples w/o distortion |  | Time domain samples after PA distortion | Time domain samples |
|  | Output in inference | Time domain samples after pre-distortion | LUT coefficients | Time domain samples after PA distortion |  |
| Assumption on training | Training type | Offline training；Online finetune |  |  | Offline training Online training/finetune2 |
|  | Label construction (if applicable) |  |  |  |  |
| model location for inference |  | UE-sided |  |  | UE-sided model |
| Collaboration/interaction between UE and NW |  | As for UE side model |  |  | Similar to UE-sided model as NR |
| Evaluation methodology |  | link level simulation |  |  |  |
| Evaluation assumption |  | See R4-2521440 |  |  |  |
| Evaluation KPI |  | BLER, EVM, MPR |  |  | BLER, EVM, MPR |
| Evaluation benchmark |  | No DPD |  |  | No DPD |
| preliminary evaluation results |  | See 2.1.1.2 in R4-2521440 |  |  |  |
| High level potential specification impact |  | See 2.1.1.3 in R4-2521440 and summarized as below: 1. RAN4 requirements, e.g. MPR, power boosting 2. SRS and/or PUSCH enhancement/Tx power determination for data collection 3. Signaling/ procedure related to LCM for UE-sided model |  |  | 1. RAN4 requirements, e.g. EVM 2. Tx power determination 3. Signaling/ procedure related to LCM for UE-sided model |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Inference complexity |  |  |  |
|  |  | Depend on AI model complexity | similar as LUT-based DPD | similar as LUT-based DPD |  |

#### Issue 2-6: AI-DPD in UE – evaluation related proposals

Agreement:

- Evaluation methodology:

  - For a given non-linearity models, evaluate the performance with following assumptions to find the target output power (after applying AI-based DPD) to meet RAN4 Tx requirements, including ACLR/SEM/IBE/EVM

    - FFS how to perform the evaluation on the impact of remaining non-linearity after AI-DPD.

    - Case 2: No DPoD + non-linearity TX model (AI-based DPD)

    - Benchmark

      - Benchmark 1): No DPoD + non-linearity TX model (non-AI DPD)

      - Additional benchmark: No DPoD + non-linearity TX model (no DPD)

    - FFS how to proceed the work before non-linearity model is not agreed in other session.

      - Strive to have aligned model as DPoD evaluation.

      - FFS how to proceed if there are multiple non-linearity models from 6G system parameter and UE RF sessions.

    - FFS Tx requirements values

    - Detailed RF transmission scheme and configuration:

      - Use 5G NR existing supported scheme and configuration as starting point, and will be finally determined based on the conclusion from 6G system parameter and UE RF sessions

# Topic #3: AI/ML use cases for Demod issues (Per-use case discussion)

## 3.1 Use Case #2: AI-SRS-assisted channel reconstruction

#### Issue 3-1: AI-SRS-assisted channel reconstruction (sub-)use case selection/prioritization

Tentative Agreement:

- For AI-SRS-assisted channel reconstruction:

  - RAN4 continue the discussion on AI-assisted SRS power imbalance compensation.

  - RAN4 hold the discussion on other AI-SRS-assisted channel reconstruction use cases which can be considered in RAN1 6G study.

#### Issue 3-2: AI-SRS-assisted channel reconstruction – key information for use case description

For information:

|  |  | Huawei (AI-assisted SRS power imbalance compensation) |
| --- | --- | --- |
| AI model input | Input in training | UL measured channel matrix from SRS with residual power imbalance |
|  | Input in inference | UL measured channel matrix from SRS with residual power imbalance |
| AI model output | Label in training (if applicable) | CSI measurement without power imbalance, candidate solutions could be: - CSI feedback based on CSI-RS - SRS measurements that are processed via time domain filtering to achieve high enough SNR - SRS measurements with perfect pre-compensation at UE |
|  | Output in inference | Ideal DL channel matrix |
| Assumption on training | Training type | Offline training |
|  | Label construction (if applicable) | Dataset construction: A total of 100,000 TTIs (Transmission Time Intervals) are allocated as follows: - 70,000 TTIs: Only S slots are used for model training; - 10,000 TTIs: Both S slots and D slots are used for model validation; - 20,000 TTIs: Both S slots and D slots are used for model testing. |
| model location for inference |  | NW-sided AI |
| Collaboration/interaction between UE and NW |  | 1. Inference: UE reporting on the range of SRS residual power imbalance for model training and model selection 2. Data collection/monitoring: UE reporting/assistance information for constructing labels, e.g., CSI feedback based on CSI-RS or SRS measurements with perfect pre-compensation |
| Evaluation methodology |  | Link level simulation |
| Evaluation assumption |  | Parameters; Values. Carrier frequency/BW; 5GHz/100MHz/272RBs. CSI-RS ports; 32. Tx/Rx number @UE; 1T4R. Channel model; TDL-A XPL medium. Doppler; 1Hz. TDD ULDL pattern; DDDDDDDSUU. Power imbalance distribution; [0, -2.5, -5, -7.5] dB. [0, -2.5, -5, -7.5]±1dB. SRS pattern; 4 frequency hopping, 40ms between two SRS hopping. A common AI model is trained which applies to each channel matrix extracted from 4RBs. The model input/output is the channel matrix with the dimension [M×N], where M denotes the number of CSI-RS ports and N denotes the number of UE Rx. Channel aging issue is not considered in this use case. |
| Evaluation KPI |  | SGCS Steps for SGCS calculation during model testing - First, obtain full-bandwidth ideal channel matrix for D slot - Then perform SVD decomposition for each subband to derive eigenvectors - Subsequently, find the full-bandwidth channel matrix constructed from AI-recovered CSI at the S slots closest to the D slot, and also perform SVD decomposition for each subband to derive its eigenvectors - Finally, compare these eigenvectors (from the AI-recovered CSI) with the eigenvectors obtained from the ideal channel matrices to calculate SGCS |
| Evaluation benchmark |  | 1) SRS measurements without residual power imbalance 2) non-AI based SRS residual power imbalance compensation |
| preliminary evaluation results |  | •AI/ML based SRS residual power imbalance compensation outperforms non-AI/ML based compensation across the entire SNR range, with more obvious performance gains observed at medium to low SNRs. •AI/ML is capable of compensating without actual values reporting, meaning there is no need to report updates to the SRS residual power imbalance when changes fall within reasonable range. |
| High level potential specification impact |  | 1) UE assistance information/reporting related to SRS residual power imbalance 2) Potential UE RF related testing |
| Feasibility issues, including complexity, and other aspects related to implementation |  | CNN-based AI model |

#### Issue 3-3: AI-SRS-assisted channel reconstruction – evaluation related proposals

Tentative Agreement:

- For AI-assisted SRS power imbalance compensation,

  - FFS assumptions on simulation parameters, including channel models, number of UE antennas, frequency band and SRS residual power imbalance distributions are determined to be used in the evaluation.

  - FFS evaluation methodology

## 3.2 Use Case #3: AI-enabled PRACH receiver

#### Issue 3-4: AI-enabled PRACH receiver use case selection/prioritization

Tentative Agreement:

- For AI-enabled PRACH receiver:

  - RAN4 hold the discussion on AI-enabled PRACH receiver which can be considered in RAN1 6G study.

# Topic #4: AI/ML use case for RRM (Per-use case discussion)

#### Issue 4-1: AI-RRM (sub-)use case selection/prioritization

For information:

- AI-RRM prediction use cases are summarized in Issue 4-2.

- Other AI-RRM use case proposals are summarized as below:

| Index | use cases | Model location (Training method) | Positive for the sub-use case to be included in 6G study with priority | Positive for the use case to be included in 6G study after the prioritized sub-use cases are concluded | Expected benefits (e.g., RS reduction, MG reduction etc.) | Dependence with other WG-led use cases (in 5GA or 6G) | Which specific aspect RAN4 should study |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Multi-Domain L3 level Prediction for Measurement-Gap Reduction | UE sided model (Offline training) | CMCC (non-collocated), Samsung*, LGE, vivo, CTC, CATT |  | MG reduction | Rel-20 AI-Mob | Identify which use cases can serve the purpose of measurement gap reduction |
| 5a | Dynamic Adaptation of Measurement Procedure | UE sided model | Qualcomm*, MediaTek*, Samsung, Xiaomi (AI cell meas. selection), LGE, vivo*, CATT |  | - Measurement reduction - Improved mobility: HO/LTM | Prediction performance: Rel-20 AI-Mob | How to define UE measurement requirements in a way that AI/ML based UE measurement adaptation can be spec-compliant. |
| 7a | Spatial domain RX beam sweeping reduction (L1/L3) | UE sided model | CMCC, Samsung, Qualcomm*, Xiaomi, LGE, vivo*, CTC, Nokia |  | Measurement delay reduction | Minor dependence on other WG-led use cases. FR2 RX beam sweep factor is transparent to RAN1/RAN2 spec. | Benefits and feasibility of the use of AI-ML to reduce spatial domain RX beam sweeping factor |
|  | AI/ML based RLM/BFD/CBD, LTM by measurement prediction | UE-sided model | Samsung, LGE, vivo |  |  |  |  |
|  | AI/ML based SCell related enhancement by measurement prediction | UE-sided model | OPPO |  |  |  |  |
|  | AI-based prediction for enhanced s-measure mechanism | UE-sided model | LGE |  |  |  |  |
|  | AI/ML based AGC through temporal/frequency domain prediction | UE-sided model | ZTE* |  |  |  |  |
|  | AI/ML based DL sync through temporal/frequency domain prediction | UE-sided model | ZTE* |  |  |  |  |
|  | AI abstraction of the wireless environment to facilitate data scheduling and network power saving | NW-sided model | CATT |  | facilitate data scheduling and network power saving |  |  |

#### Issue 4-2: General view on how to proceed for AI-RRM prediction models

For information:

- AI-RRM prediction (sub)-use cases are summarized as below.

| use case | Sub use cases | Positive for the sub-use case to be included in 6G study with priority | Positive for the use case to be included in 6G study after the prioritized sub-use cases are concluded | Dependence with other WG-led use cases (in 5GA or 6G) | Which specific aspect RAN4 should study |
| --- | --- | --- | --- | --- | --- |
| 1) L1 beam-level prediction for TX beam | (1a) FR1/FR2-1 spatial domain (intra-cell) |  |  | FR2-1 is already in Rel-19 AI-BM (RAN1), FR1 not |  |
|  | (1b) FR1/FR2-1 time domain (intra-cell) |  |  |  |  |
|  | (1c) FR1/FR2-1 spatial + time domain (intra-cell) |  |  | FR2-1 is already in Rel-19 AI-BM (RAN1), FR1 not |  |
|  | (1d) FR1/FR2-1 spatial domain (inter-cell, co/non-collocated) |  |  |  |  |
|  | (1e) FR1/FR2-1 freq. + spatial domain (inter-cell, co/non-collocated) |  |  | [RAN1 agree to be studied in 6G study] |  |
|  | (1f) FR1/FR2-1 time + spatial domain (inter-cell, co/non-collocated) |  |  |  |  |
| 2) L3 beam-level prediction for TX beam | (2a) FR1/FR2-1 spatial domain (intra-cell) | Nokia, vivo, Apple, OPPO (for both FR1 and FR2-1, FFS RAN2 or RAN4 driven) |  | Already in Rel-19 AI-Mobility study (RAN2) but not enter into Rel-20 work item. | Nokia: Impact on prediction accuracy (accuracy requirements), reporting delay requirements. Impacts on ML Management and performance monitoring should also be considered. |
|  | (2b) FR1/FR2-1 time domain (intra-cell) |  |  | Already in Rel-19 AI-Mobility study (RAN2) but not enter into Rel-20 work item. |  |
|  | (2c) FR1/FR2-1 spatial + time domain (intra-cell) | Nokia, vivo, OPPO (for both FR1 and FR2-1, FFS RAN2 or RAN4 driven) | Apple |  | Nokia: Impact on prediction accuracy (accuracy requirements), reporting delay requirements. Impacts on ML Management and performance monitoring should also be considered. |
|  | (2d) FR1/FR2-1 frequency domain (inter-cell, collocated) | Apple, OPPO (FFS RAN2 or RAN4 driven) |  | Already in Rel-19 AI-Mobility study (RAN2) but not enter into Rel-20 work item. |  |
|  | (2e) FR1/FR2-1 spatial domain (inter-cell, co/non-collocated) | Nokia, Apple (FFS RAN2 or RAN4 driven) |  |  | Nokia: Impact on prediction accuracy (accuracy requirements), reporting delay requirements. Impacts on ML Management and performance monitoring should also be considered. |
|  | (2f) FR1/FR2-1 freq. + spatial domain (inter-cell, non-collocated) | Nokia, OPPO (FFS RAN2 or RAN4 driven) | Apple |  | Nokia: Impact on prediction accuracy (accuracy requirements), reporting delay requirements and measurement gaps configuration and scheduling restrictions. Impacts on ML Management and performance monitoring should also be considered. |
|  | (2g) FR1/FR2-1 time + spatial domain (inter-cell, co/non-collocated) | Nokia, OPPO (FFS RAN2 or RAN4 driven) | Apple |  | Nokia: Impact on prediction accuracy (accuracy requirements), reporting delay requirements. Impacts on ML Management and performance monitoring should also be considered. |
|  | (2h) FR1/FR2-1 time + freq. + spatial domain (inter-cell, co/non-collocated) |  |  |  |  |
| 3) L3 cell-level prediction | (3a) FR1/FR2-1 time domain (intra-cell) |  |  | Already in Rel-20 AI-Mobility (RAN2) Need clarification for FR2-1 beam level measurement can be used for model input |  |
|  | (3b) FR1/FR2-1 freq. domain (inter-cell, collocated) |  |  | Already in Rel-20 AI-Mobility (RAN2) Need clarification for FR2-1 beam level measurement can be used for model input |  |
|  | (3c) FR1/FR2-1 freq. domain (inter-cell, non-collocated) | Nokia, LGE, CMCC, Apple, Samsung, ZTE (FFS RAN2 or RAN4 driven) |  |  | Nokia: Impact on prediction accuracy (accuracy requirements), reporting delay requirements and measurement gaps configuration and scheduling restrictions. |

## Reference for RAN4#117 (Nov. 2025, Dallas)

R4-2520371	Running Summary of 6G AI										Samsung

R4-2521396	Views on 6G AI														NTT DOCOMO, INC.

R4-2520043	Further comments on the RAN4-driven AI/ML framework of 6GR	Qualcomm Technologies Ireland

R4-2520153	Discussion on general parts of RAN4 driven AI use cases		CATT

R4-2520331	Discussion on general aspects for RAN4 6G AI					Huawei, HiSilicon

R4-2520372	Discussion on AI/ML general aspects for 6GR					Samsung

R4-2520442	Discussion on general aspects for 6G AI/ML						CMCC

R4-2520625	Discussion on 6G AI/ML Framework and General Issues		Apple

R4-2520710	6G AI/ML General Aspects											Nokia

R4-2521209	Discussion on general issues related to AI/ML for 6G study		Ericsson

R4-2521242	General issues for AI/ML in RAN4									OPPO

R4-2521439	General aspect on 6G AI											vivo

R4-2521536	Discussion on 6G AI general part									ZTECorporation,Sanechips

R4-2520044	Further comments on RAN4-driven AI/ML use cases of 6GR	Qualcomm Technologies Ireland

R4-2520154	Discussion on RAN4 driven AI use cases							CATT

R4-2520332	Discussion on RAN4-driven AI/ML use cases						Huawei, HiSilicon

R4-2520345	Views on 6G AI/ML													MediaTek Inc.

R4-2520373	Discussion on RAN4-driven AI/ML use cases for 6GR			Samsung

R4-2520443	Discussion on use cases for 6G AI/ML								CMCC

R4-2520492	Overview for 6GR RAN4 centric AI use case						Xiaomi

R4-2520624	Discussion on AI/ML-Based 6G Use Cases						Apple

R4-2520711	AI/ML use cases in 6G												Nokia

R4-2520721	Discussion of RAN4-driven AI/ML use cases for 6G AI			LG Electronics Inc.

R4-2521047	Discussion on 6G AI/ML study for RAN4 RRM driven use cases	China Telecom

R4-2521210	Discussion on AI/ML use cases for 6G study						Ericsson

R4-2521243	Use cases for AI/ML in RAN4										OPPO

R4-2521440	Initial discussion on 6G AI for RAN4 driven cases				vivo

R4-2521537	Views on AI study in 6G RAN4-driven use cases					ZTECorporation,Sanechips