Simulation results on KPI-1 and KPI-2

Simulated scenario:

- Scenario 1: Wide beam (SSB based) to predict narrow beam (CSI-RS based)

- Scenario 2: Narrow beam (SSB based) to predict narrow beam (CSI-RS based)

Background:

Agreement in RAN4#116bis - Regarding the metric for beam ID only prediction, where top-K predicted beam(s) are reported, RAN4 only specifies the requirements for the following scenario(s): - K=1 - In this case, the ground truth RSRP of the predicted beam is larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB Agreement in RAN4#114 Issue 2-1: Metrics/KPIs for beam ID prediction Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB,

KPI 1: Beam prediction accuracy

The successful rate for the correct prediction which is considered as maximum ground truth RSRP among top-K predicted beams is larger than the ground truth RSRP of the strongest genie-aided beam – x dB, where K=1,2,3,4,5, X= 1, 2, 3dB

Scenario 1: Wide beam (SSB based) to predict narrow beam (CSI-RS based)

Scenario 2: narrow beam (SSB based) to predict narrow beam (CSI-RS based)

KPI 2: RSRP accuracy(dB)

90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ground truth L1-RSRP of the same beams

Scenario 1: Wide beam (SSB based) to predict narrow beam (CSI-RS based)

Scenario 2: narrow beam (SSB based) to predict narrow beam (CSI-RS based)

Whether/How to define requirement based on simulation results

Case 1: Beam ID only prediction (K=1 only)

Based on the aforementioned simulation results with a very large gap/difference (span is over 20%), we recommend

- Option 1: Not to define accuracy requirement for Case 1

- Option 2: To define accuracy requirements with the exclusion of results with significant gaps/differences from companies

  - Further, RAN4 to discuss and decide

    - X value (e.g., RAN4 to define requirements based on K=1, X=3)

    - the criteria for how to exclude results with significant gaps/differences

    - How to define accuracy requirements (e.g., averaging the results of various companies after excluding results with significant differences)

Case 2: both Beam ID and RSRP prediction

we recommend RAN4 to define accuracy requirements with the exclusion of results with significant gaps/differences from companies.

  - Further, RAN4 to discuss and decide

    - K and X value

Recommendation: RAN4 to define requirements on

      - K=1, X=3

      - K=2, X=3

      - K=4, X=2

    - the criteria for how to exclude results with significant gaps/differences

    - How to define accuracy requirements (e.g., averaging the results of various companies after excluding results with significant differences)