3GPP TSG-RAN WG4 Meeting #117                							  R4-2522430

Dallas, TX, USA, November 17-21, 2025

Agenda item:			6.11.1

Source: 	Qualcomm Incorporated

Title: 	WF on [117][112] R19 AI for air interface

Document for:	Approval

# Introduction

This WF captures the agreements for the discussion carried out on AI/ML under the [117][112] [117][112] R19 AI for air interface.

# Agreements

## CSI reporting requirement framework for CSI prediction

### Agreements in main session:

Issue 1-2: Averaging ofγ

Agreement: Averaging ofγwill not be specified.

Issue 1-3: Parameters for the test cases

Agreement on Parameters for the test cases:

    - FDD 2Rx: 16 CSI-RS ports, TDLA30-20, MCS19

    - TDD 4Rx: 16 CSI-RS ports, TDLA30-20, MCS17

    - FFS: FDD 2Rx: 16 CSI-RS ports, TDLC300-20, MCS X (for generalization purpose)

Issue 1-4: Simulation results

Agreement:

Regarding CSI prediction, it is agreed to define the requirements based on the worst one, which is no worse than non-AI sample and hold, based one, in the current simulations without removing the outliers

## RRM core requirement and testing framework for beam management

### Agreements in main session:

Issue 2-1: Measurement period for inference – case 2

Agreement:

For BM-case 2 measurement period for inference, RAN4 only specifies the requirements for M=1.

- K: number of RS transmission occasions, which can be measured per Rx beam.

- M:CSI reference resource occasions

Issue 2-2: TCI State Switching Clarifications

Agreement

- RAN4 to confirm that the statement ‘TCI state remains detectable’ means side condition for detectable is met throughout the time period.

- The term ”predictable” won’t be used in the description of the side condition.

Issue 2-6: Value of x

Agreement:

      - Regarding the metric for beam ID only prediction, where top-1 predicted beam(s) are reported, RAN4 requirements are specified based on the Y% of probability that the ground truth RSRP of the predicted beam is larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – X dB

        - X and Y are TBD based on the simulations

Regarding the metric for RSRP+beam ID prediction, only RSRP accuracy requirement will be defined.

Issue 2-7: Channel model simplification

Agreement:

Take option 2 below as the baseline channel model simplification for Umi CDL-C at 28GHz and if no significant issue is identified on option 2,  the final agreement will be confirmed in RAN4#118.

  - The value of CASA in table 3 is FFS

  - Interested companies are encouraged to evaluate the option 2, including based on the methodology described in option 1.

Option 2: Simplification based on weak clusters

    - Consider the channel models described in Table 2 (Option A/B) and Table 3 (Option C) as candidates for CDL-based simplified channel models for multi-AoA testing of AI/ML BM.

- Table 2 – Channel model parameters for UMi CDL-C at 28 GHz with flat ZoA

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -134.4434 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 129.1633 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -152.8206 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | 164.1145 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 93.1719 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -112.0441 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 0 | 7 |  |

- Table 3 – Channel model parameters for UMi CDL-C at 28 GHz
 with AoA aligned to probe layout and no intra-cluster angle spread

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -114.436 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 125.5639 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -174.436 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | -174.436 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 95.5639 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -114.436 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | FFS | 0.5726 | 0 | 7 |  |

Issue 2-8:	Channel quality/performance evaluation

Agreement:

Evaluation of channel model suitability to be used in the tests:

Use reference (Table 7.2.2-3 from TR 38.827 )/simplified (defined as in table 2 in R4-2521418) CDL-C in agreed as baseline in Topic 2-7(include reference to table in R&S paper)

    - Step 1: Simulate performance (inference) with the reference CDL channel.

    - Step 2: Simulate performance (inference) with the simplified CDL channel.

    - Step 3: The prediction difference between the “results” of step 1 and step 2 in the above steps should meet some criteria.

- For beam-ID only prediction, the results are defined as

  - Regarding the metric for beam ID only prediction, where top-1 predicted beam(s) are reported, RAN4 requirements are specified based on the Y% of probability that the ground truth RSRP of the predicted beam is larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – X dB

    - Use same UE AI/ML model(same fully trained model with the same training data) for step 1 and step 2

Average over multiple UE orientations (one orientation means all data for a single inference is taken in a fixed orientation)

FFS how to average over different UE locations/Tx beams

FFS whether other parameters are needed

Issue 2-9:	Test cases/scenarios

Tests only for case 1

Test scenarios:

SSB to CSI-RS

CSI-RS to CSI-RS

Test case scenarios:

    - TCI state switch with predicted known conditions (CSI-RS to CSI-RS is applicable for UE supports FG59-1)

  - FFS: SSB to CSI-RS

    - Test L1-RSRP prediction reporting delay.

    - Test prediction accuracy based on measurement, and prediction covers the below options with respect to UE capability:

        - L1-RSRP prediction

        - Beam ID prediction

FFS on any other tests

FFS how many tests, testing time and coverage to also be considered

## RRM core requirement and testing framework for Positioning accuracy enhancement

### Agreements in main session

Issue 3-5: UE capabilities for inference time per frequency layer

Agreement: introduce the following new capability:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_pos_AIML | 58-x-x | Inference time per positioning frequency layer for AI-ML positioning case 1 | The maximum required inference time per configured positioning frequency layer for AI-ML positioning case 1 | 58-x-x | n/a | n/a | Candidate values: {1,2,3, 4, 5, 6, 7, 8, 9, 10, 15, 20} ms | Optional with capability signaling |

- Prerequisite FG info is needed

- The granularity of the feature is to be included.

Issue 3-6: UE capabilities for positioning

Agreement: introduce the following new capability:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_pos_AIML | 58-x-x | PRS measurement for reduced sample for AI-ML positioning case 1 | The capability to support reporting UE co-ordinate based on measuring on 1 or 2 samples of a DL PRS resource set in AI-ML positioning case 1 | 58-x-x | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| NR_pos_AIML | 58-x-x | Support of lower RX beam sweeping factor for AI-ML positioning case 1 | 1. Support of the lower RX beam sweeping factor than 8 for FR2 in AI-ML positioning case 1 2. Number of RX beam sweeping factor in AI-ML positioning case 1 | 58-x-x | n/a | n/a | Component 2: candidate values {1, 2, 4, 6} Need for location server to know if the feature is supported | Optional with capability signaling |
| NR_pos_AIML | 58-x-x | UE-RxTEGs for AI-ML positioning case 1 | 1. Support of UE-RxTEGs for UE-based AI-ML positioning case 1 2. The maximum number of UE-RxTEG, which is supported and reported by UE for UE-based AI-ML positioning case 1 | 58-x-x | n/a | n/a | Component 2 candidate values: {1, 2, 3, 4, 6, 8} Need for location server to know if the feature is supported | Optional with capability signaling |

- Prerequisite FG info is needed

- The granularity of the feature is to be included.

- Other missing info based on the UE feature list format is to be included

Issue 3-7: Test for case 1

Agreement:

  - Reuse the existing reporting delay tests for FR1 and FR2

    - Modify configuration such that UE is configured to report AI/ML based positioning

    - FR1: test configurations in Table A.6.6.12.1.1-1, general test parameters in Table A.6.6.12.1.1-2, and cell specific test parameters in Table A.6.6.12.1.1-3

      - TDL-A channel model

    - FR2: test configurations in Table A.7.6.9.1.1-1, general test parameters in Table A.7.6.9.1.1-2, and cell specific test parameters in Table A.7.6.9.1-3

      - TDL-C channel model

## Topic #4: General

### Agreements in main session

Issue 4-2:	Model update/activation

Agreement:

No R19 requirements will be specified in RAN4 for model update/activation