3GPP TSG-RAN WG4 Meeting #117		R4-2522278
Dallas, USA, Nov. 17-21, 2025

Agenda item:			4.1.1

Source:	Moderator (OPPO)

Title:	Topic summary for [117][113] R19_UERF_maintenance

Document for:	Information

Introduction

This is the summary for R19_UERF_maintenance under agenda 4.2 and except 4.2.7.

List of topics below:

- Discussion papers and corresponding CRs (20)

  - Draft CRs for 38.101-1 (3)

  - Draft CRs for 38.101-2 (7)

  - Draft CRs for 38.101-3 (1)

  - Draft CRs for 38.101-5 (2)

  - Draft CRs for 36.102 (1)

  - Draft CRs for 38.863 (1)

Topic #1: R19 spectrum related WI maintenance

Main technical topic overview. The structure can be done based on sub-agenda basis.

2Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2522072 | Sony | CR for TS 36.102 Rel-19 to clarify the guardband for FCC emission limit for b252 |
| R4-2520354 | China Unicom, Ericsson, Qualcomm, Huawei, NTT Docomo | CR on TS38.101-2 correction of UE RF requirements for EESS protection Removing square brackets for values under CA_NS_205 in clause 6.2A.3.6 |
| R4-2521286 | ZTE Corporation,Sanechips | Withdraw |
| R4-2521588 (R15) CAT-A: R4-2521287 R4-2521288 R4-2521289 R4-2521290 | ZTE Corporation,Sanechips | CR for 38.101-2 Correct the descriptions for BWintraCA and A-MPR NS_202 This formal CR is re-submission of the draft CR R4-2514002/4003/4004/4005/4006 which was endorsed in RAN4#116bis meeting. |
| R4-2520220 | Skyworks Solutions, Inc., Murata Manufacturing Co, Qualcomm France, T-Mobile USA | CR to 38.101-1 on Band n71 mandatory support of 25MHz CBW Refer to agreed WF R4-2514596 in RAN4 #116bis Including CA_n29A-n71A |
| R4-2520226 | Murata Manufacturing Co Ltd., Skyworks Solutions, Inc., Qualcomm France | CR 38.101-3 to change MSD for n41 10MHz CBW and remove redundant MSD due to IMD Refer to agreed WF R4-2514596 in RAN4 #116bis |
| R4-2520300 | Murata Manufacturing Co Ltd. | Observation 1: The Tx noise of n71 UL 25 MHz is 18.4 dB higher than that of UL 20 MHz due to the IM3 product of the Tx signal and IQ image falls into n29 DL. Proposal 1: RAN4 to consider the MSD value highlighted in Table 1 for PC3 CA_n29A n71A. Proposal 2: RAN4 to determine the PC2 MSD via the LUT of HPUE [1] once the PC3 MSD has been determined. Table 1. PC3 MSD for CA_n29A-n71A UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n29; 688; 20; 15; 20 (RBstart=86); 719.5; 5; 17.5; ACLR2. n71; n29; 685.5; 25; 15; 20 (RBstart=113); 719.5; 5; 35.8; ACLR1. |
| R4-2520568 | Apple | CR on TS38.101-5: Correction of the NTN NR band n250 UL configuration for REFSENS testing This formal CR is re-submission of the draft CR R4-2514627 which was endorsed in RAN4 #116bis |
| R4-2520569 | Apple, Qualcomm | CR on TR 38.863: Addition of power back-off simulation results for the NTN L-bands. This formal CR is re-submission of the draft CR R4-2513846 which was endorsed in RAN4 #116bis |
| R4-2520570 | Apple | CR on TS 38.101-5: Correction of the emission level definitions in NS_09N associated with NTN NR bands n250 and n253 |
| R4-2520583 | Apple | Proposal 1: Remove Note 12 from Table 5.3.5-1 for 25MHz CBW as highlighted in yellow in the table below: NR Band; SCS (kHz); UE Channel bandwidth (MHz). 3; 5; 10; 15; 20; 25; 30; 35; 40; 45; 50; 60; 70; 80; 90; 100. n71; 15; 5; 10; 15; 20; 2512; 3012; 3512. 30; 10; 15; 20; 2512; 3012; 3512. NOTE 12: This UE channel Bandwidth is optional for uplink in this release of the specification.. Proposal 2: Remove optional CBW REFSENS requirement and Note 10 from the Table 7.3.2-1a as highlighted in yellow in the table below: Operating band / SCS / Channel bandwidth. Operating Band; SCS kHz; 3 MHz (dBm); 5 MHz (dBm); 10 MHz (dBm); 15 MHz (dBm); 20 MHz (dBm); 25 MHz (dBm); 30 MHz (dBm); 35 MHz (dBm); 40 MHz (dBm); 45 MHz (dBm); 50 MHz (dBm). n71; 15; -97.2; -94.0; -91.6; -86.0; 84.19 -74.810; -82.59 -67.110; -80.79 -64.010. 30; -94.3; -91.9; -87.4; -84.29 -74.9; -82.69 -67.210; -80.89 -64.110. NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE and two Rx antenna port XR UEs indicating UE capability supportOf2RxXR-r18. Four Rx antenna ports for (e)RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 9: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 10: Applies to UEs that support optional symmetric UL/DL for this BW.. Proposal 3: Remove optional CBW REFSENS requirement and Note 3 from the Table 7.3.2-1c as highlighted in yellow in the table below: Operating Band; 3 MHz (dB); 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB); 25 MHz (dB); 30 MHz (dB); 35 MHz (dB); 40 MHz (dB); 45 MHz (dB); 50 MHz (dB). n71; 0.5; 0.9; 0.9; 2.2; 2.42 2.53; 2.52 2.43; 2.92 3.13. NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW.. Proposal 4: Remove optional CBW REFSENS requirement and Note 3 from the Table 7.3.2-1d as highlighted in yellow in the table below: Operating Band; 3 MHz (dB); 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB); 25 MHz (dB); 30 MHz (dB); 35 MHz (dB); 40 MHz (dB); 45 MHz (dB); 50 MHz (dB). n71; 1.1; 1.1; 1.7; 5.5; 5.92 6.93; 6.22 7.23; 6.52 7.33. NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW.. Proposal 5: Remove Note 6 from the Table 7.3.2-3 as highlighted in yellow in the table below: Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode. Operating Band; SCS; 3; 5; 10; 15; 20; 25; 30; 35; 40; 45; 50; 60; 70; 80; 90; 100; Duplex Mode. n71; 15; 25; 251; 201; 201; 201,6; 201,6; 201,6; FDD. 30; 121; 101; 101; 101,6; 101,6; 101,6. Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. Note 6: UEs supporting the optional symmetrical UL/DL bandwidths shall use this UL configuration. For UEs not supporting this uplink channel bandwidth, the UL configuration of the 20MHz UL channel bandwidth and the nominal Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used, i.e. FTX-RX as defined in clause 5.3.6 does not apply.. Proposal 6: Modify CA_n5A-n71A MSD with n71 UL in Table 7.3A.6-1 as highlighted below: UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n5; 688685.5; 2025; 15; 20 (RBstart=86); 871.5; 5; 2.0; >ACLR2. Proposal 7: In Table 7.3A.6-1 and for CA_n71-n85, we propose the changes highlighted in yellow for the Band n85 PC3 MSD due to Band n71 25MHz UL CBW cross-band isolation interference. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n85; 685.5; 25; 15; 20 (RBstart=113); 730.5; 5; 10.86; ACLR2. n71; n85; 680.5; 35; 15; 20 (Rbstart=168); 730.5; 5; 237; ACLR1. NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 7: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. Proposal 8: In Table 7.3A.6-1a-1 and for CA_n71-n85, we propose the changes highlighted in yellow for the Band n85 MSD due to Band n71 25MHz UL CBW PC2 cross-band isolation interference. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n85; 685.5; 25; 15; 20 (RBstart=113); 730.5; 5; 13.84,6 16.84,7; ACLR2. n71; n85; 680.5; 35; 15; 20 (Rbstart=168); 730.5; 5; 265,6 32.35,7; ACLR1. NOTE 4: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 5: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. NOTE 6: Applicable to UE’s supporting PC2 with 1Tx NOTE 7: Applicable to UE’s supporting PC2 with 2Tx. |
| R4-2520894 | Skyworks Solutions Inc. | Proposal 1: In Table 7.3A.6-1 and for CA_n29-n71, consider adopting the Band n29 PC3 changes highlighted in yellow for MSD due to cross-band isolation interference. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n29; 685.5; 25; 15; 20 (RBstart=113); 719.5; 5; 39.56,9; ACLR1. n71; n29; 688; 20; 15; 20 (RBstart=86); 719.5; 5; 17.59; ACLR2. NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 9: Not applicable to UEs indicating support of low NR band aggregation via switching supportedLowBandSwitching-r19 for this band combination. Proposal 2: In Table 7.3A.6-1a-1 and for CA_n29-n71, consider adopting the Band n29 PC2 MSD changes highlighted in yellow for MSD due to cross-band isolation interference. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n29; 685.5; 25; 15; 20 (RBstart=113); 719.5; 5; 42.56,8 45.57,8; ACLR1. n71; n29; 688; 20; 15; 20 (RBstart=86); 719.5; 5; 20.46,8 23.37,8; ACLR2. NOTE 4: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 5: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. NOTE 6: Applicable to UEs supporting PC2 with 1Tx NOTE 7: Applicable to UEs supporting PC2 with 2Tx NOTE 8: Not applicable to UEs indicating support of low NR band aggregation via switching supportedLowBandSwitching-r19 for this band combination. |
| R4-2521665 | Qualcomm | Proposal 1: Include the following PC3 MSD evaluation as part of averaging process. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n29; 685.5; 25; 15; 20 (RBstart=113); 719.5; 5; 37.89; ACLR1. |
| R4-2522167 | Rakuten Mobile, Inc, T-Mobile,Anterix,Southern Linc | CR to 38.101-1 Rel-19 for 3MHz Bandwidth change from Optional to Mandatory for n28,n26,n85,n100. |
| R4-2522169 | Rakuten Mobile, Inc, T-Mobile,Anterix,Southern Linc | Proposal 1: Change the 3 MHz CBW for Band n28,n26,n85,n100 from optional to mandatory in Release 19. This CBW to be excluded from MSD interference impact analysi |
| R4-2520219 | Skyworks Solutions, Inc., Murata Manufacturing Co, Qualcomm France, T-Mobile USA | CR to 38.101-1 to change MSD for band n41 10MHz CBW and to remove redundant dual-UL IMD MSD Refer to agreed WF R4-2514596 in RAN4 #116bis |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: n71 with UL CBW 25MHz

Sub-topic description

- Band n71:

  - Companies are invited to study how to mandate symmetrical UL 25MHz / DL 25MHz CBW for band n71 as mandatory support, in particular

    - Study removing Note 10 from the DL 25MHz REFSENS requirement,

    - Study the impact on PC3 CA_n5A-n71A and PC3 PC2 CA_n71A-n85A cross-band isolation MSD due to BCS4/5 new greatest CBW,

  - UL 30MHz and UL 35MHz remain optional CBW,

  - No change to the band n71 asymmetric channel bandwidths requirements, i.e.

    - No change to the UL 20MHz / DL 30MHz and UL 20MHz / DL 35MHz REFSENS

    - No change to the UL 30MHz / DL 30MHz and UL 35MHz / DL 35MHz REFSENS

    - No need to study UL 25MHz / DL 30MHz and UL 25MHz / DL 35MHz REFSENS

In RAN4 #116 meeting, it was agreed in WF R4-2511779

In RAN4 #116bis meeting, it was agreed in WF R4-2514596, the agreement included

- Modify n71 Channel Bandwidths, REFSENS, and UL RB allocation for REFSENS

- Modify MSD and related test configuration for CA_n5-n71 and CA_n71-n85

Issue 1-1-1: PC3 MSD for CA_n29-n71 with UL 25MHz CBW at n71

- Proposals

  - Proposal 1: RAN4 to consider the MSD value for PC3 CA_n29A-n71A (Murata)

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n29 | 688 | 20 | 15 | 20 (RBstart=86) | 719.5 | 5 | 17.5 | ACLR2 |
| n71 | n29 | 685.5 | 25 | 15 | 20 (RBstart=113) | 719.5 | 5 | 35.8 | ACLR1 |

  - Proposal 2: for PC3 CA_n29-n71, consider adopting the Band n29 PC3 changes highlighted in yellow for MSD due to cross-band isolation interference. (Skyworks)

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n29 | 685.5 | 25 | 15 | 20 (RBstart=113) | 719.5 | 5 | 39.56,9 | ACLR1 |
| n71 | n29 | 688 | 20 | 15 | 20 (RBstart=86) | 719.5 | 5 | 17.59 | ACLR2 |
| NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 9: Not applicable to UEs indicating support of low NR band aggregation via switching supportedLowBandSwitching-r19 for this band combination |  |  |  |  |  |  |  |  |  |

  - Proposal 3: PC3 MSD evaluation for CA_n29-n71 as part of averaging process. (Qualcomm)

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n29 | 685.5 | 25 | 15 | 20 (RBstart=113) | 719.5 | 5 | 37.89 | ACLR1 |

- Recommended WF

  - Discuss whether the average value can adopt

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD |  |  |  | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Murata | Skyworks | Qualcomm | Average |  |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) | (dB) | (dB) | (dB) |  |
| n71 | n29 | 685.5 | 25 | 15 | 20 (RBstart=113) | 719.5 | 5 | 35.8 | 39.56,9 | 37.89 | 37.7 | ACLR1 |

  - Check whether note 6 and/or note 9 are needed for CA_n29-n71

NOTE 6:	Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth

NOTE 9:	Not applicable to UEs indicating support of low NR band aggregation via switching supportedLowBandSwitching-r19 for this band combination

Issue 1-1-2: PC2 MSD for CA_n29-n71 with UL 25MHz CBW at n71

- Proposals

  - Proposal 1: PC2 MSD via the LUT of HPUE once the PC3 MSD has been determined (Murata)

  - Proposal 2: Consider adopting the Band n29 PC2 MSD changes highlighted in yellow for MSD due to cross-band isolation interference. (Skyworks)

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n29 | 685.5 | 25 | 15 | 20 (RBstart=113) | 719.5 | 5 | 42.56,8 45.57,8 | ACLR1 |
| n71 | n29 | 688 | 20 | 15 | 20 (RBstart=86) | 719.5 | 5 | 20.46,8 23.37,8 | ACLR2 |
| NOTE 4: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 5: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. NOTE 6: Applicable to UEs supporting PC2 with 1Tx NOTE 7: Applicable to UEs supporting PC2 with 2Tx NOTE 8: Not applicable to UEs indicating support of low NR band aggregation via switching supportedLowBandSwitching-r19 for this band combination |  |  |  |  |  |  |  |  |  |

- Recommended WF

  - PC2 MSD can decide via the LUT of HPUE.

Related CRs

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2520220 (R19 38.101-1) | Skyworks Solutions, Inc., Murata Manufacturing Co, Qualcomm France, T-Mobile USA |  |  |

Sub-topic 1-2: Change the 3 MHz CBW from optional to mandatory in Release 19

Sub-topic description

3MHz CBW is optional in current Spec.

Open issues and candidate options before meeting:

In RAN4#115, WF R4-2508050 agreed to further check whether 3MHz CBW should become mandatory for bands n26, n28, n31, n72, n85 and n100.

Issue 1-2-1: Change the 3 MHz CBW from optional to mandatory in Release 19

- Proposals

  - Proposal 1: Change the 3 MHz CBW for Band n28, n26, n85, n100 from optional to mandatory in Release 19. This CBW to be excluded from MSD interference impact analysis.

- Recommended WF

  - Check proposal 1 can be agreeable

Related CRs

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2522167 (R19 38.101-1) | Rakuten Mobile, Inc, T-Mobile,Anterix,Southern Linc |  |  |

CRs for TS 38.101-1

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2520219 | Skyworks Solutions, Inc., Murata Manufacturing Co, Qualcomm France, T-Mobile USA |  |  |

CRs for TS 38.101-2

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2521588 (R15) CAT-A: R4-2521287 R4-2521288 R4-2521289 R4-2521290 | ZTE Corporation,Sanechips |  |  |
| R4-2520354 (R19) | China Unicom, Ericsson, Qualcomm, Huawei, NTT Docomo |  |  |

CRs for TS 38.101-3

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2520226 (R19) | Murata Manufacturing Co Ltd., Skyworks Solutions, Inc., Qualcomm France |  |  |

CRs for TS 38.101-5

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2520568 | Apple |  |  |
| R4-2520570 | Apple |  |  |

CRs for TS 36.102

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2522072 | Sony |  |  |

CRs for TS 38.863

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2520569 | Apple, Qualcomm |  |  |