3GPP TSG-RAN4 Meeting #117	R4-2522412

# Dallas, United States, November 17th – 21st 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.307 | CR | 0199 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.307: Release independence of Rel-19 NR NTN features [NTN_Release_Independence] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For 3 MHz CBW, during the work item it was agreed that 3 MHz CBW can be release independent to rel-18 and related LS was sent to RAN2. However, release independence specification was not updated. Similarly for RedCap and high power UE, RAN4 agreed on the release independence but specification was not updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include release independence from rel-18 to 3 MHz CBW for NR NTN. Include release independence from rel-17 to NR NTN HPUE. Include release independence for FD-FDD RedCap from rel-17, FD-FDD eRedCap from rel-18 and for HD-FDD RedCap from rel-19. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear which requirements rel-17 or rel-18 UE needs to meet when it supports early implementation of rel-19 features, or whether it is possible to support rel-19 features at all. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1, 9.2, F.1.1, F.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## 9.1	Additional NR operating bands for NR NTN in frequency range 1

Requirements for NR UE supporting satellite access operation, for additional NTN operating bands of TS 38.101-5 in Rel-P [8] are introduced via this clause.

Table 9.1-1: Additional NR operating bands for NR NTN in FR1-NTN

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| NTN operating bands | FDD | Rel-17 | Table F.1.1-1, Table F.1.2-1, Table F.1.3-1 |

Table 9.1-2: NR NTN UE power class in FR1-NTN

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the power class was introduced) |
| --- | --- | --- | --- |
| Power Class 3, Power Class 2, Power Class 1 | FDD | Rel-17 | Table F.1.1-1 |

Table 9.1-3: NR NTN UE channel bandwidth in FR1-NTN

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the UE channel bandwidth was introduced) |
| --- | --- | --- | --- |
| UE channel bandwidth, all unless otherwise stated | FDD | Rel-17 | Table F.1.1-1 |
| Channel bandwidth of 3 MHz (Note) | FDD | Rel-18 | Table F.1.1-1 |
| NOTE: When UE supports both 3 MHz channel bandwidth and higher than default power class, the UE shall apply all requirements for power class 3 for 3 MHz channel bandwidth as defined in TS 38.101-5 [8]. |  |  |  |

## 9.2	Other release independent features for NR NTN in frequency range 1

This clause covers requirements for a Rel-P UE coming from all other release independent features that are not covered under clause 9.1.

Table 9.2-1: Additional requirements of other release independent features

| Feature | Release independent from | Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| DMRS bundling for GSO scenario | Rel-17 | Table F.1-4 | Rel-18 WI NR_NTN_enh introduced phase continuity requirements for DMRS bundling for the GSO scenario: see Table C.1-1 |
| FD-FDD RedCap | Rel-17 | Table F.1.1-2, Table F.1.2-2 | Rel-19 WI NR_IoT_NTN_req_test_enh introduced requirements for (e)RedCap for NR NTN |
| FD-FDD eRedCap | Rel-18 | Table F.1.1-2, Table F.1.2-2 |  |
| HD-FDD (e)RedCap | Rel-19 | Table F.1.1-2, Table F.1.2-2 |  |

Annex A :
Frequency arrangement for overlapping operating bands

<End of Change 1>

<Start of Change 2>

## F.1.1	Common UE RF requiremets for a release independent NR NTN band in FR1-NTN

The requirements and test cases listed in Table F.1.1-1 and F.1.1-2 are specified in Rel-P version of TS 38.101-5 [8].

Table F.1.1-1: UE RF requirements for a release independent NTN band in FR1-NTN

| Clause | Description |
| --- | --- |
| 5.2 | Operating bands |
| 5.3 | UE Channel bandwidth |
| 5.4 | Channel arrangement |
| 6.2 | Transmitter power |
| 6.3 | Output power dynamics |
| 6.4 | Transmit signal quality |
| 6.5 | Output RF spectrum emissions |
| 7.3 | Reference sensitivity |
| 7.4 | Maximum input level |
| 7.5 | Adjacent Channel Selectivity |
| 7.6 | Blocking characteristics |
| 7.7 | Spurious response |
| 7.8 | Intermodulation characteristics |
| 7.9 | Spurious emissions |

Table F.1.1-2: UE RF requirements for (e)RedCap band in FR1-NTN

| Clause | Description |
| --- | --- |
| 5.2 | Operating bands |
| 5.3I | Channel bandwidth for (e)RedCap |
| 5.4 | Channel arrangement |
| 6.2 | Transmitter power |
| 6.2I | Transmitter power for (e)RedCap |
| 6.3 | Output power dynamics |
| 6.4 | Transmit signal quality |
| 6.5 | Output RF spectrum emissions |
| 7.1I | General |
| 7.3I | Reference sensitivity for (e)RedCap |
| 7.4 | Maximum input level |
| 7.5 | Adjacent Channel Selectivity |
| 7.6 | Blocking characteristics |
| 7.7 | Spurious response |
| 7.8 | Intermodulation characteristics |
| 7.9 | Spurious emissions |

## F.1.2	Common RRM requirements for NR NTN in FR1-NTN

The requirements and test cases listed in Table F.1.2-1 and F.1.2-2 are specified in Rel-P version of TS 38.133 [6].

Table F.1.2-1: RRM requirements for NR NTN in FR1-NTN

| Clause | Description |
| --- | --- |
| 4.2C | Cell Re-selection for NR UE for Satellite Access |
| 4.3C | Minimization of Drive Tests (MDT) for Satellite Access |
| 5.1C | Cell Re-selection |
| 5.3C | Minimization of Drive Tests (MDT) for Satellite Access |
| 6.1C | Handover for SAN |
| 6.2C | RRC Connection Mobility Control for Satellite Access |
| 7.1C | UE transmit timing for Satellite Access |
| 7.2C | UE timer accuracy for satellite access |
| 7.3C | Timing advance for satellite access |
| 8.1C | Radio Link Monitoring for Satellite Access |
| 8.5C | Link Recovery Procedures for Satellite Access |
| 8.6C | Active BWP switch delay for satellite access |
| 8.10C | Active TCI state switching delay for satellite access |
| 8.14C | Pathloss reference signal switching delay for satellite access |
| 9.1C | General measurement requirement for SAN |
| 9.2C | NR intra-frequency measurements for SAN |
| 9.3C | NR inter-frequency measurements for SAN |
| 9.5C | L1-RSRP measurements for Reporting for satellite access |
| 10.1.2C | Intra-frequency RSRP accuracy requirements for FR1 SAN |
| 10.1.4C | Inter-frequency RSRP accuracy requirements for FR1 SAN |
| 10.1.7C | Intra-frequency RSRQ accuracy requirements for FR1 SAN |
| 10.1.9C | Inter-frequency RSRQ accuracy requirements for FR1 SAN |
| 10.1.12C | Intra-frequency SINR accuracy requirements for FR1 SAN |
| 10.1.14C | Inter-frequency SINR accuracy requirements for FR1 SAN |
| 10.1.19C | L1-RSRP accuracy requirements for FR1 SAN |
| A.14 | NR standalone tests for Satellite access |

Table F.1.2-2: RRM requirements for NR NTN (e)RedCap in FR1-NTN

| Clause | Description |
| --- | --- |
| 4.2E | Cell Re-selection for RedCap UE with Satellite Access |
| 4.3D | Minimization of Drive Tests (MDT) for NR RedCap UE with Satellite Access |
| 5.1E | Cell Re-selection for RedCap UE with Satellite Access |
| 5.3D | Minimization of Drive Tests (MDT) for NR RedCap UE with Satellite Access |
| 6.1F | Handover for RedCap UE with satellite access |
| 6.2E | RRC Connection Mobility Control for RedCap UE with Satellite Access |
| 7.1E | UE transmit timing for RedCap with Satellite Access |
| 7.2E | UE timer accuracy for RedCap with Satellite Access |
| 7.3E | Timing advance for RedCap with Satellite Access |
| 8.1E | Radio Link Monitoring for RedCap UE with Satellite Access |
| 8.5E | Link Recovery Procedures for RedCap UE with Satellite Access |
| 8.6E | Active BWP switch delay for RedCap UE with satellite access |
| 8.10F | Active TCI state switching delay for RedCap UE with satellite access |
| 8.14E | Pathloss reference signal switching delay for RedCap UE with satellite access |
| 9.1E | General measurement requirement for RedCap with satellite access |
| 9.2E | NR intra-frequency measurements for RedCap with SAN |
| 9.3E | NR inter-frequency measurements for Redcap UEs with satellite access |
| 9.5E | L1-RSRP measurements for Reporting for RedCap UEs with satellite access |

## F.1.3	Common UE demodulation requirements for NR NTN in FR1-NTN

<End of Change 2>