3GPP TSG-RAN4 Meeting # 117	R4-2522705

# Dallas, USA, 17th Nov – 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Common Test Parameters for LP-WUS Demodulation in FR1 and FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Perf |  |  |  |  |  | Date: |  |  | 2025-11-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Adding tables with common test parameters for new LP-WUS feature |  |  |  |  |  |  |  |  |
| Summary of change: |  | New Section 5.X, new Section 7.X |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Common test parameters will not be included |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

## 5.X	LP-WUS demodulation requirements

The parameters specified in Table 5.X-1 are valid for all LP-WUS detection tests unless otherwise stated.

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Common serving cell parameters | Active DL BWP index |  | 1 |
|  | Physical Cell ID |  | 0 |
|  | SSB position in burst |  | First SSB in Slot #0 |
|  | SSB periodicity | ms | 20 |
| LP-SS Configuration | OOK Waveform |  | OOK-4 |
|  | LP-SS OOK Chip Rate (MLP-SS) |  | 4 |
|  | Binary Sequence Length |  | [32] |
|  | BW | subcarriers | 132 |
|  | Periodicity | ms | [160] |
|  | Offset | ms | 0 |
| LP-WUS Configuration | OOK Waveform |  | [OOK-4] |
|  | Number of OOK symbols in an OFDM symbol (MWUS) |  | 4 |
|  | Number of overlaid sequences (Nseq) |  | 4 |
|  | Number of root sequence numbers (Nroot) |  | [1] |
|  | Root sequence number (q) |  | 1 |
|  | Periodicity | ms | 320 |
|  | EPRE ratio between WUS and SSB | dB | 0 |

==============End of change==============

==============Second change==============

## 7.X	LP-WUS demodulation requirements

The parameters specified in Table 7.X-1 are valid for all LP-WUS detection tests unless otherwise stated.

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Common serving cell parameters | Active DL BWP index |  | 1 |
|  | Physical Cell ID |  | 0 |
|  | SSB position in burst |  | First SSB in Slot #0 |
|  | SSB periodicity | ms | 20 |
| LP-SS Configuration | OOK Waveform |  | OOK-1 |
|  | LP-SS OOK Chip Rate (MLP-SS) |  | 1 |
|  | Binary Sequence Length |  | 8 |
|  | BW | subcarriers | 132 |
|  | Periodicity | ms | [160] |
|  | Offset | ms | 0 |
| LP-WUS Configuration | OOK Waveform |  | [OOK-4] |
|  | Number of OOK symbols in an OFDM symbol (MWUS) |  | 1 |
|  | Number of overlaid sequences (Nseq) |  | 2 |
|  | Number of root sequence numbers (Nroot) |  | 1 |
|  | Root sequence number (q) |  | 1 |
|  | Periodicity | ms | 320 |
|  | EPRE ratio between WUS and SSB | dB | 0 |

==============End of change==============