3GPP TSG-WG4 Meeting #117	R4-252xxxx

# Dallas, USA,  November 17, 2025 - November 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101-4 | CR | draftCR | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR: RMC for WUS demodulation performance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Reference measurement channels (RMC) for LP-WUS demodulation performance requirements are not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the RMC used for LP-WUS demodulation performance requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear the LP-WUS configuration used for LP-WUS demodulation performance requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.5 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2521230 |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change 1 ------------------------------------------------------------

## A.3.5	Reference measurement channels for wake-up signal demodulation requirements

### A.3.5.1	Reference measurement channels for FR1

Table A.3.4.1-1: Wake-up Signal Reference Channel for FDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.WUS.1-1.1 | R.WUS.1-1.2 |
| Subcarrier spacing | kHz | 15 | 15 |
| Number of bits after Rate matching (E0) | Bits | 12 | 32 |
| Number of bits after Line coding (G0) | Bits | 24 | 64 |
| Wake-up information length (K) | Bits | 2 | 5 |

Table A.3.4.1-2: Wake-up Signal Reference Channel for TDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.WUS.1-2.1 | R.WUS.1-2.2 |
| Subcarrier spacing | kHz | 30 | 30 |
| Number of bits after Rate matching (E0) | Bits | 12 | 32 |
| Number of bits after Line coding (G0) | Bits | 24 | 64 |
| Wake-up information length (K) | Bits | 2 | 5 |

### A.3.5.2	Reference measurement channels for FR2

Table A.3.4.2-1: Wake-up Signal Reference Channel

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.WUS.2-1.1 | R.WUS.2-1.2 |
| Subcarrier spacing | kHz | 120 | 120 |
| Number of bits after Rate matching (E0) | Bits | 12 | 32 |
| Number of bits after Line coding (G0) | Bits | 24 | 64 |
| Wake-up information length (K) | Bits | 2 | 5 |

------------------------------------------------------------- End of change 1 ------------------------------------------------------------