3GPP TSG-RAN WG4 Meeting #117		   				 		  								R4-252xxxx

Dallas, USA, 17th – 21th Nov., 2025

Source:	Moderator (MediaTek inc.)

Title:	Way Forward for [117][310] Rel-19 RRM performance_Part2

Document for:	Approval

# Introduction

This document is to capture all agreements for thread [117][310] NTN RRM performance part 2, including the following topics:

- Topic#1: RRM performance requirements for NR NTN Ku band (AI 5.10.2)

- Topic#2: NTN testing for NGSO (AI 6.6.3.2)

- Topic#3: RRM performance requirements for IoT NTN phase 3 (AI 6.20.3)

- Topic#4: RRM performance requirements for IoT-NTN TDD mode (AI 6.21.2)

# Topic# 1: NR NTN Ku band (AI 5.10.2)

#### Issue 1-1-1: Measurement Accuracy

It was discussed online on Monday.

Agreement

- Not add the new band group for FR1-NTN Ku band accuracy requirement.

- Extend the current FR2-NTN measurement accuracy requirement to cover FR1-NTN Ku band VSAT.

#### Issue 1-2-1: Test cases for Ku bands in FR1-NTN

It was discussed online on Monday.

Agreement

- Reuse the existing test cases defined for FR2-NTN for Ku band with FR1-NTN numerology with the necessary modifications

  - Introduce a new configuration that corresponds to the reference channels used for FR1-NTN numerology.

- Specify the applicability of test cases for FR1-NTN in the specification.

#### Issue 1-2-2: Reference Channel for Ku band with FR1-NTN numerology

R4-2521737	Nokia

- Proposal 2: For FR1-NTN numerology, reuse the reference channels used for 15 kHz SCS and 10 MHz of bandwidth existent for FR1.

Agreement

- Use 30 kHz SCS reference channels for testing for Ku band with FR1-NTN numerology.

#### Issue 1-2-3: Test principle for VSAT supporting both FR1 and FR2 numerology

It was discussed online on Monday.

Agreement

- the UE shall pass two test configurations: one with FR1 numerology and one with FR2 numerology.

#### Issue 1-2-4: UE transmit timing test for NGSO VSAT

No agreement

# Topic# 2: NTN testing for NGSO (AI 6.6.3.2)

#### Issue 2-1-1: R19 UL timing tests and test requirement impact

It was discussed online on Monday.

Agreement

- The TE-emulated channel model with varying Doppler and delay shifts is applied to R19 UL transmit timing test cases

- UL timing advance adjustment accuracy test can be FFS

# Topic# 3: Testing for CB-Msg3-EDT (AI 6.20.3)

No open issue

# Topic# 4: IoT-NTN TDD mode (AI 6.21.2)

#### Issue 4-1-1: Measurement accuracy

It was discussed online on Monday.

WayForward

- Discuss option 1 and 2 for Minimum NRSRP on the following table.

Table B.3.25A-1: NB-IoT intra-frequency absolute NRSRP and NRSRQ Accuracy Requirements

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum NRSRP |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_SAB _G | -139.8 |
|  | NTDD_SAB_O | Option 1: -133 Option 2: [-124 … -126] |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |

#### Issue 4-2-1: Principle of testing TDD and FDD frame structure

It was discussed online on Monday.

Agreement

- UE supporting both FDD and TDD should pass the test cases with each of the FDD and TDD configurations as default.

#### Issue 4-2-2: Test case list Test parameters - TSI_NB-IoT  and TPRACH_NB-IoT

It was discussed online on Monday.

WayForward

- FFS TSI_NB-IoT  and TPRACH_NB-IoT  for TDD.

#### Issue 4-2-3: Test configuration for random access test

It was discussed online on Monday.

- For IoT NTN TDD Reference NPRACH Configuration, keep coverage level 2.

- Please check the proposed values directly in the corresponding CR.

#### Issue 4-2-4: Test configuration for timing advance accuracy Test

It was discussed online on Monday.

- The following test parameters are adopted.

| Number of repetitions | NPDCCH | 4 |
| --- | --- | --- |
|  | NPDSCH | 8 |
|  | NPUSCH | 8 |

- Note that the timing advance accuracy result should not be averaged across TDD cycles.

#### Issue 4-2-5: Test configuration for RLM test

It was discussed online on Monday.

- The Qin = 1 dB and Qout = -5 dB can be used as a reference for specifying SNR values in RLM test cases.

- Please check the proposed values directly in the corresponding CR.