3GPP TSG-RAN WG4 Meeting #117	R4-2521474

# Dallas, USA, 17th - 21th, Nov, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.181 | CR |  | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on NPUSCH requirement for IoT NTN phase 3 in TS 36.181 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-19 IoT NTN ehnacement WI, RAN4 has introduced NPUSCH requirement with OCC feature for both 15KHz and 3.75KHz SCS with single tone scenario, new FRC was introduced for 15KHz SC |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add requirements for NPUSCH with 15KHz and 3.75KHz SCS for OCC feature in section 8.5 and 11.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NPUSCH requirement with 15KHz SCS for OCC feature can not be verfied |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5, 11.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2521474 |  |  |  |  |  |  |  |  |

==============First change==============

## 8.5	Performance requirements for Narrowband IoT

### 8.5.1	Performance requirements for NPUSCH format 1

#### 8.5.1.1	Definition and applicability

The performance requirement of NPUSCH format 1 is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

The tests for 3.75 kHz subcarrier spacing are applicable to the base stations supporting 3.75 kHz subcarrier spacing requirements. The tests for single-subcarrier/multi-subcarrier of 15 kHz subcarrier spacing are applicable to the base stations supporting the number of subcarriers of 15 kHz subcarrier spacing requirements.

The test for 15kHz subcarrier spacing with slot-level length-2 OCC is applicable to the base stations supporting 15kHz subcarrier spacing with slot-level length-2 OCC requirements. The test for 3.75kHz subcarrier spacing with symbol-level length-2 OCC is applicable to the base stations supporting 3.75kHz subcarrier spacing with ymbol-level length-2 OCC

#### 8.5.1.2	Minimum Requirement

The minimum requirement is in TS 36.108 [2] clause 8.5.1.

#### 8.5.1.3	Test Purpose

The test shall verify the receiver’s ability to achieve the throughput under multipath fading propagation conditions for a given SNR.

#### 8.5.1.4	Method of test

##### 8.5.1.4.1	Initial Conditions

Test environment: Normal, see annex B.

RF channels to be tested: M; see clause 4.9.1.

1)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to all SAN TAB connectors for diversity reception via a combining network as shown in annex D.5.

##### 8.5.1.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth, defined in table 8.5.1.4.2-1.

Table 8.5.1.4.2-1: AWGN power level at the SAN input

| Channel bandwidth (kHz) | AWGN power level |
| --- | --- |
| 200 | -100.5dBm /180kHz |

2)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameters in table 8.5.1.4.2-2 and table 8.5.1.4.2-3

Table 8.5.1.4.2-2: Test parameters for testing NPUSCH format 1

| Parameter | unit | Value |
| --- | --- | --- |
| Maximum number of HARQ transmissions |  | 4 |
| RV sequences |  | RV0, RV2 |
| Duration of NPUSCH format 1 segment transmission (Nsegmentprecompensation) | ms | 256 for 3.75kHz SCS, 16 for 15kHz SCS |
| Repetition |  | 4 for 3.75kHz SCS, 16 for 15kHz SCS |

Table 8.5.1.4.2-3: Test parameters for testing NPUSCH format 1 with OCC feature

| Parameter | unit | Value |
| --- | --- | --- |
| Maximum number of HARQ transmissions |  | 4 |
| RV sequences |  | RV0, RV2 |
| Duration of NPUSCH format 1 segment transmission (Nsegmentprecompensation) | ms | 256 for 3.75kHz SCS and 15kHz SCS |
| Repetition |  | 2 for 3.75kHz SCS, 8 for 15kHz SCS |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex F.

4)	Adjust the equipment so that required SNR specified in tables 8.5.1.5-1 to 8.5.1.5-4 is achieved at the SAN input.

5)	The signal generator sends a test pattern with the pattern outlined in Figure 8.5.1.4.2-1. The test signal generator sends frames, and the receiver tries to demodulate the content. The frames are sent with certain frequency and gradual time offset per segment as described below in table 8.5.1.4.2-3 and table 8.5.1.4.2-4. The time offset is reset back to zero between segments.

Figure 8.5.1.4.2-1: Test signal pattern for NPUSCH format 1 demodulation tests

Table 8.5.1.4.2-3: Frequency Offsets for testing NPUSCH format 1

| Cases | Accumulated Frequency Offset |
| --- | --- |
| Table 8.5.1.5-1 Table 8.5.1.5-3 Table 8.5.1.5-4 | 128 Hz |
| Table 8.5.1.5-2 | 8 Hz |

Table 8.5.1.4.2-4: Timing Offsets for testing NPUSCH format 1

| Cases | Step size Δt |
| --- | --- |
| Table 8.5.1.5-1 Table 8.5.1.5-3 | 0.32 us per RU |
| Table 8.5.1.5-2 | 0.01 us per RU |
| Table 8.5.1.5-4 | [0.08] us per RU |

6)	For each of the reference channels in table 8.5.1.5-1 to 8.5.1.5-2 applicable for the base station, measure the throughput. For each of the reference channels in table 8.5.1.5-3 to 8.5.1.5-4 applicable for the base station with supporting symbol-level length-2 OCC and slot-level length-2 OCC, repectively , measure the throughput for each UE.

#### 8.5.1.5	Test Requirement

The throughput measured according to clause 8.5.1.4.2 shall not be below the limits for the SNR levels specified in table 8.5.1.5-1 for 3.75 kHz subcarrier spacing tests and not be below the limits for the SNR levels specified in table 8.5.1.5-2 for 15kHz subcarrier spacing with the supported number of subcarrier tests.

The throughput measured for each UE according to clause 8.5.1.4.2 shall not below the limitdn for the SNR levels specified in table 8.5.1.5-3 for 3.75 kHz subcarrier spacing with suppoting symbol-level length-2 OCCtests and not be below the limits for the SNR levels specified in table 8.5.1.5-4 for 15kKHz subcarrier spacing with supporting slot-level length-2 OCCtests.

Table 8.5.1.5-1 Required SNR for NPUSCH format 1 test, 200 kHz channel bandwidth, 3.75 kHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (annex F) | FRC (annex A) | Repetition number | Fraction of maximum throughput | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75 kHz | 1 | NTN-TDLA100-1 | A16-1 | 4 | 70% | -1.4 |
| 1 | 1 | 3.75 kHz | 1 | NTN-TDLC5-1 | A16-1 | 4 | 70% | -3.0 |
| 1 | 2 | 3.75 kHz | 1 | NTN-TDLA100-1 | A16-1 | 4 | 70% | -4.8 |
| 1 | 2 | 3.75 kHz | 1 | NTN-TDLC5-1 | A16-1 | 4 | 70% | -5.9 |

Table 8.5.1.5-2 Required SNR for NPUSCH format 1 test, 200 kHz channel bandwidth, 15 kHz SCS, multiple subcarriers, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (Annex F) | FRC (annex A) | Repetition number | Fraction of maximum throughput | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15kHz | 12 | NTN-TDLA100-1 | A16-2 | 16 | 70% | -2.6 |
| 1 | 1 | 15kHz | 12 | NTN-TDLC5-1 | A16-2 | 16 | 70% | -3.4 |
| 1 | 2 | 15kHz | 12 | NTN-TDLA100-1 | A16-2 | 16 | 70% | -6.5 |
| 1 | 2 | 15kHz | 12 | NTN-TDLC5-1 | A16-2 | 16 | 70% | -7.2 |

Table 8.5.1.5-3 Required SNR for NPUSCH format 1 test, 200 kHz channel bandwidth, 3.75 kHz SCS, symbol-level length-2 OCC, single subcarrier, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (annex F) | FRC (annex A) | Repetition number | Fraction of maximum throughput | Occ Index | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75 kHz | 1 | NTN-TDLA100-1 | A16-1 | 2 | 70% | 0 | TBD |
|  |  |  |  |  |  |  |  | 1 | TBD |
| 1 | 2 | 3.75 kHz | 1 | NTN-TDLA100-1 | A16-1 | 2 | 70% | 0 | TBD |
|  |  |  |  |  |  |  |  | 1 | TBD |
| Note : The total number of slots in NPUSCH transmission after transmission after OCC is applied Repetition number* OCC length *NslotsULNRU, where NslotsUL is number of UL slot and NRU is the number of RU. |  |  |  |  |  |  |  |  |  |

Table 8.5.1.5-4 Required SNR for NPUSCH format 1 test, 200 kHz channel bandwidth, 15 kHz SCS, slot-level length-2OCC, single subcarrier, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (Annex F) | FRC (annex A) | Repetition number | Fraction of maximum throughput | OCC sequence index | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15kHz | 1 | NTN-TDLA100-1 | A16-X | 8 | 70% | 0 | TBD |
|  |  |  |  |  |  |  |  | 1 | TBD |
| 1 | 2 | 15kHz | 1 | NTN-TDLA100-1 | A16-X | 8 | 70% | 0 | TBD |
|  |  |  |  |  |  |  |  | 1 | TBD |
| Note : The total number of slots in NPUSCH transmission after transmission after OCC is applied Repetition number* OCC length *NslotsULNRU, where NslotsUL is number of UL slot and NRU is the number of RU. |  |  |  |  |  |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

==============End of change==============