3GPP TSG-RAN WG4 Meeting #117	R4-2523086

Dallas, USA, November 17 ‒ November 21, 2025

Title: 	Way Forward for [117][328] Rel-19 Demod_Part3_NTN

Agenda Item:	6.23

Source: 	Ericsson

Document for:	Approval

# SAN demodulation for UE supporting less than 5MHz for NTN

## PUCCH format 2 demodulation requirements for NTN with 3MHz CBW

Agreement: Formal CR

Responsible companies to submit the formal CRs to TS 38.108 and TS 38.181 in RAN#118 to complete the WI.

# UE NTN testing for NGSO

## Test applicability of satellite-motion based varying Doppler and delay shift NTN channel model channel model

Agreement: Applicability of Rel-19 satellite-motion based varying Doppler and delay shift NTN channel model

- RAN4 focuses on the discussion of applicability of Rel-19 satellite-motion based varying Doppler and delay shift NTN channel model for FR1-NTN bands excluding VSAT UE.

## RRM tests

Agreement: Which NR NTN RRM tests apply Rel-19 satellite-motion based varying Doppler and delay shift NTN channel model

Within the scope of Rel-19, Rel-19 satellite-motion based varying Doppler and delay shift NTN channel model will only be applied to the following test cases in TS 38.133:

- RRC_IDLE state mobility:

  - A.14.1.1 Cell reselection to FR1 intra-frequency NR case

- CONNECTED state mobility:

  - A.14.2.1.1 Intra-frequency SAN Handover from FR1 to FR1

- Timing for Satellite Access:

  - A.14.3.1.1 NR UE Transmit Timing Test for FR1

- Measurement procedure:

  - A.14.5.3.1 SSB based L1-RSRP measurement for satellite access when DRX is not used

Way forward: Necessary updates in TS 38.133 to complete the WI.

For the test cases to which Rel-19 satellite-motion based varying Doppler and delay shift NTN channel model can be applied as listed above, the following updates are necessary. The changes will be made via CRs at RAN4#118.

- Option 1: Add an additional Config ID in the “supported test configurations” table in the respective test cases

  - The additional Config ID is for NGSO with Rel-19 satellite-motion based varying Doppler and delay shift NTN channel model.

  - All test parameters are inherited from the NGSO configuration defined in the existing corresponding test case.

- Option 2: Define separate test cases with Rel-19 satellite-motion based varying Doppler and delay shift NTN channel model

Agreement: Test applicability

For Rel-19 UE and onwards, FR1 NR NTN UE supporting NGSO, the following test applicability rules apply:

- The UE shall pass test “A.14.3.1.1 NR UE Transmit Timing Test for FR1” based on new Config ID defined for Rel-19 satellite-motion based varying Doppler and delay shift NTN channel model.

- For the rest test cases where Rel-19 satellite-motion based varying Doppler and delay shift NTN channel model can be applied, i.e. A.14.1.1, A.14.2.2.4 and A.14.5.3.1,

  - The UE shall pass the test cases based on either existing Config ID with the existing static NGSO channel, or the new Config ID defined for Rel-19 satellite-motion based varying Doppler and delay shift NTN channel model.

- For other test cases where Rel-19 satellite-motion based varying Doppler and delay shift NTN channel model cannot be applied, the UE shall pass the existing test cases.

- The above test applicability rules are to be implemented in the following clauses by CR:

  - A.3.36.2 Principle of testing GSO and NGSO scenarios

  - A.3.36.2 Principle of testing different RRM requirements

  - As a note in the “Supported test configurations” table in the respective test cases if appropriate.

## UE demodulation requirements

Agreement: NR NTN UE demodulation requirements

For Rel-19 UE,

- UE shall only pass TS 38.101-5 Table 8.2.1.2.2.1.1-4 Tests 2-1 and 2-2 with Rel-19 satellite-motion based on both time and Doppler-varying NTN channel model (draft CR R4-2513627 endorsed in RAN4#116bis)

  - UE can skip all the tests in TS 38.101-5 Table 8.2.1.2.2.1.1-3 if UE passes the test with Rel-19 satellite-motion based on both time and Doppler-varying NTN channel model.

Rel-19 satellite-motion based varying Doppler and delay shift NTN channel model channel model is not applied for pre-Rel-19 UEs.

Agreement: IoT-NTN UE demodulation requirements

The NGSO channel model is applied only to the following tests (draft CR R4-2513628 endorsed in RAN4#116bis):

- TS 36.102 Table 8.3.1.1.1.1-3 Test 1

- TS 36.102 Table 8.2.1.1.1.1-3 Test 1 and Test 2

Rel-19 satellite-motion based varying Doppler and delay shift NTN channel model channel model is not applied for pre-Rel-19 UEs.

## Satellite-motion based varying Doppler and delay shift NTN channel model design

Way forward: How to ensure the satellite elevation angles more than 30 degrees during the test

Background: RAN5 conformation test procedure specifies to run multiple subtest to measure the average throughput for a duration sufficient to achieve statistical significance (See TS 38.521-5 8.2.1.2.2.1.1_1.3.2).

- Interested companies are encouraged to study how TE ensures to apply the same Doppler shift and propagation delay model for each subtest. FFS how to capture in the specification.

  - Option 1: Use different cell ID for each sub-test so that UE identify the disappeared satellite is different from the new one. RRC is Release after each sub-test running, and UE re-enters RRC Connect mode for a different cell id before each sub-test.

  - Option 2: Add the following sentence in TS 38.101-5 A.4.1 and TS 36.102 A.3.1: “The ephemeris info shall be reset to the initial ephemeris info for each subtest.”

  - Other options are not precluded.

Way forward: How to derive Ephemeris information in format of orbital parameters in ECI.

Background: TS 38.331 states that Ephemeris information may be expressed in format of position and velocity state vector in ECEF or in format of orbital parameters in ECI. According to TS 38.133 Table A.3.36.3-2, RRM test cases assume ephemeris information is provided in format of orbital parameter in ECI. However, the current draft big CR (R4-2513627) assumes that the Ephemeris information is provided only in format of position and velocity state vector in ECEF.

- Revise the draft big CR to derive the Ephemeris information in format of orbital parameters in ECI also.