3GPP TSG-RAN WG4 Meeting #117	R4-251xxxx

Dallas, USA, 17th – 21st Oct, 2025

Title: 	           Way Forward for [117][329] NR_UE_RF_Ph5_Part2 -- 6 MHz channel

Agenda Item:	7.1.1

Source: 	Samsung

Document for:	Approval

1. UE RF

1.1 NS_06

Agreement:

- For PC3 NS_06, no A-MPR is applicable for 6MHz CBW.

- For PC2 NS_06, have the same A-MPR for 6 MHz CBW as for other CBWs as specified in Table 6.2.3.32-1 in TS 38.101-1.

- Above two agreements lead to no spec change.

1.2 NS_15

Agreement:

- Evaluation A-MPR for NS_15 and 6 MHz channel bandwidth assuming deployment within 814-845 MHz or within 841.5-849 MHz.

- Reuse as starting point the regions defined for 7 MHz channel bandwidth (Table 1 and Table 2), and further evaluate/align the A-MPR regions and values in next meeting.

Table 1: A-MPR regions for NS_15 and PC3

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 841.5 < Fc ≤ 845.5 | ≥ 4.5 | > 0 | A7 |
|  |  | ≥ 2.16 , < 4.5 | ≥ 1.8 | A2 |
|  |  | ≤ 1.44 | ≤ 0.36 | A8 |
|  | 838 < Fc ≤ 841.5 | > 0 | > 2.7 | A9 |
|  |  | ≥ 5.94 | ≤ 2.7 | A10 |

Table 2:  A-MPR regions for NS_15 and PC2

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 841.5 < Fc ≤ 845.5 | ≥ 4.5 | > 0 | A8 |
|  |  | ≥ 2.16, < 4.5 | ≥ 1.8 | A9 |
|  |  | ≥ 2.16, < 4.5 | ≥ 1.44, < 1.8 | A10 |
|  |  | < 1.44 | ≤ 0.36 | A11 |
|  | 837 < Fc ≤ 841.5 | ≥ 0 | > 2.7 | A12 |
|  |  | ≥ 5.94 | ≤ 2.7 | A13 |

2. BS RF

2.1 ACS

Agreement:

- Specify the ACS interferer frequency offset with ±2.5125 MHz value.

2.2 Narrowband blocking

Agreement:

- Specify the interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap with value ± (360+m*180) KHz, with m=0, 1, 2, 3, 4, 9, 14, 19, 24.

2.3 Intermodulation

Agreement:

- Specify the interfering signals for the intermodulation requirement according to the following table:

Table: Interfering signals for intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal (Note 3) |
| --- | --- | --- |
| 6 | ±7.475 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |

2.4 Narrow band Intermodulation

Agreement:

- Specify the narrowband intermodulation requirement according to the following table:

Table: Interfering signals for narrowband intermodulation requirement in FR1

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 3) | Type of interfering signal |
| --- | --- | --- |
| 6 | ±425 | CW |
|  | ±1240 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |

2.5 In-Chanel selectivity

Agreement:

- Specify the In-channel selectivity requirements according to the following table:

Table: Wide Area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 6 | 15 | G-FR1-A1-7 | -100.6 | Option1: -79.6(Nokia, ZTE, Ericsson); Option 2:-80.6(CATT, Huawei); | DFT-s-OFDM NR signal, 15 kHz SCS, Option1: 15 RBs(Nokia, ZTE, Ericsson); Option 2: 12RBs (CATT, Huawei) |

2.6 Plan for BS dCRs for RAN4#118

For information:

- Target for RAN4#118 is to endorse all BS dCRs.