3GPP TSG-RAN WG4 Meeting #117                                                   R4-2522974

Dallas, US, Nov 17 – 21, 2025

Title: 	Way Forward for [117][329] NR_UE_RF_Ph5_Part2: HPUE coexistence

Agenda Item:	7.1.1

Source: 	Huawei, HiSilicon, MediaTek Inc.

Document for:	Approval

0 Introduction

This WF captures agreed updates on simulation assumptions for adjacent channel co-existence study for 4Tx HPUE with up to 32dBm max output power.

1 Simulation scenarios

Table 1 lists the updated co-existence study scenarios for FWA HPUE with 4Tx.

Table 1: Co-existence study scenarios

| Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Note |
| --- | --- | --- | --- | --- | --- | --- |
| eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 2.6GHz, 4GHz, 7GHz | Urban macro | 1st priority |
|  |  |  |  | 2.6GHz | Rural macro | 2nd priority |
|  |  |  |  | 7GHz | Suburban macro | 2nd priority |

2.6GHz stands for NR band n41, 4GHz stands for bands n77/n78 and n79, while 7GHz stands for band n104.

FFS on how to harmonize ACLR numbers from the coexistence study from different bands when defining final ACLR requirement for PC1.

2 Network layout model

Table 2-1 lists the updated network layout model for urban macro.

Table 2-1: Network layout model for urban macro

| Parameters | 2.6GHz | 4GHz | 7GHz |
| --- | --- | --- | --- |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |  |
| Inter-site distance | 750m | 450m/500m | 450m/500m |
| BS antenna height | 45m/25m | 20m/25m | 20m/25m |
| Channel model | UMa in TR 38.901 (Note 2) |  |  |
| NOTE 1: Companies should claim the adopted inter-site distance and BS antenna height in the simulations. The values for 4GHz and 7GHz should be the same for the claimed parameter. NOTE 2: Channel model for 2.6GHz should be aligned with assumptions adopted in TR 37.829. |  |  |  |

Table 2-2 lists the network layout model for suburban macro.

Table 2-2: Network layout model for 7GHz suburban macro

| Parameters |  | Value |
| --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance |  | 0.9 km |
| Multi operators layout |  | Uncoordinated |
| BS antenna height |  | [25] m |
| UE location | Outdoor/indoor | Outdoor and indoor |
|  | Indoor UE ratio | 2% in commercial building, 18% in residential building (Table 7.4.2-1 in TR 38.901) |
|  | Low/high penetration loss ratio | 50% low loss and low loss A, 50% high loss (Table 7.4.3-2 in TR 38.901) |
|  | LOS/NLOS | LOS and NLOS |
|  | UE antenna height | 1.5 m for outdoor, 1.5 or 4.5 m for residential buildings, 1.5/4.5/7.5/10.5/13.5 m for commercial buildings (Table 7.2-5 in TR 38.921) |
| UE distribution (horizontal) |  | Uniform |
| Minimum BS - UE distance (2D) |  | 35 m For Uncoordinated deployment, minimum distance applies also to UEs and BS across different deployments. |
| Channel model |  | SMa (Table 7.4.1-1 in TR 38.901) |

Table 2-3 lists the updated network layout model for rural macro.

Table 2-3: Network layout model for 2.6GHz rural macro

| Parameters | Value |
| --- | --- |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance | 6000m |
| FWA/UE Outdoor/ indoor ratio | 100% outdoor |
| BS antenna height | 45m |
| Channel model | RMa in TR 37.829 |

3 Propagation model

Relevant channel models are given in Tables 2-1, 2-2, and 2-3.

4 Antenna characteristics

No update for BS antenna characteristics.

For UE, baseline is isotropic with 0dBi (TR 38.921). The directional antenna pattern based on TR 38.901 can be considered for 7GHz optionally.

5	Uplink transmission power control model

Power control model is updated for the parameters.

where,

-	Pmax = 32dBm

-	CLx-ile = Pmax - Targeted UL SNR - BS Noise Floor

= Pmax - Targeted UL SNR - (-174 + NFBS + 10log10(BWUL) + 60)

Targeted UL SNR = 15dB

NFBS = 5dB for frequency below 6GHz, 6dB for 7GHz.

BWUL depends on the scheduled UE numbers.

-	γ = 1

-  Rmin=10^(( Pmin - Pmax)/10)

Pmin = -33dBm

Companies are encouraged to share the UL transmit power and SINR CDF in their simulations for the different deployment scenarios.

6 Other simulation parameters

No update.

7 Performance metric

Option 1 is adopted as baseline.

If option 2 is considered in the simulation, the reference victim system performance degradation in step 1 is also based on 5% throughput loss criterion.

- Option 1: Average throughput loss and 5%-ile throughput loss should be less than 5%.

- Option 2: The following procedure shall be performed to obtain the performance metric:

1)	Run the UL to UL coexistence study, assuming UE max TX power of both systems are 23 or 26 dBm. This corresponds to the coexistence of two commercial networks operating in adjacent channel and with similar deployment parameters. This is used as the reference. The victim system performance degradation results in this scenario are used as the baseline.

2)	Run the UL to UL coexistence study, assuming 32 dBm power class UE is deployed in interfering system only and obtain the victim system performance degradation results.

3)	Compare the victim system performance degradation obtaining in steps 1) and 2), choose the 32 dBm UE ACLR value so that the victim system performance degradation due to 32 dBm UE in 2) is similar to 1).

8 Other issue

Update the WID to add an internal TR for recording the study outcomes for the 4Tx HPUE co-existence objective.

Reference

- R4-2520089, Discussion on R20 HPUE coexistence study, CATT

- R4-2520279, Simulation results for HPUE coexistence study, Nokia

- R4-2520280, Proposals on simulation assumptions for HPUE coexistence study, Nokia

- R4-2522972, Discussion on co-existence for HPUE	ZTE Corporation, Sanechips

- R4-2521261, Co-existence study for HPUE with maximum output power up to 32dBm, Huawei, HiSilicon

- R4-2521896, Views on the coexistence study for High Power UE, Qualcomm Germany

- R4-2522171, On HPUE 4Tx PC1 initial coexistence study results, Ericsson

- R4-2522323, Initial coexistence evaluation results for TN FWA 4x26dBm device in Rel-20, MediaTek

- TR 38.921, Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz (Release 18)

- TR 38.922, Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz (Release 19)

- ITU WP 5D/792. Working document on characteristics of terrestrial component of IMT sharing and compatibility studies in preparation for WRC-27(Annex 4.32 to Working Party 5D Chair’s Report)

Annex: Co-existence simulation assumptions

A.1 Co-existence simulation scenarios

To be updated.

A.2 Network layout model

Table 2-1 lists the network layout model for urban macro, some parameters such as inter-site distance and BS antenna height are to be updated.

Table 2-1: Network layout model for urban macro

| Parameters |  | Value |
| --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance |  | Depending on frequency (ref. TR 37.829 for 2.6GHz, 3.5GHz and [TR 38.921, 600m] for other frequencies) |
| Multi operators layout |  | Uncoordinated |
| BS antenna height |  | Depending on frequency (ref. TR 37.829 for 2.6GHz, 3.5GHz and TR 38.922 for other frequencies) |
| UE location | Outdoor/indoor | Outdoor and indoor |
|  | Indoor UE ratio | 20% (TR 38.921) |
|  | Low/high penetration loss ratio | 50% low loss, 50% high loss |
|  | LOS/NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 (TR 38.921) |
| UE distribution (horizontal) |  | Uniform |
| Minimum BS - UE distance (2D) |  | 35 m For Uncoordinated deployment, minimum distance applies also to UEs and BS across different deployments. |
| Channel model |  | UMa |

Network layout models for suburban macro and rural macro are to be added.

A.3 Propagation model

Table 3 lists the propagation model.

Table 3: Propagation model

| Propagation loss | Value |
| --- | --- |
| Path loss | TR 38.901 |
| LOS probability | TR 38.901 |
| Penetration loss | TR 38.901 O2I building penetration loss |
| Shadowing correlation | Between cells: 1.0 Between sites: 0.5 |

A.4 Antenna characteristics

Table 4-1 lists the antenna radiation patterns and array antenna parameters for BS.

Table 4-1: BS antenna characteristics of urban macro

| Parameter | Symbol | 2.6GHz, 3.5GHz, 4.9GHz | 7GHz |
| --- | --- | --- | --- |
| Antenna pattern |  | TR 38.922 | TR 38.922 |
| Front to back ratio | Am | 30 dB | 30 dB |
| Side lobe suppression | SLAv | 30 dB | 30 dB |
| Horizontal HPBW | θ3dB | 90 deg. | 90 deg. |
| Vertical HPBW | θ3dB | 65 deg. | 65 deg. |
| Element peak gain | GE,max | 6.4 dBi | 6.4 dBi |
| Number of element rows in sub-array | Msub | 3 | 3 |
| Vertical element separation in sub-array | dv,sub | 0.7λm | 0.7λm |
| Pre-set sub-array down-tilt | θsubtilt | 3 deg. | 3 deg. |
| Number of radiating elements rows | M | 4 | 8 |
| Number of radiating elements columns | N | 8 | 16 |
| Horizontal element separation | dh | 0.5λm | 0.5λm |
| Vertical element separation | dv | 2.1λm | 2.1λm |
| Electrical down-tilt angle | θetilt | 90-100 deg. | 90-100 deg. |
| Electrical scan angle | φescan | ±60 deg. | ±60 deg. |
| Mechanical downtilt | θmech | 6 deg. | 6 deg. |

Table 4-2 lists the antenna pattern for UE, which is to be updated.

Table 4-2: UE antenna characteristics

| Parameter | Value |
| --- | --- |
| Antenna pattern | isotropic with 0dBi (TR 38.921) |

A.5	Uplink transmission power control model

Power control model specified in Section 9.1 TR 36.942 as follows is adopted.

where,

-	Pmax = 32dBm

-	CLx-ile = [88 + 10*log10(200/X) + 11 – Y + Pmax – 23, where X is UL transmission BW (MHz) and Y is the BS noise figure]

-	γ = 1

Specific parameters are to be updated.

A.6 Other simulation parameters

Table 6 lists other simulation parameters.

Table 6: Other simulation parameters

| Parameters | Value |
| --- | --- |
| Frequency reuse | 1x3x1 |
| The number of active UE (UL) | 1 or 3 |
| Scheduled channel bandwidth per UE (UL) | 98.28 MHz (1 user) 32.76 MHz (3 user) |
| Link Level Performance Model | TR 38.921 |
| Received power model | TR 38.921 |
| Traffic model | Full buffer |
| UE max TX power in dBm | 23 (as reference), 32 |
| UE min TX power in dBm | -33 |
| BS Noise figure in dB | 6 (@7 GHz) 5 (@below 6 GHz) |
| UE Noise figure in dB | 9 or 12 (Note 1) |
| ACLR of UE | 30 dB +X |
| ACS of BS | 45 dB (@below 6 GHz) 42 dB (@7 GHz) |
| Handover margin | 3 dB |
| Note 1: NF of 12 dB is for band n104. For bands below 6 GHz, NF is 9dB. |  |

A.7 Performance metric

To be updated.