3GPP TSG-RAN WG4 Meeting #117													R4- 2522206

Dallas, TX, USA, November 17-21, 2025

Agenda item:			10.1.3

Source:	Moderator (Ericsson)

Title:	Topic summary for [117][333] BSRF_Maintenance_Legacy

Document for:	Information

# Introduction

The scope of this topic summary is the BS RF maintenance legacy agenda items. Topic according to the agenda:

Up to Rel-18 maintenance for LTE and NR and TEI:

- BS RF requirements and BS conformance testing (other than NTN)	(10.3)

# Topic #1: Up to Rel-18 maintenance for LTE and NR and TEI; BS RF requirements and BS conformance testing (other than NTN) (10.3)

## Companies’ contributions summary

Discussion and approval papers

| T-doc number | Company | Title/Proposals |
| --- | --- | --- |
| R4-2520072 | CATT | Discussion on reference sensitivity levels requirement apply to BS that supports NB-IoT operation in NR in-band for 3 MHz channel bandwidth Proposal 1: Add Note 2 for the reference sensitivity power level for G-FR1-A1-21 (Note 6) for 3MHz CBW in Tables 7.2.2-1, 7.2.2-2 and 7.2.2-3 of TS 38.104. |
| R4-2521672 | Ericsson | Removal of UTRA TDD from MSR BS specifications Proposal 1: All UTRA TDD requirements are removed from the MSR BS specifications, including UTRA TDD bands for BC3, applicability of requirement for UTRA TDD operation, specific UTRA TDD RF requirements and additional UTRA TDD requirements for Tx IM, dynamic range, blocking and Rx IM. In addition, for the MSR BS test specification, UTRA TDD specific Test Configurations, Capability Sets for BC3 that include UTRA TDD capability and UTR TDD test signals are removed. Proposal 2: Protection of UTRA TDD are removed from BS spurious emission limits for co-existence and co-location, and from BS ACLR in unpaired spectrum with synchronized operation. This concerns all UTRA TDD bandwidths (1.28 Mcps, 3.84 Mcps and 7.68 Mcps). |
| R4-2522080 | Ericsson | Discussion on the usage of Radio Distribution Network (RDN) term for OTA BS. Proposal 1: RAN4 to acknowledge that current definition of the RDN as being linear and passive is opening for an unintended restriction on OTA AAS BS structure. Proposal 2: Change the current definition of RDN. Proposal 3: Implement the corrections from Proposal 2 starting with Rel-15. |

Submitted CRs (Cat A CRs not listed)

| T-doc number | Company | Title / Summary of change |
| --- | --- | --- |
| R4-2520073 | CATT | (NR_FR1_lessthan_5MHz_BW-Core)CR for TS38.104, Correction on reference sensitivity levels requirement apply to BS that supports NB-IoT operation in NR in-band for 3 MHz channel bandwidth. Summary of change: Add Note 2 for the reference sensitivity power level for G-FR1-A1-21 (Note 6) for 3MHz CBW in Tables 7.2.2-1, 7.2.2-2 and 7.2.2-3 of TS 38.104. |
| R4-2520211 | Keysight Technologies UK Ltd | (NR_ext_to_71GHz-Perf) Correction to Table G.2.1.2-4 Summary of change: Table values are modified to achieve the desired delay spread and made coincident with TS 38.101-4 table B.2.1.2-6. |
| R4-2520212 | Keysight Technologies UK Ltd | (NR_ext_to_71GHz-Perf) Correction to Table J.2.1.2-4 Summary of change: Table values are modified to achieve the desired delay spread and made coincident with TS 38.101-4 table B.2.1.2-6 |
| R4-2520213 | Keysight Technologies UK Ltd | (NR_ext_to_71GHz-Perf, TEI18) Addition of delay profiles for channel models TDLA10 and TDLD10 Summary of change: Adding delay profiles for channel models TDLA10 and TDLD10. |
| R4-2520415 Revised in R4-2522296 | Huawei, HiSilicon | (NB_IOT-Core) CR for TS36104 to correct NB-IoT Medium Range BS OBUE requirements for R15 Cat F |
| R4-2521199 Revised in R4-2522310 | Nokia | (TEI17) Correction of Operating band unwanted emissions for multi-band connector [TO_BE_ADDED] |
| R4-2521456 | Nokia, Ericsson | (NR_IAB-Perf) CR to TS 38.176-2 with clarification for channel bandwidths below 10 MHz Summary of change: Addition of clarification that CBWs below 10 MHz are not supported – the same as already exist in IAB core 38.174 and test 38.176-1 specifications. |
| R4-2521459 | Nokia | (NR_repeaters-Perf) CR to TS 38.115-1 with updates to Repeater ACRR test Summary of change: - Removal of note [RF channels to be update] – missleading text, RF channels are present. - Addition of downlink and uplink test models for ACRR test procedure. |
| R4-2521462 | Nokia | (NR_netcon_repeater-Perf ) CR to TS 38.115-2 bracket removal for test requirement for NCR-MT Summary of change: Removal of brackets in table 6.2.2.6.1.2-1. |
| R4-2521673 Revised in R4-2522313 | Ericsson | (RInImp9-Rfmulti,TEI) CR to 37.104: Removal of UTRA TDD from MSR BS specifications |
| R4-2521676 Revised in R4-2522314 | Ericsson | (RInImp9-Rfmulti,TEI) CR to 37.141: Removal of UTRA TDD from MSR BS specifications |
| R4-2521704 | Huawei, HiSilicon | (AASenh_BS_LTE_UTRA-Perf) CR to 37.145-1: test procedure for additional transmitter intermodulation for MSR operation Summary of change: The interfering signal generation for additional co-locattion (BC1, BC2 and BC3) are added. |
| R4-2521709 | Huawei, HiSilicon | (AASenh_BS_LTE_UTRA-Perf) CR to 37.145-2: test procedure for additional transmitter intermodulation for MSR operation Summary of change: 1. The interfering signal generation for additional co-locattion (BC1, BC2 and BC3) are added. 2. The frequency offsets are added for test requirements. |
| R4-2522109 | Nokia | (LTE_V2X) CR to Rel-14 36.104 on removal of Band 47 Summary of change: Band 47 is removed from relevant Clauses. |
| R4-2522114 | Nokia | (LTE_V2X) CR to Rel-19 36.104 on removal of Band 47 Summary of change: Band 47 is removed from relevant Clauses. |
| R4-2522115 | Nokia | (LTE_V2X) CR to Rel-14 36.141 on removal of Band 47 Summary of change: Band 47 is removed from relevant Clauses. |
| R4-2522120 | Nokia | (LTE_V2X) CR to Rel-14 36.141 on removal of Band 47 Summary of change: Band 47 is removed from relevant Clauses. |
| R4-2522259 | Ericsson | CR to TS 37.105: Clarification on RDN for Hybrid AAS BS Summary of change: [OTA_AAS_BS_RDN] |
| R4-2522260 | Ericsson | CR to TS 37.145-2 Corrections [OTA_AAS_BS_cleanup] Summary of change: |
| R4-2522296 | Huawei, HiSilicon | (NB_IOT-Core) CR for TS36104 to correct NB-IoT Medium Range BS OBUE requirements for R15 Cat F Summary of change: Correct the 0.015MHz f_offset < 0.615MHz to 0.165MHz f_offset < 0.615MHz. |
| R4-2522310 | Nokia | (TEI17) Correction of Operating band unwanted emissions for multi-band connector [TO_BE_ADDED] Summary of change: The paragraph on the mulit-band connector is corrected back to the originally intended text according to CR0360. |
| R4-2522313 | Ericsson | (RInImp9-Rfmulti,TEI) CR to 37.104: Removal of UTRA TDD from MSR BS specifications Summary of change: The CR removes the UTRA TDD RAT by removing the following: - TS 25.105 in list of References - UTRA TDD bands for BC3 - Applicability of requirement for UTRA TDD operation - Additional UTRA TDD requirements for BS output power, Output power dynamics, Dynamic range, Modulation quality, Frequency error and Time alignment. - Specific UTRA TDD RF and demodulation requirements (mostly references to the UTRA TDD specification) The CR removes co-existence and colocation with UTRA TDD by removing the following: - Co-existence and co-location with UTRA TDD operating bands named a) to f) in BS Spurious emissions - UTRA TDD bandwidths (1.28 Mcps, 3.84 Mcps and 7.68 Mcps) as “assumed adjacent carrier” for Base Station ACLR in unpaired spectrum with synchronized operation - Transmit intermodulation for colocation with UTRA TDD - Receiver blocking for co-existence with UTRA TDD - UTRA TDD (1.28 Mcps) as “RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge or sub-block edge” for Receiver intermodulation |
| R4-2522314 | Ericsson | (RInImp9-Rfmulti,TEI) CR to 37.141: Removal of UTRA TDD from MSR BS specifications Summary of change: The CR removes the UTRA TDD RAT by removing the following: - TS 25.105 and TS 25.142 in list of References - UTRA TDD bands for BC3 - Applicability of requirement for UTRA TDD operation - Additional UTRA TDD requirements for BS output power, Output power dynamics, Dynamic range, Modulation quality, Frequency error and Time alignment. - Specific UTRA TDD RF and demodulation requirements (mostly references to the UTRA TDD specification) The CR removes co-existence and co-location with UTRA TDD by removing the following: - Co-existence and co-location with UTRA TDD operating bands named a) to f) in BS Spurious emissions - UTRA TDD bandwidths (1.28 Mcps, 3.84 Mcps and 7.68 Mcps) as “assumed adjacent carrier” for Base Station ACLR in unpaired spectrum with synchronized operation - Transmit intermodulation for colocation with UTRA TDD - Receiver blocking for co-existence with UTRA TDD - UTRA TDD (1.28 Mcps) as “RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge or sub-block edge” for Receiver intermodulation The CR removes the following conformance testing aspects: - UTRA TDD specific Test Configurations (TC1b, TC3b and TC6c) - Capability Sets for BC3 that include UTRA TDD capability (CS1, CS3 and CS14) - Specific UTRA TDD test signals. - Reference to UTRA TDD for derivation of test requirements |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Sub-topic description:

Open issues and candidate options before meeting:

Issue 1-1: TBA

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Recommended WF

  - TBA

### Sub-topic 1-2

Sub-topic description

Open issues and candidate options before meeting:

Issue 1-2: TBA

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Recommended WF

  - TBA