3GPP TSG-RAN WG4 Meeting #118                                                                             R4-2602250

Gothenburg, Sweden, 9th – 13th Feb 2026

Agenda item:			8.1

Source:	Feature lead (Huawei, HiSilicon)

Title:	WF on [118][101-A] 6GR system parameter (part I)

Document for:	Approval

- Topic #1: Waveform

Waveform

Agreement (Main session):

- Uplink PAPR reduction to UL DFT-s-OFDM and CP-OFDM,

  - RAN4 carry out individual evaluations on PAPR-reduction techniques, mainly focusing on the Tx power gain by taking RF requirements into consideration.

    - Coordination with RAN1 is required on the aspects which have RAN1 impacts.

  - The evaluation should be based on the agreed waveform evaluation assumptions

    - RAN1’s agreements on the evaluation assumption should be taken into consideration.

- Other Waveforms

  - Defer consideration of other waveform (like DL DFT-s-OFDM) until RAN1 reaches consensus.

- Topic #2: Channel bandwidth

Sub-topic 3-1: Max Channel Bandwidth

- Main proposals

  - Maximum CBW Limits:

    - 200 MHz Baseline: Many companies, especially UE vendors, propose 200 MHz as the maximum single-carrier CBW for both UL and DL in ~7 GHz bands.

    - 400 MHz Support: Some propose supporting 400 MHz in the ~7 GHz band, either as optional for flagship users, restricted to DL only, or as a single carrier to maximize efficiency.

  - FFT Size:

    - Many companies propose to limit the mandatory baseline FFT size to 8k, while some suggest 16k should be an implementation choice or supported for specific cases.

  - CA approach:

    - Some companies in their proposals clearly suggest 2x200 MHz CA as the preferred method to reach 400 MHz total bandwidth to ease RF implementation.

  - Device types:

    - Proposals exist to differentiate CBW support based on device type (e.g., 200 MHz for smartphones vs. 400 MHz for FWA/CPE).

- Chair’s guidance on max CBW in RAN4#118

  - To facilitate the discussion and eventually agreements, the following aspects should be discussed and clarified

  - DL

    - Feasibility to support 400MHz as single CC at both UE and BS

    - By assuming single 400MHz CC is feasible at both UE and BS, comparison between 2x200MHz CA and 400MHz single CC should be clarified including the aspects of spectrum efficiency, SCell activation/BWP switching delay, power consumption and complexity, etc.

  - UL

    - Feasibility to support 400MHz as single CC at both UE and BS.

- Recommended WF

  - Discuss the feasibility of supporting 400 MHz separately for DL and UL.

  - The feasibility analysis should account for both current and near-future implementation constraints, as well as long-term technology advancements.

  - 200 MHz for both UL and DL can serve as a baseline for specifying RF requirements in RAN4, while keeping the door open for future evolution to larger channel bandwidths after the initial 6G release.

Sub-topic 3-2: Min Channel Bandwidth

- WF

  - Joint effort with RAN1 for the minimum CBW relevant to initial access

  - Collect views on minimum CBW for specific SCS

  - Postpone the decision on minimum CBW for specific 6G operating bands until the WI stage

  - CBW step size could be further considered after conclusion of max and min CBW with inputs from operators

Sub-topic 3-4: Numerology

Agreement (Main session):

- Single and same SCS per operating band for both UL and DL, except PRACH

  - FR1 FDD (Sub-6GHz): 15 kHz

  - FR1 TDD (Sub-6 GHz): 30 kHz

  - ~7 GHz Band including U6G: 30 kHz

  - ~15 GHz Band for TN:	FFS (30, 60, or 120 kHz)

    - Postpone the discussion until clarification is made in RAN plenary

  - FR2-1 (24.25–52.6 GHz): 120 kHz

    - Numerology of SSB is FFS including 120kHz and 240kHz

- Postpone discussion of numerology for NTN and ISAC until further progress is made in RAN1.

Sub-topic 3-5: Spectrum utilization

- WF

  - Agree on a set of common simulation assumptions for SU evaluation, including PA models, RF impairments (e.g., carrier leakage, I/Q imbalance, phase noise, etc.), and baseline RF requirements (e.g., SEM, ACLR, EVM).

    - 5G NR channel bandwidth, requirements can be considered as starting point for the SU evaluation with new assumptions for 6G

    - PA model

      - Companies can choose their own models

    - RF impairments

      - 6G new assumptions depend on the progress on UE RF discussion

      - 5G assumptions could be used for initial evaluation for existing CBWs with new spectrum confinement techniques

Sub-topic 3-6: Asymmetric channel bandwidths

- Agreement in AH

  - Defer the detailed evaluation of asymmetric channel bandwidth at least after max/min CBW is finalized.

Sub-topic 3-6: Irregular channel bandwidths

- WF

  - Investigate and compare candidate solutions for irregular channel bandwidth including the definition.

  - Investigate the feasibility of defining RF requirements based on the actual activated bandwidth (BWP-like) or using scalable formulas, assessing the impact on testability and performance.

  - Encourage operators to provide inputs on candidate irregular CBWs