3GPP TSG-RAN WG4 Meeting #118                                                                R4-260xxxx

Gothenburg, Sweden, Feb. 09-13, 2026

Agenda item:			8.2.1

Source:	Huawei, HiSilicon, [vivo, OPPO, Samsung, MediaTek, ZTE]

Title:	WF on PA models in 6GR waveform evaluations

Document for:	Approval

- Introduction

The following PA models were proposed by different companies for RAN1 waveform evaluations.

The PA models used for waveform evaluation and RAN4 RF requirements assessment are closely related to operating bands/sub-frequency ranges, channel bandwidth, power classes and calibration conditions. It is recognized by RAN4 that, with increased channel bandwidth (e.g., CBW ≥ 100 MHz), the PA exhibits a noticeable memory effect, a distinguishing characteristic that sets it apart from 5G PA models designed for smaller channel bandwidths.

The PA models captured in this WF are those currently available, which may be further refined with additional inputs from companies during the follow-up study. It should be noted that these models are not intended to fully replace those used for 5G evaluation, but rather serve as supplementary models with different factors. Existing models in TS 38.803 may still be applicable in certain cases, such as for small channel bandwidths and lower frequency bands.

- PC3 PA Models:

The models were calibrated using reference waveform: 100MHz CBW, 30kHz SCS, 270RB0, QPSK, DFT-s-OFDM, 1dB MPR to meet 30dB ACLR.

- GMP1_PC3 [1]:

The model was obtained for a at 6.8GHz.
youtputn= m=0Mk=1Kam,2k-1xinputn-mxinputn-m2(k-1)

Where xinput, youtput, am,2k-1, M and K are the PA input signal in volts, PA output signal in volts, model coefficients, memory depth, and the polynomial order, respectively.

| Memory based PC3 PA Polynomial model (100M, K=7, M=3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| a0,1 | -4.77719730e-01 + 2.24168134e+01j | a1,1 | 8.38188385e+00 + 1.18702410e+01j | a2,1 | -7.67804529e+00 - 1.22093585e+01j | a3,1 | 2.32750874e+00 + 4.20967176e+00j |
| a0,3 | -1.74492957e+01 + 4.92359296e+01j | a1,3 | 2.96063305e+01 - 5.32354340e+01j | a2,3 | 5.62951175e+00 + 4.34443316e+01j | a3,3 | 2.01399386e+01 - 1.09605427e+01j |
| a0,5 | -8.98826975e+02 + 1.27577672e+03j | a1,5 | -9.22569925e+02 - 4.58901528e+02j | a2,5 | -6.61005032e+02 - 1.35739360e+03j | a3,5 | -1.21142964e+03 + 1.99559746e+02j |
| a0,7 | 3.09544724e+04 - 1.07105471e+05j | a1,7 | 2.37092395e+04 + 5.68230115e+04j | a2,7 | 1.96480251e+04 + 1.88253421e+04j | a3,7 | 3.31901649e+04 - 8.28113612e+02j |
| a0,9 | -4.51805825e+05 + 1.50721849e+06j | a1,9 | -3.34660576e+05 - 9.66045483e+05j | a2,9 | -2.85105812e+05 - 1.24927829e+05j | a3,9 | -4.61058530e+05 - 1.55413027e+04j |
| a0,11 | 3.12726871e+06 - 8.61147896e+06j | a1,11 | 2.28790752e+06 + 6.56559099e+06j | a2,11 | 1.98672175e+06 + 3.15477725e+05j | a3,11 | 3.10446930e+06 + 1.95853544e+05j |
| a0,13 | -8.12757739e+06 + 1.80971663e+07j | a1,13 | -5.93387391e+06 - 1.61211722e+07j | a2,13 | -5.23054157e+06 + 7.06402679e+03j | a3,13 | -7.95973750e+06 - 6.33528334e+05j |
| NOTE 1: PA input signal and PA output signal sampling rate was 491.52 MHz. NOTE 2: The model input signal is clipped at 4.6dBm (0.3801 VRMS with an impedance of 50 Ohm) |  |  |  |  |  |  |  |

It is noted that the input data sampling rate should be aligned with the model training sampling rate to avoid performance degradation.

- GMP2_PC3 [2]:

The model was obtained at 5GHz.

yn=k=0Ka-1l=0La-1akl∙x(n-l)∙|xn-l|2k+k=1Kb-1l=-LbLbm=1Mbbklm∙x(n-l)∙xn-l2k∙|xn-l-m|2

With Ka = 7, La = 5, Kb = 7, Lb = 2

akl;k ∈0,1,…,6: l=2: [0.050221-0.0008979i; 0.0071456+0.0022591i; -0.0078367+0.0037419i; 0.0019788-0.0018115i; -0.0002329+0.000338i; 0.0000131-0.0000289i; -0.0000003 + 0.0000009i] l=1: [0.0764254-0.0022761i; 0.0205606+0.0145471i; -0.0241173+0.0031995i; 0.0064507-0.0022644i; -0.0008253+0.0004248i; 0.0000523-0.0000347i; -0.0000013+0.0000011i] l=0: [15.2864241-0.4639873i; 2.0956681+1.3655244i; -2.441954+0.4802089i; 0.6500257-0.3037694i; -0.0823278+0.058242i; 0.0051372-0.004931i; -0.0001269+0.0001569i] l=-1: [0.0767111-0.0021611i; 0.0190786+0.0123216i; -0.023051+0.0060845i; 0.0061274-0.0035563i; -0.0007769+0.0006822i; 0.0000487-0.0000583i; -0.0000012+0.0000019i] l=-2: [0.0509934-0.0015377i; 0.0066738+0.0054215i; -0.0077434+0.0006127i; 0.0019344-0.0006365i; -0.0002169+0.0001292i; 0.000011-0.0000112i; -0.0000002+0.0000004i] bklm;k ∈0,1,…,6: l=1, m=0: [0.0764254-0.0022761i; 0.0001153-0.0019156i; -0.0003342+0.0028445i; 0.0000369-0.0013797i; 0.0000221+0.0002917i; -0.0000046-0.000028i; 0.0000002+0.000001i] l=-1, m=0: - [0.0767119-0.0021553i; -0.0030492+0.000024i; 0.0040687-0.0001464i; -0.0018971+0.0000699i; 0.000397-0.0000117i; -0.0000382+0.0000007i; 0.0000014]

- PC3 Memoryless model [3]:

The model was obtained at 4 GHz.

AM-AM

y = 4.10540378e-12*x^9 + 8.34889528e-10*x^8 + 0.0000000527815378*x^7 + 0.00000136322797*x^6 + 0.00000957459383*x^5 - 0.000205038337*x^4 - 0.00423915426*x^3 - 0.0256252074*x^2 + 0.967801028*x + 27.1826545

where x is the PA input power with unit dBm and y is the PA output power with unit dBm.

AM-PM

y = 2.22503156e-10*x^9 + 0.0000000205977704*x^8 + 0.000000640395531*x^7 + 0.00000482639611*x^6 - 0.000121813382*x^5 - 0.00208845328*x^4 + 0.00334876579*x^3 + 0.235994334*x^2 + 1.45386121*x + 76.2546141

where x is the PA input power with unit dBm and y is the PA output phase shift with unit degree.

- PC2 PA Models:

The models were calibrated using reference waveform: 100MHz CBW, 30kHz SCS, 270RB0, QPSK, DFT-s-OFDM, 1dB MPR to meet 31dB ACLR (unless stated otherwise)

- GMP3_PC2 [4]:

The model was obtained at ~7GHz.
youtputn= m=0Mk=1Kam,2k-1xinputn-mxinputn-m2(k-1)

Where xinput, youtput, am,2k-1, M and K are the PA input signal in volts, PA output signal in volts, model coefficients, memory depth, and the polynomial order, respectively.

| Memory based PC2 PA Polynomial model (K=7, M=3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| a0,1 | 1.78125421e+01+3.96234802e+00j | a1,1 | 7.75483096e+00-1.06941812e+01j | a2,1 | -7.72899765e+00+9.04484189e+00j | a3,1 | 2.48795348e+00-2.76518830e+00j |
| a0,3 | 9.75722189e+01+9.49718204e+01j | a1,3 | -6.46736904e+01+1.55427298e+01j | a2,3 | 2.65557457e+00-1.41372308e+01j | a3,3 | 8.14834939e+00+3.68035394e+00j |
| a0,5 | -4.36921775e+03-2.90487370e+03j | a1,5 | 3.25651083e+03-5.88392401e+02j | a2,5 | -5.23038528e+02+4.58746411e+02j | a3,5 | -1.51258701e+02-1.40300633e+02j |
| a0,7 | 6.67412504e+04+3.62371316e+04j | a1,7 | -4.19280551e+04+6.06657427e+03j | a2,7 | 3.95605656e+03-5.72662196e+03j | a3,7 | 4.02914882e+03+1.39078164e+03j |
| a0,9 | -5.42918789e+05-2.29695739e+05j | a1,9 | 2.56101445e+05-3.11728454e+04j | a2,9 | 9.08023985e+02+3.43943069e+04j | a3,9 | -4.26449208e+04-3.95132361e+03j |
| a0,11 | 2.19218318e+06+7.35795077e+05j | a1,11 | -7.70951908e+05+8.54072559e+04j | a2,11 | -8.90153246e+04-1.02613131e+05j | a3,11 | 1.90424815e+05-6.66193945e+03j |
| a0,13 | -3.40445900e+06-9.45835587e+05j | a1,13 | 9.20919772e+05-1.00871883e+05j | a2,13 | 2.14152570e+05+1.22085816e+05j | a3,13 | -3.07290090e+05+3.43617693e+04j |
| NOTE 1: The front-end IL of 4dB is already considered in the model. NOTE 2: PA input signal and PA output signal sampling rate was 614.4 MHz. NOTE 3: The model input signal is clipped at 5dBm (0.39764 VRMS with an impedance of 50 Ohm) |  |  |  |  |  |  |  |

- GMP4_PC2 [5]:

The model was obtained at 4.9GHz.

yn= m=0Mk=1Kamk*xn-m*|x(n-m)|k-1

| amk | k = 1 | k = 2 | k = 3 | k = 4 | k = 5 |
| --- | --- | --- | --- | --- | --- |
| m = 0 | 1.24100038872962 - 0.0831368575300736i | -0.463340338937706 + 0.101363976888891i | 1.26901654946376 - 0.0720028018021318i | -1.43394604933662 + 0.239529480083968i | 0.311614480209606 - 0.341679426545316i |
| m = 1 | -0.124904834225151 + 0.0985131060907833i | 0.00308291501654481 + 0.0514108710576260i | -0.181239524012283 - 0.287147428096619i | 0.532131005999187 + 0.508260249170445i | -0.273169554049464 - 0.308931076102838i |
| m = 2 | 0.266858623504599 - 0.0960971965549797i | -0.0211895925916308 - 0.00596034296608958i | 0.164616933475616 + 0.101644617541411i | -0.371232225003208 - 0.198394522680648i | 0.185836703475180 + 0.130436532035389i |
| m = 3 | -0.186254778642275 + 0.0464461010255326i | 0.0353184555081896 + 0.0719981097161610i | -0.157707539584257 - 0.261669803610032i | 0.256173858313972 + 0.383789794112876i | -0.112000236856695 - 0.200342960880782i |

- GMP5_PC2 [6]:

The model was obtained at 6.8 GHz.

| Order | A | b | c | d | e |
| --- | --- | --- | --- | --- | --- |
| 1st | 34.5740 - 30.4430i | -73.8674 + 60.7998i | 83.3917 + 64.6447i | -1.0110e+02 - 1.1335e+02i | 56.4092 + 42.0192i |
| 3rd | -2.1020e+01 + 1.6457e+02i | 2.3961e+00 - 2.9547e+02i | -2.5102e+02 - 5.2031e+02i | 3.6879e+02 + 1.2987e+03i | -1.6143e+02 - 6.5106e+02i |
| 5th | 3.2327e+02 - 5.4457e+02i | 3.9517e+02 + 2.4179e+03i | -1.1425e+03 - 4.2744e+03i | 1.0211e+03 + 3.5902e+03i | -1.6374e+02 - 1.3977e+03i |
| 7th | -1.9519e+02 + 3.1487e+03i | -3.0089e+03 - 1.4757e+04i | 5.9442e+03 + 2.6497e+04i | -4.8623e+03 - 2.2651e+04i | 7.5488e+02 + 8.5670e+03i |
| 9th | -1.1596e+03 - 5.9812e+03i | 6.6253e+03 + 3.0041e+04i | -8.4557e+03 - 5.4828e+04i | 4.6526e+03 + 4.7036e+04i | 5.9613e+02 - 1.7654e+04i |
| 11th | 1.8405e+03 + 4.8550e+03i | -5.8466e+03 - 2.6831e+04i | 4.3521e+03 + 5.0627e+04i | 1.3994e+02 - 4.4149e+04i | -2.3675e+03 + 1.6650e+04i |
| 13th | -8.1103e+02 - 1.4225e+03i | 1.8653e+03 + 8.8912e+03i | -3.9780e+02 - 1.7500e+04i | -1.4314e+03 + 1.5603e+04i | 1.3971e+03 - 5.9424e+03i |

- PC2 Memoryless model [3]:

The model was obtained at 4 GHz.

AM-AM

y = 4.10540378e-12*x^9 + 7.97940894e-10*x^8 + 0.0000000462502161*x^7 + 0.00000101678926*x^6 + 0.00000245740177*x^5 - 0.000234252316*x^4 - 0.00334871859*x^3 - 0.0142143574*x^2 + 1.00719419*x + 29.5632541

where x is the PA input power with unit dBm and y is the PA output power with unit dBm.

AM-PM

y = 2.22503156e-10*x^9 + 0.000000018595242*x^8 + 0.000000483623481*x^7 + 0.000000901674698*x^6 - 0.000138448892*x^5 - 0.00142799046*x^4 + 0.0104091962*x^3 + 0.214694967*x^2 + 0.999638948*x + 75.0314361

where x is the PA input power with unit dBm and y is the PA output phase shift with unit degree.

- Reference

[1] R4-2600667, Discussion on 6G waveform and PA, vivo

[2] R4-2601122, Discussion on waveform for 6GR, Samsung

[3] R4-2600785, Discussion on 6GR waveform, ZTE

[4] R4-2600885, On system parameters for 6G Waveform. Huawei, HiSilicon

[5] R4-2601448, On 6GR waveform, OPPO

[6] R4-2601446, Views on 6G PA model, MediaTek